| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F069 M20481 |
| --- | --- |

| Title: | Parallelized merge/skip mode for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

# The current HEVC merge/skip mode design is highly sequential and introduces dependency among neighboring PUs, which can lead to significant quality loss if motion estimation (ME) is performed in parallel for throughput or implementation cost reasons. For typical parallel ME level of 32x32, the measured average loss is 6.0% in RA-HE, 6.3% in RA-LC, 7.7% in LB-HE and 8.8% in LB-LC. The loss is caused by fact that the merge/skip mode cannot be tested for those PUs inside the 32x32 block whose neighboring motion data are still unavailable during the parallel processing process. It is proposed to add a high-level syntax element to signal the parallel level of merge/skip mode, divide a LCU into parallel motion estimation regions (MERs) and allow only those neighboring PUs which belong to different MERs from the current PU to be included in the merge/skip MVP list construction process.  Simulation results reveal that an average gain of 3.9% in RA-HE, 4.2% in RA-LC, 5.5% in LB-HE and 6.4% in LB-LC can be achieved for 32x32 block level parallel ME when compared to the current HM3.0 design. The proposed design is backward compatible to the current design and offers flexibility for high throughput and high quality encoder designs.

# Introduction

# The current HEVC merge/skip mode design [2] is highly sequential and introduces dependency among neighboring PUs, which creates difficulty for the motion estimation on the encoder side for the pipelined architecture, in which motion vectors might be estimated in parallel for multiple PUs. In this contribution an algorithm is proposed to enhance parallelism of the merge/skip mode and facilitate high throughput and high quality encoder designs.

# Problem statement

Current PU

5

4

2

1

Co-located PU

CR 3

H 3

Figure 1. Illustration of the merge/skip MVP list construction of HM3.0

Figure 1 illustrates the MVP list construction of the merge/skip mode defined in HM3.0. For the merge/skip mode of the current PU, a total of four neighboring PUs (i.e. left (1), upper (2), upper-right (4) and bottem-left (5) neighboring PUs) and temporally co-located PUs (CR 3 and H3) are involved to form the merge/skip mode MVP list. The reference index for temporal TMVP derivation also depends on the availability and reference index of neighbors of the current PU.

2N x2N

N x 2N

2N xN

N xN

Figure 2. For a block of 2N x 2N, merge/skip search is only possible for the first PU of each partition if motion estimation is carried out in parallel for entire 2N x 2N region.

With the current HM3.0 design, if motion estimation is carried out in parallel for entire 2N x 2N block region, only the first PU of each partition (see green blocks in Figure 2) is able to have merge/skip mode searched in parallel to the regular motion estimation process (the regular motion estimation process estimates motion vectors for a PU independent of neighboring PUs), the rest of PUs will have to drop testing of the merge and skip mode because the neighboring motion data (motion vector, prediction direction, reference index, prediction size, intra/inter flag) are still unavailable when the parallel motion estimation is taking place. In general, for recursive motion estimation in which a 2N x 2N CU can be further partitioned into smaller CUs, only those PUs whose top-left corner sample is coincident with the top-left corner sample of the 2N x 2N block are able to test merge/skip mode in parallel motion estimation environment.   This can lead to significant quality loss because merge and skip mode have great impact on video quality. As shown in Table 1, significant quality loss is observed if the parallel motion estimation is carried out at 64x64 LCU level and 32x32 block level.

| Parallel ME level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | 8.5 | 9.3 | 10.0 | 12.4 |
| 32 x 32 | 6.0 | 6.3 | 7.7 | 8.8 |
| 16 x 16 | 2.7 | 2.7 | 3.5 | 3.7 |
| 8 x 8 | 0.4 | 0.5 | 0.4 | 0.5 |

Table 1, BD-rate increase relative to HM3.0 anchor when the parallel motion estimation is performed at 2N x 2N block level (N = 32, 16, 8 and 4, LCU size is fixed to 64x64)

Parallel motion estimation is needed for practical video encoder implementation for meeting throughput requirements and lowering implementation cost of the motion estimation engine. For example, for the previous standards including AVC the typical parallel motion estimation level is 16x16, which is aligned with the macroblock size. For AVC, some encoders are doing 16 x 32 level parallel motion estimation for MBAFF support. For HEVC in which LCU size can go up to 64 x 64 and UHD (4K x 2K or 8K x 4K )  application is expected, performing parallel motion estimation at 32 x 32 can become a minimum requirement to meet real-time requirement at reasonable costs.

# Proposed algorithm

To reduce the quality loss in the parallel motion estimation environment, we propose to modify the spatial MVP availability rules for merge/skip MVP list to enhance the parallelism of merge/skip mode, so that  merge/skip mode can be tested for all the PUs during the parallel motion estimation. The key parts of the algorithm are as follows:

- Define a high level syntax element (log2_parallel_merge_level_minus2) in picture parameter set to indicate parallel processing level of merge/skip mode.

- Based on the value of log2_parallel_merge_level_minus2, divide a LCU into a number of non-overlapped of motion estimation regions (MER). A MER has square shape.

- Modify the spatial MVP availability rules:  if a neighboring PU and the current PU belong to a same MER, this neighboring PU is treated as unavailable for spatial MVP derivation of the merge/skip MVP list construction process.

- Modify the temporal MVP reference index derivation rules: if a neighboring PU and the current PU belong to a same MER, this neighboring PU is treated as unavailable for the temporal MVP reference index derivation process.

## Definition of  log2_parallel_merge_level_minus2

A high level syntax element log2_parallel_merge_level_minus2 is added to picture parameter set as shown in table 2.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| log2_parallel_merge_level_minus2 | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2, modified pic_parameter_set_rbsp()

log2_parallel_merge_level_minus2 can have value of 0, 1, 2, 3 and 4, and 1 <<( log2_parallel_merge_level_minus2+2) shall not exceed the LCU size defined in the sequence parameter set.

The parallel merge/skip levels defined by log2_parallel_merge_level_minus2 are present in Table 3.

| log2_parallel_merge_level_minus2 | MER size | Remark |
| --- | --- | --- |
| 0 | 4x4 | Sequential merge/skip mode for all PUs in a LCU because minimum PU size allowed by HEVC is 4x4 |
| 1 | 8x8 | Parallel merge/skip mode search allowed for all PUs inside an 8x8 block |
| 2 | 16x16 | Parallel merge/skip mode search allowed for all PUs inside a 16x16 block |
| 3 | 32x32 | Parallel merge/skip mode search allowed for all PUs inside a 32x32 block |
| 4 | 64x64 | Parallel merge/skip mode search allowed for all PUs inside a 64x64 block |

Table 3, parallel merge/skip level defined by log2_parallel_merge_level_minus2

## Partitioning a LCU into motion estimation regions (MERs)

As shown in Figure 3, a LCU (e.g. 64x64 size) is divided into a number non-overlapped motion estimation regions of equal size. From MER to MER, the motion estimation is carried out sequentially, i.e. motion estimation is performed on the next MER after the motion estimation of the current MER is finished. Within a MER, the motion estimation is carried out in parallel for all the PUs residing in the MER. The MER size should be less or equal to LCU size. In the example shown in Figure 3, a LCU is divided into four MERs.

PU0

PU1

PU2

MER0

MER1

Not unavailable MVPs

(different MERs)

unavailable MVPs

(same MER)

unavailable MVPs

(neighboring PUs are not coded yet)

LCU

MER2

MER3

Figure 3. Illustration of motion estimation region (MER) partitioning, and availability rules of spatial MVPs for PUs at different locations

## Spatial MVP availability change for merge/skip MVP list construction

To enable testing merge/skip mode for every PU of a MER during the parallel motion estimation process, an additional check is introduced to define the MVP availability for spatial MVP derivation and for temporal MVP reference index derivation of merge/skip mode. Let (xP, yP) and (xN, yN) be the coordinates of the top-left corner pixel of the current PU and a neighboring PU (see Figure 4), respectively, whether the current PU and its neighboring PU belong to a same MER can be easily determined. This neighboring PU is treated unavailable if the following condition satisfies.

(xP >>( log2_parallel_merge_level_minus2+2))

== (xN >> ( log2_parallel_merge_level_minus2+2))

and

(yP >>( log2_parallel_merge_level_minus2+2))

== (yN >> ( log2_parallel_merge_level_minus2+2))

Current PU

N

(xN, yN)

(xP, yP)

Figure 4. Illustration of the current PU and its neighboring PU

Otherwise, the spatial MVP availability rules defined in current HM3.0 [2] (see section 8.4.2) are used to determine the availability of the spatial MVP for merge/skip MVP list and for reference index determination of temporal MVP in merge/skip mode.

For example, in Figure 3 for PU0 all its spatial neighboring PUs are all located in different MERs, for PU1 all its spatial neighboring PUs are unavailable as they all belong to a same MER as the current PU, while for PU2 only part of its spatial neighboring PUs belong to different MERs.  In the case of all the spatial neighboring PUs belonging to a same MER, only temporal MVP is used for the merge/skip mode.

Note that log2_parallel_merge_level_minus2 equal to zero indicates 4x4 block level parallel motion estimation, which is essentially same as what being done in the HM reference software.  Therefore, the proposed algorithm is backward compatible to the existing HM3.0 design, but offers more flexibility on the encoder side to meet throughput and quality requirements.

The proposed algorithm won’t force an encoder to align value of log2_parallel_merge_level_minus2 with the actual parallel motion estimation level implemented in the encoder. But a smart encoder would choose to set value of log2_parallel_merge_level_minus2 aligned with the parallel motion estimation level implemented in the encoder to drive maximum quality out of the motion estimation engine.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

Following the common testing conditions specified in [1], simulation is carried out to verify the effectiveness of the proposed algorithm. As shown in Table 3, with the proposed change in the merge/skip list construction, two third of loss caused by the parallel motion estimation can be gained back.

| Parallel ME (merge/skip) level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | 3.7 | 3.6 | 4.1 | 4.4 |
| 32 x 32 | 2.1 | 2.1 | 2.2 | 2.4 |
| 16 x 16 | 0.8 | 0.7 | 0.5 | 0.7 |
| 8 x 8 | 0.1 | 0.0 | -0.1 | 0.0 |

Table 3, BD-rate increase of the proposed algorithm relative to HM3.0 anchor when the parallel motion estimation is performed at 2N x 2N block level (N = 32, 16, 8 and 4, LCU size is fixed to 64x64)

The relative bit-rate saving of the proposed algorithm compared to the current HM3.0 design in the parallel motion estimation environment is summarized in Table 4. For typical parallel motion estimation level of 32x32, an average gain of 3.9% in RA-HE, 4.2% in RA-LC, 5.5% in LB-HE and 6.4% in LB-LC are observed. For detailed results please refer to attached spreadsheets.

| Parallel ME (merge/skip) level 2N x 2N (LCU = 64x64) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| 64 x 64 | -4.8 | -5.7 | -5.9 | -8.0 |
| 32 x 32 | -3.9 | -4.2 | -5.5 | -6.4 |
| 16 x 16 | -1.9 | -2.0 | -3.0 | -3.0 |
| 8 x 8 | -0.3 | -0.5 | -0.5 | -0.5 |

Table 4, BD-rate saving of the proposed algorithm relative to the current HM3.0 design when the parallel motion estimation is performed at 2N x 2N block level (N = 32, 16, 8 and 4, LCU size is fixed to 64x64)

# Conclusion and recommendations

The proposed algorithm achieves significant quality improvement in the parallel ME environment as compared to the current HM3.0 design. It improves parallelism of the merge/skip mode and offers flexibility for high quality an high throughput HEVC encoder designs. It is recommended to adopt this change into the HEVC test model.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD text changes

#### 7.3.2.2 Picture parameter set RBSP syntax

#### 7.4.2.2. Picture parameter set RBSP semantics

##### 8.4.2.1.2 Derivation process for spatial merging candidates

##### 8.4.2.1.3 Derivation process of reference indices for temporal merging candidate

#### Replace

#### 7.3.2.2 Picture parameter set RBSP syntax

With

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| log2_parallel_merge_level_minus2 | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### In 7.4.2.2. Picture parameter set RBSP semantics

Add

log2_parallel_merge_level_minus2  specifies parallel processing level of merge/skip mode. The value of  log2_parallel_merge_level_minus2  should be in the range of [0: log2_min_coding_block_size_minus 3 + 1 + log2_diff_max_min_coding_block_size], inclusive.

##### In 8.4.2.1.2 Derivation process for spatial merging candidates

Replace

…

For the derivation of availableFlagN, with N being A, B, C or D and ( xN, yN ) being ( xP – 1,  yP ), ( xP,  yP – 1 ), ( xP + nPSW,  yP – 1 ) or ( xP – 1,  yP+nPSH ) the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

With

…

For the derivation of availableFlagN, with N being A, B, C or D ,  ( xN, yN ) being ( xP – 1,  yP ), ( xP,  yP – 1 ), ( xP + nPSW,  yP – 1 ) or ( xP – 1,  yP+nPSH ) and (xN’, yN’) being ( xP – 4,  yP ), ( xP,  yP – 4 ), ( xP + nPSW,  yP – 4 ) or ( xP – 4,  yP+nPSH ) the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

  - if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to (xN’ >> (log2_parallel_merge_level_minus2 +2)) and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to (yN’ >> (log2_parallel_merge_level_minus2 +2))

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

##### In  8.4.2.1.3 Derivation process of reference indices for temporal merging candidate

Replace

…

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

For the derivation of refIdxLXC, the following applies.

- If the prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[  xP-1, yP+nPSH ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA refIdxLXC is assigned to refIdxLX[  xP-1, yP-1 ].

- Otherwise, refIdxLXC is assigned to -1.

With

…

For the derivation of refIdxLXA, the following applies.

- if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((xP-4) >> (log2_parallel_merge_level_minus2 +2)), refIdxLXA is assigned to -1.

- Otherwise, if the prediction unit covering luma location ( xP-1, yP ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- if (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP-4) >> (log2_parallel_merge_level_minus2 +2)), refIdxLXB is assigned to -1.

- Otherwise, if the prediction unit covering luma location ( xP, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

For the derivation of refIdxLXC, the following applies.

- If (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((xP+nPSW) >> (log2_parallel_merge_level_minus2 +2)) and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP-4) >> (log2_parallel_merge_level_minus2 +2)), refIdxLXC is assigned to -1.

- Otherwise, if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((xP - 4) >> (log2_parallel_merge_level_minus2 +2)) and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP+nPSH ) >> (log2_parallel_merge_level_minus2 +2)), refIdxLXC is assigned to -1.

- Otherwise, if (xP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((xP - 4) >> (log2_parallel_merge_level_minus2 +2)) and (yP >> (log2_parallel_merge_level_minus2 +2)) is equal to ((yP -4 ) >> (log2_parallel_merge_level_minus2 +2)), refIdxLXC is assigned to -1.

- Otherwise, if the prediction unit covering luma location ( xP+nPSW, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[ xP+nPSW, yP-1 ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP+nPSH ) is available and PredMode is not MODE_INTRA, refIdxLXC is assigned to refIdxLX[  xP-1, yP+nPSH ].

- Otherwise if the prediction unit covering luma location ( xP-1, yP-1 ) is available and PredMode is not MODE_INTRA refIdxLXC is assigned to refIdxLX[  xP-1, yP-1 ].

- Otherwise, refIdxLXC is assigned to -1.