| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F074 |
| --- | --- |

| Title: | Cross-check of TI’s study on HEVC parsing throughput issue JCTVC-F068 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-F068, a study on HEVC parsing throughput issue proposed by Texas Instruments. Two methods for decoupling motion data decoding from entropy decoding are tested. The proposed code has been checked, compiled and the results reported by Texas Instruments can be confirmed.

# Tested Proposals

The tested proposal has been integrated into HEVC HM 3.0 [2]. Two methods for decoupling motion data decoding from entropy decoding are tested.

## Method1

Compared to HM3.0, the following was changed for merge index and amvp index coding:

- Merge index and amvp index are coded using truncated unary code with fixed size of 5 and 3, respectively.

- The merge index context is independent of available merge candidates.

  - Method2

Compared to HM3.0, the following was changed for merge index and amvp index coding:

- Merge index and amvp index are coded using truncated unary code with fixed size of 3 and 2, respectively.

- The merge index context is independent of available merge candidates.

- The merge candidate list is reduced to contain not more than three candidates. Thus, the list index corresponds to the merge_idx that is coded. The construction of the merge candidate list is modified by:

  - removing intra coded candidates (invalid) and all candidates with merge idx > 3

  - the zero motion vector is not always added but if the number of valid candidates is less than 3, the zero motion vector is added

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [3]. The HM3.0 default settings, both for configuration file and compile-time, are used in this test as anchor. Due to its influence on motion vector coding, a bug in the spatial motion vector scaling process was fixed in the anchor as well as in the tested software (http://hevc.kw.bbc.co.uk/trac/ticket/146).

## Hardware

Two types of computers have been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system. They are listed in Table 2-1.

Table 2-1	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Decoding on 1 core without yuv | 1x4 | 4 x Intel(R) Core(TM)2 Extreme CPU Q9300 @ 2.53GHz | 4 GB |
| 2 | Encoding for test SP12 and HM3.0 anchor to be compared with | 2x6 | 2 x 6 x Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB |

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-F074-Method1.xls and JCTVC-F074- Method2.xls.

Table 2-2	Method1

Table 2-3	Method2

# References

- Minhua Zhou, Vivienne Sze, Yasutomo Matsuba, “A study on HEVC parsing throughput issue”, JCTVC-F068, Turino, July, 2011.

- JCT-VC, “HEVC Test Model Software 3.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.