| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F077_r1 |
| --- | --- |

| Title: | Transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini, Nikola Sprljan, David Flynn Centre House, 56 Wood Lane W12 7SB London, UK | Email: | marta.mrak@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution addresses transformation of residuals, where a row and/or a column transforms can be skipped. Encoder determines a selection of transforms / skipped transforms, i.e. so-called Transform Skip Mode (TSM), for each block based on RDO search. The transform skip mode choice is signalled to the decoder where inverse transforms of rows/ columns are performed or skipped.

This method has been implemented in HM3.2, for motion compensated blocks. Skipped transforms are replaced with scaling of input coefficients. The presented design relies on the block transform with 16-bit intermediate data representation [E243]. Underlying transform properties allow for a simple design of the transform skip - HM3.2 quantization does not require any changes and the choice of scaling factor for a row/column where transform is skipped depend only on the block size.

Tests have been performed without RQT for targeted blocks (inter coded). Compared to HM3.2 anchors the presented approach shows gains of 0.2 % BD-rate averaged over all inter coded configurations. Since the proposed approach has been implemented without RQT, its performance has also been compared to HM3.2 with disabled RQT for inter coded blocks. In that case, the gain is 1.0 % BD-rate averaged over all inter coded configurations.

The performance of the new method introduces gains comparable to the RQT gains on inter coded blocks, while preserving non-recursive design.

# Introduction

An integer implementation of DCT is used in the HM3.x as 1D transforms applied on rows and columns of TUs. While for certain classes of signals the implementation of transforms in both directions can be an optimal choice in terms of energy compaction, for signals with different properties some other strategies may lead to better RD performance. It is possible, and indeed it is also common under some circumstances, for the 2D signal in input to the transform to display different statistics along the two vertical and horizontal axes. This has already been addressed in HEVC by adoption of DST for intra residuals related to certain directional prediction modes, [E125]. However, for sparser signals and those that perhaps exhibit different properties in horizontal and vertical direction alternative processing options can be derived. This contribution addresses this problem by proposing a method that skips transforms on TUs in horizontal and/or vertical directions, to improve compression.

# Transform skip mode

The Transform Skip Mode (TSM) defines transform skip in one or both directions on which a transform would be applied under common conditions, Figure 1. As illustrated in Figure 1.a), TSM covers the traditional approach where the transform is applied on both rows and columns of a block.

| a) 2D transform | b) transform on rows only | c) transform on columns only | d) no transform |
| --- | --- | --- | --- |

Figure 1. Transform choices enabled by the proposed transform skip mode

The choice of transform or its skip equivalent, as defined by TSM, can be beneficial for compression of blocks with certain properties. For example, in inter frame prediction the properties of the residual signal varies and depends on many factors, including extent and type of motion in the sequence, level of quantisation, accuracy of motion estimation, etc. Such signals are not necessarily best compressed using 2D transform and forcing the 2D transform can limit the effectiveness of DCT as a coding tool. This realization was the motivation for considering alternative, low complexity solutions, i.e. for the design presented in this contribution.

Table 1. TSM modes with its corresponding transform choices and codewords

| TSM | Transform on rows | Transform on columns | Code words (CABAC and CAVLC) | Note |
| --- | --- | --- | --- | --- |
| TS0 | + | + | 1 | 2D transform |
| TS1 | + | - | 01 | 1D transform |
| TS2 | - | + | 001 | 1D transform |
| TS3 | - | - | 000 | no transform |

Four transform skip modes illustrated in Figure 1 are further summarized in Table 1. In addition to the TS0 mode, which corresponds to the 2D transform, three new transform modes are defined: skipping vertical transform (TS1), skipping horizontal transform (TS2) and skipping both horizontal and vertical transforms (TS3). Each transform unit (TU) that falls into a category on which TSM is used (e.g. inter coded blocks) is at the encoder side assigned one of the four available modes and the choice must be signalled to the decoder.

Main benefits of the proposed solutions are:

- better adaptation to underlying signal in the transformation stage of the coding, and

- reduced complexity required for the transformation stage at the decoder (computationally heavy transform not needed for some directions).

# Implementation design

The implementation of the support of TSM mode in HM3.2 was guided by the following principles:

- compatibility with HM3.2 core transform [E243]

  - main transform functions (void partialButterflyN( )) are left unchanged,

  - 16 bit intermediate data representation is preserved,

- quantisation is left unchanged for all TSM choices,

- scaling of coefficients when transform is used derived from norms of related transform vectors

  - integer multipliers are used for scaling.

The integer DCT is in HM3.2 implemented as two separate one dimensional transforms - first to the rows of the residual, then to columns. This basic design allows for simple implementation of the support for TSM - to support TSM each of the two stages has to be used or not, depending on the skip choice.

If the transform is skipped, then a scaling is applied to preserve signal levels. The scaling is derived from the transform vector norms. Important feature of the HM3.2 core transform is that it has nearly identical norms of all transform bases. This significantly contributes to the scaling design since same scaling factors can be used for all coefficients in a row/column on which the transform is skipped.

Scaling is at both encoder and decoder performed as follows:

y = (x  scale + offset) >> shift,	(1)

where scale is the scaling factor, shift and offset are the same as in the core transform [E243], i.e. the same values that would be used if the transform was not skipped. x is the value of an element of the row or column for which the transform is skipped, and y is its scaled equivalent.

Scaling of rows and columns uses the same scaling factors. For N  N blocks the factors are defined in Table 1. Note that for block sizes 4  4 and 16  16 the scaling factors correspond to 27 and 28, respectively, so in those cases (1) can be further simplified by removing actual multiplication stage.

Table 1. Scaling factors for N  N blocks.

| N | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| scale | 128 | 181 | 256 | 362 |

Note that the same scaling operation described above is carried out for the forward and inverse skipped transform.

When implemented as in the described scaling fashion and on the top of HM3.2 core transform, the proposed TSM approach does not require changes to the quantisation stage.

Modes TS1 and TS2 generate a different distribution of transformed coefficients across the block to the one generated by TS0. In particular, TS1 tends to concentrate the coefficients towards the left edge of the block while TS2 towards the top of the block. This particular distribution across the block can be exploited by utilising a different coefficient scanning pattern for TS1 and TS2; in particular a vertical scanning order (by columns) is used for TS1 and a horizontal scanning order (by rows) is used for TS2. For the remaining modes, zig-zag scanning is used.

# Tests

The proposed skipping of transforms in the directions defined by the TSM has been implemented in HM3.2. The implementation is valid under RQT splitting disabled for inter coded blocks (--QuadtreeTUMaxDepthInter=1), while intra coded blocks are unaffected by the TSM choice. This means that 2D transform comprising of DCT/DST combinations is applied on all intra coded blocks, with the default RQT settings (depth 3). The transform skip mode is applied jointly on Y, U and V components of a block.

The tests were performed for all inter coding configuration [E700] and the results are compared to HM3.2 anchors, Table 2. Visual inspection of decoded sequences did not show any distortions patterns that have not been common to the HM3.2 anchors at the same test points.

Table 2. Test results for coding with TSM, in comparison to HM3.2 anchors

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5 | 0.6 | 0.3 | 1.1 | 0.1 | 0.4 |
| Class B | 0.2 | -0.2 | 0.2 | 0.5 | -0.9 | 0.0 |
| Class C | -0.1 | 0.1 | 0.4 | 0.2 | -0.3 | 0.4 |
| Class D | -0.4 | 0.4 | 0.5 | 0.0 | 0.2 | 0.5 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.2 | 0.3 | 0.4 | -0.3 | 0.3 |
| Enc Time[%] | 99% |  |  | 109% |  |  |
| Dec Time[%] | 95% |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.9 | 0.6 | -2.4 | 0.0 |
| Class C | -0.5 | 0.0 | 0.5 | -0.1 | -0.6 | 1.0 |
| Class D | -1.2 | 1.4 | 1.6 | -0.5 | 0.9 | 1.4 |
| Class E | -0.5 | 0.5 | 0.0 | 0.0 | -0.5 | -0.9 |
| Overall | -0.5 | 0.5 | 0.8 | 0.0 | -0.7 | 0.4 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.7 | -0.5 | 0.5 | -2.3 | 0.2 |
| Class C | -0.6 | -0.3 | 0.2 | -0.3 | -0.6 | 0.7 |
| Class D | -1.4 | 0.9 | 1.1 | -0.9 | 0.6 | 0.8 |
| Class E | -0.8 | 0.2 | 0.2 | -0.1 | 0.4 | -0.5 |
| Overall | -0.7 | 0.0 | 0.2 | -0.2 | -0.6 | 0.3 |
| Enc Time[%] | 106% |  |  | 109% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The results presented in Table 2 show that the new approach leads to small average gains over HM3.2. Additionally, the same results are presented in Table 3, in comparison to HM3.2 with RQT disabled for inter coded blocks, which is a customized HM3.2 configuration on top of which the TSM support is implemented. In this way only the impact of the TSM is evaluated, in comparison to its underlying HM3.2 setting.

The results in Table 3 show that TSM selection at the encoder increases the complexity by about 20 %. This increase is due to RDO search that performs each choice of TSM mode on given inter coded CU. In the same fashion as for other corresponding HM coding options, the RDO check for each TSM combination involves performing forward TSM transform choice, RDOQ, entropy coding, inverse quantization and performing inverse TSM transform choice for Y, U and V components.

Decoding times are in the range of the anchor's times. For all configurations an average BD-rate gain is observed (from 0.5 % for ra_lc to 1.4 % for ldP_he).

Table 3. Test results for coding with TSM, in comparison to a customized HM3.2 setting 
(RQT off for inter coded blocks) on top of which the TSM support is implemented.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | -0.2 | 0.3 | 0.0 | 0.2 |
| Class B | -0.6 | -0.7 | -0.3 | -0.5 | -0.4 | 0.1 |
| Class C | -0.8 | -0.2 | 0.0 | -0.7 | -0.1 | 0.3 |
| Class D | -1.1 | 0.3 | 0.3 | -1.1 | 0.4 | 0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.6 | -0.2 | -0.1 | -0.5 | 0.0 | 0.3 |
| Enc Time[%] | 119% |  |  | 122% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -1.1 | -0.8 | -0.7 | -0.9 | -0.1 |
| Class C | -1.5 | -0.7 | -0.1 | -1.4 | -0.4 | 0.6 |
| Class D | -2.2 | 0.6 | 1.0 | -2.1 | 1.2 | 1.2 |
| Class E | -0.8 | 1.0 | 0.3 | -0.8 | 0.9 | 0.4 |
| Overall | -1.3 | -0.2 | 0.0 | -1.2 | 0.1 | 0.5 |
| Enc Time[%] | 116% |  |  | 110% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -1.6 | -1.5 | -0.5 | -0.6 | 0.2 |
| Class C | -1.5 | -0.9 | -0.4 | -1.4 | -0.3 | 0.5 |
| Class D | -2.2 | 0.5 | 0.3 | -2.2 | 0.9 | 0.5 |
| Class E | -1.1 | 0.3 | 0.5 | -0.9 | 1.5 | 0.7 |
| Overall | -1.4 | -0.5 | -0.4 | -1.2 | 0.2 | 0.4 |
| Enc Time[%] | 128% |  |  | 127% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Bit-stream modifications and semantics

This section describes the modification to the HEVC syntax that this proposal requires. The changes are described in reference to the latest version of The Working Draft [E603].

Modifications to 7.3.9 Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode != MODE_INTRA && cIdx = = 0 ) |  |
| tsmIdx[ x0 ][ y0 ][ trafoDepth ] | ae(v) ¦ ce(v) |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS1 ) { |  |
| scanIdx = 2 |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS2 ) { |  |
| scanIdx = 1 |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

Addition to 7.4.9 Transform coefficient semantics

tsmIdx[ x0 ][ y0 ][ trafoDepth ] signals the choice of Transform Skip Mode (TSM) to use for the transform block for the luma and chroma components. The array indices x0 and y0 specify the location (x0, y0) of the top luma sample of the considerd block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding. Four possible values are allowed for tsmIdx[ x0 ][ y0 ][ trafoDepth ], as shown in the table below. The TSM index is only signalled when PredMode is different to MODE_INTRA and only for the luma component (cIdx = = 0).

When tsmIdx[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to TS0.

| tsmIdx | Transform on rows | Transform on columns | Scan Idx | Note |
| --- | --- | --- | --- | --- |
| 0 | + | + | 0 | 2D transform |
| 1 | + | - | 2 | 1D transform |
| 2 | - | + | 1 | 1D transform |
| 3 | - | - | 0 | no transform |

A transform skip mode set to 1 requires a scanning index scanIdx = 2, i.e. the vertical scanning. A transform skip mode set to 2 requires a scanning index scanIdx = 1, i.e. the horizontal scanning.

Notes related to 8.5.4 Transformation process for scaled transform coefficients, assuming HM3.x core transform design [E243]

Inverse transformation process for scaled coefficients follows 8.5.4 for tsmIdx = = TS0. The following describes the inverse transformation process for the other three Transform Skip Modes, TS1, TS2 and TS3.

TS1: the first stage of the inverse transform, applied to each vertical column of the scaled transformed coefficients is skipped. Each coefficient is normalised using the following formula instead:

ri,j = (fi,j * scale + 64) >> 7, with i,j=0..(nS)x(nS)-1

Each horizontal row of the normalised scaled transformed coefficients is then transformed as described in [E603/E243], limited to the horizontal stage only.

TS2: each vertical column of scaled transform coefficients is transformed as described in [E603/E243], limited to the vertical stage only. The second stage of the inverse transform is skipped and each coefficient is normalised using the following formula:

ri,j = (fi,j * scale + 211-trafoPrecisionExt ) >> ( 12 - trafoPrecisionExt ), with i,j=0..(nS)x(nS)-1

TS3: both stages of the transformation process, horizontal and vertical, are skipped. The scaled transformed coefficients are instead normalised using the following formula:

ri,j = (fi,j * scale * scale + 218-trafoPrecisionExt ) >> (19 - trafoPrecisionExt), with i,j=0..(nS)x(nS)-1

The value of scale depends on the block size and is described in the table below.

| N | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| scale | 128 | 181 | 256 | 362 |

Notes related to 9 Parsing process

Table 9-X Codeword / binarization for tsmIdx

| Value of tsmIdx | Codeword |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 000 |

Parsing for syntax element tsmIdx[ x0 ][ y0 ][ trafoDepth ] defined in subclause 7.4.9 is done in a similar fashion for both entropy coders. It is invoked if PredMode ! = MODE_INTRA and cIdx = = 0. Output of this process is the syntax element tsmIdx[ x0 ][ y0 ][ trafoDepth ]. Otherwise if PredMode = = MODE_INTRA or it is a chroma component and cbf_luma[ x0 ][ y0 ][ trafoDepth ] is 0, tsmIdx[ x0 ][ y0 ][ trafoDepth ] is set to 0.

The parsing process when entropy_coding_mode_flag is equal to 0, i.e. for CAVLC, is according to subclause 9.2.1, with vlcNum = 14 and cMax = 3.

Related addition to 9.3 CABAC parsing process for slice data

Inputs to this process are PredMode, cIdx and log2TrafoSize.

Table 9-Y contains the values of variables n and m used in the initialisation of context variable assigned to syntax element tsmIdx.

Table 9-Y Values of variable m and n for tsmIdx ctxIdx

| Initialisation variables | tsmIdx ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |

For tsmIdx , the ctxIdx is derived from log2TrafoSize and binIdx:

ctxIdx = (log2TrafoSize − 2) * 3 + binIdx

# Conclusions

Proposed method enables skipping of transform on rows and/or columns of a block, with a goal to enhance RD performance of a codec. The choice of performing the transform in either direction is defined by the Transform Skip Mode. This approach was tested in HM3.2 for inter coded blocks with disabled RQT and demonstrates comparable performance to the HM default solution which uses RQT with the depth of 3 on inter coded blocks.

# Patent rights declaration(s)

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[E125]	A. Saxena, F. C. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction”, JCTVC-E125, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

[E243]	A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16-bit intermediate data representation”, JCTVC-E243, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

[E603]	Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “WD3: Working Draft 3 of High Efficiency Video Coding”, revision 8, JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

[E700]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.