| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F078 |
| --- | --- |

| Title: | Cross verification report for JCTVC-F075 proposed by Zenverge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ximin Zhang Shan Liu 2860 Junction Ave. San Jose, CA95134, USA | Emails: | ximin.zhang@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This document reviews Zenverge’s JCTVC-F075 proposal for reference picture memory compression. In the proposed method, a content adaptive offset for each 4x4 block is used for reference frame compression. The verification task has been conducted, and the BD-rate results provided by Zenverge have been verified.

# Verification

Zenverge provided the HM-3.0-based source code of their proposed codec as described in JCTVC-F075 Error: Reference source not found which is essentially the same as their previous contribution JCTVD-D035. Zenverge provided us the software to generate the results for both the proposed method and the anchor.

The results for the proposed method were generated by setting the following parameter in the provided code:

#define ENABLE_ZENVERGE_RFC                        1

#define ENABLE_ZENVERGE_RFC_FAST                  1

#define ENABLE_ZENVERGE_RECON_OUT_BEFORE_RFC    0

The results for Zenverge anchor were generated by setting the following parameter in the provided code:

#define ENABLE_ZENVERGE_RFC                        0

#define ENABLE_ZENVERGE_RFC_FAST                  0

#define ENABLE_ZENVERGE_RECON_OUT_BEFORE_RFC    0

Two configurations (random access HE, low delay HE) were run for the test with the provided code.

Note that Zenverge anchor includes decoder side memory bandwidth measurement tools. As a result the decoder run time of Zenverge anchor is averagely about 62% longer than that of HM3.0.

# Results

The BD-rates obtained from our experimental results match those provided by Zenverge, compared with Zenverge anchor. There are very small BD-rate differences if using HM3.0 anchor. The results are listed in JCTVC-F078_zen.xls and JCTVC-F078_HM30.xls, respectively. The average decoder run time reported about 6% increase than the anchor provided by Zenverge.

# Conclusion

The results presented by Zenverge in JCTVC-F075 are verified. BD-rates are confirmed. Decoder run time reports slight increase, i.e. 6% increase compared with Zenverge anchor.

# References

- Dzung Hoang, “Unified scaling with adaptive offset for reference frame compression”, JCTVC-F075, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Torino, Italy, July 2011.