| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F080 M20492 |
| --- | --- |

| Title: | CE9: Cross-check results on SP05, SP06, SP08, SP13, PART16 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for CE9 tests of SP05, SP06, SP08, SP13 and SP16. It has been verified that the results reported by proponents are accurate. The source code has also been studied to understand the design detail of theproposed tools.

# Introduction

This contribution reports the cross-check results of the following CE9 tests [3]:

- SP05: reducing the occurrence of spatial MVP scaling in AMVP list construction . (Mitsubishi)

- SP06: MVP candidates with different reference pictures are considered only when no MVP candidate with a same reference picture can be found for all positions to be scanned. (LGE & Mitsubishi)

- SP08: disabling TMVP for 4x4, 4x8, 8x8 PUs in merge/skip MVP list, and in AMVP list (JVC KENWOOD)

- SP13: the merge-Skip mode uses only a subset of merge motion candidates (left and top). The related experiment is. (I2R)

- PART16: disabling partial CU merge and forcing 4x4 PUs to use merge mode. (MediaTek)

# Algorithm Description

Based on understanding of source code provided proponents, those tools operate differently from the current HM3.0 design as follows:

- SP05: performing spatial MVP scaling for only bottom-left (E) and upper-right (C) neighbouring PU positions, skipping the operation for A0, B0 and D as shown in Figure 1.  This effectively reduces the number of spatial MVP scaling from four to two. (Mitsubishi)

- SP06: for left and upper side the MVP scanning is changed to the following order: scanning the MVP of same reference picture & same reference frame first following by the MVP scanning of same reference picture and different list. Finally, if MVPs of same picture are not found, MVPs of different reference picture are scanned. This MVP scanning process is performed for the left side first followed by the upper side, same as the current HM3.0 design. This tool may reduce software complexity in average case, but does not necessarily reduce implementation cost because the worst case number of spatial MVP scaling is still the same. (LGE & Mitsubishi)

- SP08: disabling temporal MVP for 4x4, 4x8 and 8x8 PUs in merge/skip MVP list, and in AMVP list derivation processing. This reduces complexity for small blocks. However, since the spatial MVP scaling still exists in AMVP, this tool would need to be combined with disabling spatial MVP scaling for small blocks to achieve overall throughput improvements.  (JVC KENWOOD)

- SP13: only left and top MVP candidates are allowed for merge-skip mode, the merge mode still uses up to five candidates. This makes the skip MVP list construction simpler on the decoder side, but won’t reduce the worst case decoder complexity because the merge mode still uses up to five candidates. For practical encoder implementations, this tool does not necessarily reduce the encoder implementation cost because merge/skip may share the same logic during the motion estimation and motion compensation process. (I2R)

- PART16: disabling partial CU merge and forcing 4x4 PUs to use merge mode. This is an interesting tool compared to the current partial CU merge mode. (MediaTek)

Current PU

A0

E

B0

C

D

Co-located PU

CR

1

2

H

Figure 1. Illustration of  the HM3.0 AMVP list construction

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [1] are followed. The simulation platform at Texas Instruments is LSF equipped with Intel(R) Xeon(R) CPUs bits Linux machines of different frequencies.

# Experimental results

A summary of testing results are provided in Table 1, and the detailed results can be found In Table 2 to Table 6. Those results perfectly match the results reported by proponents.

| Test | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- |
| SP05 | 0.1 | 0.2 | 0.0 | 0.0 |
| SP06 | 0.0 | 0.0 | 0.0 | 0.0 |
| SP08 | 0.0 | 0.0 | 0.1 | 0.1 |
| SP13 | 0.1 | 0.1 | 0.1 | 0.1 |
| PART16 | -0.5 | -0.5 | -0.2 | -0.3 |

Table 1.  Average BD-rate increase relative to HM3.0 anchor for SP05, SP06, SP08, SP13 and PART16

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class B | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 |
| Class D | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 |
| Class D | -0.1 | -0.4 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | 0.7 | -0.5 | -0.1 | 0.2 | 0.1 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 94% |  |  | 98% |  |  |

Table 2.  BD-rate increase relative to HM3.0 anchor for SP05

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.2 | 0.0 | -0.1 | 0.0 | -0.2 |
| Class D | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | -0.2 |
| Class E | 0.0 | 0.2 | 0.0 | -0.1 | -0.2 | 0.3 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 100% |  |  |

Table 3.  BD-rate increase relative to HM3.0 anchor for SP06

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.3 | 0.1 | 0.0 | -0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class C | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.0 | 0.3 | 0.2 | 0.1 | -0.1 |
| Class E | 0.0 | 0.7 | 0.2 | -0.1 | 0.0 | 0.3 |
| All | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 94% |  |  | 99% |  |  |

Table 4.  BD-rate increase relative to HM3.0 anchor for SP08

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 |
| Class B | 0.1 | 0.2 | 0.1 | -0.1 | -0.3 | -0.2 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.2 | 0.3 | 0.2 | 0.2 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.2 | 0.2 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | 0.0 | 0.0 | -0.6 | -0.6 |
| Class C | 0.1 | 0.3 | 0.1 | 0.2 | -0.1 | -0.1 |
| Class D | 0.0 | 0.1 | 0.7 | 0.2 | -0.6 | -0.5 |
| Class E | 0.4 | 0.4 | 0.6 | 0.0 | -0.8 | 0.0 |
| All | 0.1 | 0.3 | 0.3 | 0.1 | -0.5 | -0.3 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 102% |  |  |

Table 5.  BD-rate increase relative to HM3.0 anchor for SP13

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.8 | -1.0 | -0.7 | -0.7 | -0.7 |
| Class B | -0.4 | -0.5 | -0.5 | -0.4 | -0.3 | -0.3 |
| Class C | -0.6 | -0.6 | -0.6 | -0.5 | -0.4 | -0.3 |
| Class D | -0.4 | -0.4 | -0.4 | -0.3 | -0.2 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.6 | -0.6 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 111% |  |  | 105% |  |  |
| Dec Time[%] | 114% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.5 | -0.4 | -0.3 | -0.2 | -0.1 |
| Class C | -0.3 | -0.4 | -0.4 | -0.5 | -0.4 | -0.4 |
| Class D | -0.3 | -0.3 | 0.3 | -0.2 | -0.1 | -0.2 |
| Class E | -0.1 | 0.2 | -0.4 | -0.3 | 0.2 | 0.4 |
| All | -0.2 | -0.3 | -0.2 | -0.3 | -0.2 | -0.1 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 97% |  |  | 106% |  |  |

Table 6.  BD-rate increase relative to HM3.0 anchor for PART16

# Conclusions

It has been verified that the results of CE9 SP05, SP06, SP08, SP13 and PART16 reported by proponents are accurate. The source code has also been checked to understand the accuracy and design detail of the proposed tools.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] Y.-W. Huang (MediaTek), B. Bross (HHI), M. Zhou (TI), W.-J. Chien (Qualcomm), I.-K. Kim (Samsung),    “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations”,  JCT-VC Document, JCTVC-E709, Geneva, April 2011