| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F081 M20493 |
| --- | --- |

| Title: | CE1: Evaluation results on A.09, A.13-16 and an alternative solution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

# This contribution reports CE1 results for A.13 -16, and verifies CE1 A.9 results reported by LGE. In addition, an alternative TMVP position is proposed to minimize both the memory bandwidth and coding loss for TMVP derivation. For the best combination among A.9 and A.13-16, an average loss of 0.3% in RA-HE and RA-LC, and 0.5% in LB-HE and LB-LC was observed. The alterative solution can cut the average loss by 0.1% by a combined use of BR positions and those H positions still located inside the current LCU. Additional results also reveal that there is no coding loss by dropping centre TMVP position from the current design. Based on the simulation results, it is recommended to remove center (CR) TMVP position from the current design and continue CE on seeking a better combination of MVSR and TMVP positions for purpose of memory bandwidth reduction.

# Introduction

The first part of this document reports the CE1 results for A.9, A.13, A.14 and A.15 as described in Table 1. The definition of motion vector sampling representation (MVSR) positions for motion data compression and temporal MVP predication positions for TMVP derivation is depicted in Figure 1 and Figure 2 [3]. Category A CE1 tests are designed to find out the best combination of MVSR position and TMVP position.

| Experiment | MVSR (position) | Prediction (TMVP position) | Tester | Checker |
| --- | --- | --- | --- | --- |
| Anchor | HM3.0 (TL) | HM3.0 (H or C0) | - | - |
| A.9 | BR | C3 | LGE | TI |
| A.13 | HM3.0 | BR | TI | Samsung |
| A.14 | BR | BR | TI | Orange Labs |
| A.15 | C0 | BR | TI | Samsung |
| A.16 | C3 | BR | TI | Samsung |

Table 1: Summary of the experiments for category A presented in this document (from [3])

Figure 1: Description of the positions for MDSR. (from [3])

Figure 2: Description of the positions for TMVP. (from [3])

In the current HM3.0 design the TMVP prediction positions (CR, H) are misaligned with the MVSR position of motion data compression, such a misalignment leads to higher memory bandwidth for pre-fetch of motion data for the TMVP derivation.

The second part of this document analyzes drawbacks of the current HM3.0 design and provides alternative solution.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of various frequencies, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results for CE1

The test results for A.09, A.13, A.14. A.15 and A.16 are summarized in Table 2. Where In Table 2 A.09 result verifies the result reported by LGE. As shown in Table 2, for the best combination among those five tests, there is an average loss of 0.3% in RA-HE and RA-LC, and 0.5% in LB-HE and LB-LC when compared to the HM3.0. Please refer to the attached spreadsheets for detailed results.

| Experiment | MVSR (position) | Prediction (TMVP position) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- | --- | --- |
| A.09 | BR | C3 | 0.3 | 0.3 | 0.5 | 0.5 |
| A.13 | HM3.0 | BR | 0.7 | 0.7 | 0.9 | 1.0 |
| A.14 | BR | BR | 0.3 | 0.3 | 0.5 | 0.5 |
| A.15 | C0 | BR | 0.7 | 0.7 | 0.9 | 0.9 |
| A.16 | C3 | BR | 0.3 | 0.3 | 0.5 | 0.5 |

Table 2, BD-Rate comparison relative to HM3.0 anchor for CE1 A.09, A.13, A.14, A.15 and A.16

# Analysis of HM3.0 design and alternative solutions

The major drawback of the current HM3.0 for the TMVP derivation is that the co-located motion data fetch area is not aligned with LCUs. Because of H TMVP position, the co-located motion data fetch area can go beyond LCU boundaries. As shown in Figure 3 (a), with the current HM3.0 design in which both CR and H TMVP positions are involved, an 80x80 co-located motion data area (red box in Figure 3 (a)) need to be pre-fetched for a LCU of 64x64 size. The 80x80 co-located area touches motion data of four LCUs in the co-located frame. If the motion data is compressed and stored in tile format of LCU size (each tile covers motion data of entire LCU), this would imply a much higher memory bandwidth because each LCU requires pre-fetech of four LCUs amount of motion data when DDR burst alignments are considered. Dropping CR TMVP will reduce the pre-fetched motion data area from 80x80 to 64x64, but the fetched area still involves in four LCUs as shown in Figure 3 (b). It won’t save memory bandwidth if the motion data is stored in 64x64 LCU tile format.

LCUs

(a) HM3.0

(b) H only

( c) Desired

Co-located motion data fetch area

Figure 3, Illustration of spatial relation between LCU and co-located motion data fetch area

In both cases (i.e Figure 3 (a) and (b)), a better motion data storage format is to store the compressed motion data in tile format of 16x16 (i.e. each tile covers motion data of a 16x16 region).  However, analysis on the motion data size after compression reveals that it is hard to store compressed motion data in 16x16 tile format with DDR burst alignment.

As shown in Table 3, after compression motion data size for each 16x16 block area is about 74 bits, roughly 10 bytes, but the DDR burst size is normally 8, 16 or 32 bytes, etc. This requires that multiple 16x16 motion data be grouped together for storage for better alignment with DDR burst. Larger tile size implies higher memory bandwidth of motion data fetch for case (a) and (b) of Figure 3.

| Motion data | Size of data after compression for 16x16 block (bits) |
| --- | --- |
| Motion vectors | 64 (2 x 2 x 16) |
| Intra/inter flag + Prediction direction | 2 |
| Ref_idx | 8 ( 2 x 4) |
| Total for 16x16 (bits) | 74 (bits) = ~10 (bytes) |
| Total for 64x64 LCU (bytes) | 148 (bytes) |

Table 3  Motion data size after compression for 16x16 and 64x64 block size

The desired combinations of MVSR and TMVP position is illustrated in Figure 3 (c), in which the co-located motion data fetch area is exactly the same as the co-located LCU area in the reference frame. Having such an alignment is advantageous for minimizing the memory bandwidth spent on storing and fetching the compressed motion data, because the motion data can be comfortably stored and consumed in 64x64 tile format to ensure DDR burst alignments and avoid page crossing. As shown in Table 3, after compression a 64x64 LCU has 148 bytes for motion data, which is roughly 19, 10, and 5 bursts for DDR burst  size of 8, 16, 32 bytes, respectively.

Any TMVP position inside the current PU such as C0, C3 and BR defined in Figure 2 can avoid co-located motion data area for a LCU going beyond LCU boundaries.  In fact, as long as the co-located motion data fetch area for a LCU can be constrained to be fully inside the co-located LCU area in the reference frame, the memory bandwidth for TMVP derivation can be minimized.

PU0

H TMVP positions

Bottom-right TMVP positions (BR)

(inside LCU&PU)

PU1

PU2

PU6

PU3

PU7

PU4

PU5

PU8

CU0

CU1

CU2

CU3

CU/PU partitioning of the current LCU

Bottom neighboring LCU

Bottom-right neighboring LCU

Right neighboring LCU

Figure 4, Illustration of H’ + BR TMVP position, if H position is outside the current LCU, BR position is used instead

An alternative solution is illustrated in Figure 4, in which the current H TMVP positions (grey blocks in Figure 4) are still used as long as they are located inside the current LCU. For those H positions which go beyond the LCU boundaries, bottom-right TMVP positions (pink blocks in Figure 4) are used instead. With this combination, the pre-fetched motion data area is still totally within the co-located LCU. This combination of TMVP positions is namedas H’+BR position in the following text.

| Experiment | MVSR (position) | Prediction (TMVP position) | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) |
| --- | --- | --- | --- | --- | --- | --- |
| S01 | HM3.0 | H’+BR | 0.3 | 0.3 | 0.5 | 0.4 |
| S02 | BR | H’+BR | 0.2 | 0.2 | 0.4 | 0.4 |
| S03 | C0 | H’+BR | 0.3 | 0.3 | 0.4 | 0.4 |
| S04 | C3 | H’+BR | 0.2 | 0.2 | 0.4 | 0.4 |
| S05 | HM3.0 | H | 0.0 | 0.0 | 0.0 | -0.1 |
| S06 (w/o motion data compression) | HM3.0 | HM3.0 | -0.1 | -0.1 | -0.1 | -0.1 |

Table 4, BD-Rate comparison relative to HM3.0 anchor

The experimental results are summarized in Table 4. Compared to the best combination in Table 2, the proposed alternative combination can cut average loss by 0.1% (see S02 in Table 4 and A14 in Table 2). Nevertheless, this is still a noticeable loss with this alternative combination, especially in low-delay configurations (0.4% loss observed).

S05 result in Table 4 also reveals that dropping CR TMVP position from the current design seems to the best solution available. It does not cause the loss but can reduce the pre-fetched motion data area though the memory bandwidth for TMVP derivation is not minimized.

S06 result in Table 4 reveals that the loss caused by 16x motion data compression is barely a minimum with the current design. (0.1% observed).

For detailed results please refer to attached spreadsheets.

# Conclusions and recommendations

This contribution reports CE1 results for A.13 -16, and verifies CE1 A.9 results reported by LGE. In addition, an alternative TMVP position is proposed for minimizing both the memory bandwidth and coding loss of TMVP derivation. Based on the simulation results, it is recommended to drop center (CR) TMVP position from the current design and continue CE on seeking a better combination of MVSR and TMVP positions for purpose of memory bandwidth reduction.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] J. Jung, P. Onno , “Description of Core Experiment 1 (CE1): Motion Data Storage Reduction”, JCT-VC Document, JCTVC-E701, Geneva, April 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).