| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F082 M20494 |
| --- | --- |

| Title: | CE9: A study on partial CU merge (PART13, PART18) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Tel: Email: | +1 214.480.3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

The partial CU merge mode is further investigated in CE9. Study shows that disabling partial CU merge leads to an average quality gain of 0.5% in RA-HE, 0.6% in RA-LC, 0.3% in LB-HE and 0.3% in LB-LC, respectively.  An alternative method which disables both partial CU merge and 4x4 inter prediction leads to an average quality gain of 0.4% in RA-HE, 0.4% in RA-LC, 0.1% in LB-HE and 0.1% in LB-LC without encoding time increase. Study performed on fast motion sequences shows a much larger loss of partial merge for random access configurations, an average loss of 2.5% and 2.8% are observed in RA-HE and in RA-LC, respectively.  Architecture study revivals that for practical encoder designs partial CU merge causes quality loss but does not necessarily save implementation cost. It is therefore recommended to remove partial CU merge from test model to address quality and encoder design concerns, and disable 4x4 inter prediction in the HM software to regain encoding time reduction.

# Introduction

This contribution reports the evaluation results on partial CU merge conducted in CE9.

# Partial CU merge

As shown in Figure 1 currently for partition size of 2NXN and Nx2N in CU larger than 8x8, the first PU is forced to merge. Partial merge was proposed as a method to decrease encoding time since motion estimation does not have to be performed on first PU for partition size 2NxN and Nx2N for CU>8x8.  In HM reference software partial CU merge achieves encoder speed up by skipping regular motion estimation for the 1st PU of 2NXN and Nx2N mode (N > 8).

(a) Vertical partial merging                   (b) Horizontal partial merging

Figure 1 CU Partial merging (figure from [3])

However, there are several concerns with this approach.

- the current CU may have totally different motion from its spatial and temporal neighbors; the forced merge could cause visual quality degradation and coding artifacts.

- for pipelined hardware architecture in which an encoder may choose to estimate the motion vectors for CUs in parallel, this forced merge mode causes design burden due to its irregularity if encoder wishes to use 2NxN and Nx2N prediction size (N > 4).

- an encoder may choose to support CU size only down to 16x16, the partial CU merge mode essentially disables 2NxN, Nx2N motion vector partitions altogether if the partial merge mode cannot be implemented on the encoder side due to architecture constraints.

- for MTU matching with decoupled entropy encoding (i.e. bitstream slicing) architecture, front end encoder may encode a frame in single slice mode, store motion data and DCT-coefficients in off-chip memory; the backend encoder then picks up the data and encodes them into multiple slices. When a slice header is inserted, neighboring availability can change for merge/skip mode. Therefore, there is a need that PUs of merge/skip mode can be encoded as regular inter prediction mode in order to avoid re-encoding those PUs which use merge/skip mode originally but their neighboring PU availability  or motion data  get impacted by insertion of slice header.  The partial CU merge essentially comprises such design flexibility and again forces encoder to give up on 2NxN, Nx2N (N > 8) prediction size.

Architecture study also shows that the software encoding time reduction of partial CU merge does not necessarily lead to the hardware cost reduction in practical encoder designs. On the contrary, for practical encoder implementations the partial CU merge can limit encoder design choice and cause quality loss without actual area saving.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the common test conditions and reference configurations specified in [1] are followed. The test settings are summarized in the table below.

| Tests | Description | Remark |
| --- | --- | --- |
| PART02 | Partial CU merge enabled, but merge flag for the 1st PU still encoded | quantify overhead of partial merge flag |
| PART12 | Disable partial CU merge in HM3.0 | quantify quality loss of partial merge |
| PART13 | Disable partial CU merge in HM3.0, reduce refinement search range to 1 for the motion estimation of the 1st PU of 2NxN, Nx2N prediction mode (N >8). | testing alternative solution for encoder time reduction, while retaining encoder choice |
| PART18 | Disable both partial CU merge and 4x4 inter prediction | testing alternative solution for encoder time reduction |
| Disabling 2NxN and Nx2N prediction | Disable 2NxN and Nx2N inter prediction (N > 8) | quantify quality loss of disabling 2NxN and Nx2N inter prediction |

Table 1, Test settings for the study

# Experimental results

The experimental results are summarized in Table 2. Compared to HM3.0 anchor, the major observations:

- The overhead of partial CU merge flag is around 0.2 – 0.3%. (PART02)

- Disabling partial CU merge leads to an average quality gain of 0.5% in RA-HE, 0.6% in RA-LC, 0.3% in LB-HE and 0.3 in LB-LC, respectively. But it also leads to 16% encoding time increase. (PART12)

- Disabling partial CU merge and enabling RCME has an overall 0.1% gain, but increases encoding time by 7%. (PART13)

- Disabling partial CU merge & 4x4 inter prediction leads to an average quality gain of 0.4% in RA-HE, 0.4% in RA-LC, 0.1% in LB-HE and 0.1 in LB-LC, respectively. The encoding time is almost same as the HM3.0 anchor. This setting is apparently a better coding efficiency and encoder time trade-off. (PART18)

- Disabling 2NxN and Nx2N inter prediction (N > 8) introduces noticeable loss, but can further reduce encoding time by another ~20%.

| Tests | RA-HE (%) | RA-LC (%) | LB-HE (%) | LB-LC (%) | Encoding time |
| --- | --- | --- | --- | --- | --- |
| PART02 | 0.2 | 0.2 | 0.3 | 0.3 | 100% |
| PART12 | -0.5 | -0.6 | -0.3 | -0.3 | 116% |
| PART13 | -0.1 | -0.1 | -0.1 | -0.1 | 107% |
| PART18 | -0.4 | -0.4 | -0.1 | -0.1 | 101% |
| Disabling 2NxN and Nx2N prediction | 0.8 | 1.0 | 0.9 | 1.3 | 80% |
| Disabling 2NxN and Nx2N prediction w.r.t HM3.0 anchor w/ partial merge disabled. | 1.3 | 1.6 | 1.1 | 1.6 | 68% |

Table 2, BD-Rate and encoding time comparison relative to HM3.0 anchor

The detailed results are provided in Table 5 to Table 10, and in attached spreadsheets.

Additional tests have shown that the partial CU merge could lead to much larger loss for fast motion sequences. Quality study results performed on test sequences used for product development are provided in Table 3 and Table 4. As shown in Table 3, for those sequences significant loss of partial merge has been observed for RA-HE and RA_LC configurations (2.5% and 2.8%, respectively). If the 2NxN, Nx2N (N >8) prediction modes are disabled, the aggregated quality loss amounts to 3.3% in RA-HE, 4.0% in RA-LC, 1.0% in LB-HE and 1.6% in LB-HC when compared to HM3.0 anchor with partial merge disabled (see Table 4).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | -1.6 | -2.3 | -2.1 | -2.4 | -3.0 | -3.0 |
| Class FH | -3.0 | -3.7 | -3.9 | -3.0 | -3.8 | -3.5 |
| All | -2.5 | -3.3 | -3.3 | -2.8 | -3.5 | -3.3 |
| Enc Time[%] | 112% |  |  | 116% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | -0.1 | -0.8 | -0.4 | -0.1 | 0.2 | -0.3 |
| Class FH | -0.2 | -0.2 | -0.3 | -0.2 | 0.0 | 0.2 |
| All | -0.2 | -0.4 | -0.3 | -0.2 | 0.1 | 0.0 |
| Enc Time[%] | 114% |  |  | 117% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3.  BD-rate comparison relative to HM3.0 anchor when partial CU merge is disabled (fast motion sequences)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 2.2 | 3.4 | 3.0 | 3.5 | 3.9 | 3.9 |
| Class FH | 3.8 | 4.4 | 4.5 | 4.3 | 5.1 | 4.8 |
| All | 3.3 | 4.1 | 4.0 | 4.0 | 4.7 | 4.5 |
| Enc Time[%] | 71% |  |  | 65% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.8 | 1.3 | 1.4 | 1.4 | 0.9 | 0.8 |
| Class FH | 1.2 | 0.9 | 1.1 | 1.7 | 1.0 | 1.1 |
| All | 1.0 | 1.1 | 1.2 | 1.6 | 1.0 | 1.0 |
| Enc Time[%] | 71% |  |  | 66% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 4.  BD-rate comparison relative to HM3.0 anchor w/ partial CU merge is disabled when 2NxN and Nx2N (N>8) modes are disabled (fast motion sequences)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class B | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class C | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 | 0.2 |
| Class D | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.4 | 0.2 | 0.3 | 0.2 | 0.3 |
| Class C | 0.2 | 0.3 | 0.3 | 0.2 | 0.3 | 0.1 |
| Class D | 0.1 | 0.4 | 0.8 | 0.2 | 0.1 | 0.3 |
| Class E | 0.5 | 0.5 | 0.7 | 0.6 | 0.4 | 1.1 |
| All | 0.3 | 0.4 | 0.5 | 0.3 | 0.2 | 0.4 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 94% |  |  | 102% |  |  |

Table 5.  BD-rate comparison relative to HM3.0 anchor when partial CU merge is enabled but merge flag for 1st PU still encoded (PART02)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -1.1 | -1.0 | -0.7 | -0.7 | -0.7 |
| Class B | -0.4 | -0.5 | -0.5 | -0.4 | -0.3 | -0.3 |
| Class C | -0.6 | -0.6 | -0.8 | -0.7 | -0.6 | -0.6 |
| Class D | -0.5 | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.7 | -0.7 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 116% |  |  | 116% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.3 | -0.6 | -0.3 | -0.2 | -0.2 |
| Class C | -0.4 | -0.4 | -0.6 | -0.5 | -0.3 | -0.4 |
| Class D | -0.4 | -0.4 | 0.2 | -0.3 | -0.2 | -0.2 |
| Class E | -0.1 | 0.2 | -0.1 | -0.1 | 0.5 | 0.4 |
| All | -0.3 | -0.3 | -0.3 | -0.3 | -0.1 | -0.1 |
| Enc Time[%] | 114% |  |  | 122% |  |  |
| Dec Time[%] | 95% |  |  | 106% |  |  |

Table 6.  BD-rate comparison relative to HM3.0 anchor when partial CU merge is disabled (PART12)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.1 | -0.1 | 0.2 | 0.1 |
| Class B | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | 0.1 |
| Class C | -0.2 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class D | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.2 | -0.1 | -0.1 | 0.0 | -0.1 |
| Enc Time[%] | 107% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.2 |
| Class C | -0.1 | 0.1 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class D | -0.2 | -0.3 | 0.6 | -0.2 | -0.4 | -0.3 |
| Class E | -0.1 | -0.4 | -0.2 | 0.0 | 0.6 | 0.6 |
| All | -0.1 | -0.1 | 0.1 | -0.1 | 0.0 | 0.1 |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 7.  BD-rate comparison relative to HM3.0 anchor when partial CU merge is disabled and RCME is used for 1st PU of 2NxN and Nx2N mode (N >8) (PART13)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.8 | -0.9 | -0.6 | -0.5 | -0.6 |
| Class B | -0.3 | -0.5 | -0.5 | -0.4 | -0.3 | -0.3 |
| Class C | -0.5 | -0.4 | -0.5 | -0.4 | -0.2 | -0.2 |
| Class D | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.5 | -0.4 | -0.3 | -0.3 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.6 | -0.2 | -0.1 | -0.1 |
| Class C | -0.2 | 0.0 | -0.2 | -0.2 | 0.0 | -0.1 |
| Class D | 0.0 | 0.1 | 0.6 | 0.1 | 0.3 | -0.1 |
| Class E | -0.1 | -0.4 | -0.2 | -0.2 | 0.2 | 0.3 |
| All | -0.1 | -0.1 | -0.1 | -0.1 | 0.1 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 8.  BD-rate comparison relative to HM3.0 anchor when both partial CU merge and 4x4 inter prediction are disabled (PART13)

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.0 | 0.9 | 1.1 | 1.0 | 0.7 |
| Class B | 0.7 | 0.8 | 0.7 | 1.1 | 0.9 | 0.9 |
| Class C | 0.8 | 0.9 | 0.9 | 1.0 | 0.9 | 1.1 |
| Class D | 0.8 | 0.7 | 0.7 | 0.8 | 0.6 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 0.8 | 0.8 | 0.8 | 1.0 | 0.9 | 0.9 |
| Enc Time[%] | 83% |  |  | 76% |  |  |
| Dec Time[%] | 106% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 1.2 | 1.0 | 1.4 | 0.9 | 0.9 |
| Class C | 0.8 | 1.1 | 1.2 | 1.0 | 0.9 | 0.9 |
| Class D | 0.8 | 0.6 | 1.5 | 0.9 | 1.0 | 0.8 |
| Class E | 1.2 | 1.9 | 1.5 | 2.3 | 1.7 | 1.9 |
| All | 0.9 | 1.2 | 1.3 | 1.3 | 1.0 | 1.1 |
| Enc Time[%] | 82% |  |  | 77% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 9.  BD-rate comparison relative to HM3.0 anchor when 2NxN and Nx2N inter prediction is disabled.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.3 | 2.1 | 1.9 | 1.8 | 1.7 | 1.4 |
| Class B | 1.1 | 1.3 | 1.2 | 1.6 | 1.2 | 1.2 |
| Class C | 1.4 | 1.5 | 1.7 | 1.7 | 1.6 | 1.7 |
| Class D | 1.3 | 1.1 | 1.2 | 1.3 | 1.2 | 1.4 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.5 | 1.5 | 1.6 | 1.4 | 1.4 |
| Enc Time[%] | 71% |  |  | 65% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.0 | 1.5 | 1.6 | 1.6 | 1.0 | 1.1 |
| Class C | 1.2 | 1.5 | 1.8 | 1.5 | 1.1 | 1.3 |
| Class D | 1.2 | 1.1 | 1.3 | 1.2 | 1.2 | 1.0 |
| Class E | 1.3 | 1.6 | 1.6 | 2.5 | 1.2 | 1.5 |
| All | 1.1 | 1.4 | 1.6 | 1.6 | 1.1 | 1.2 |
| Enc Time[%] | 72% |  |  | 64% |  |  |
| Dec Time[%] | 106% |  |  | 95% |  |  |

Table 10.  BD-rate comparison relative to HM3.0 anchor w/ partial CU merge disabled when 2NxN and Nx2N inter prediction is disabled.

# Conclusions and recommendations

In this contribution the HM3.0 partial CU merge mode is evaluated. Based on the evaluation results it is recommended to remove partial CU merge from test model to address quality and encoder design concerns, and disable 4x4 inter prediction in the HM software to regain the encoding time reduction.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] JCT-VC, “CE9: 3.1 PU Merge & Skip tools and proposed improvements” JCTVC-D0233, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

[4] M. Zhou, “Evaluation results on merge mode in HM2.0”, JCTVC-E0115, 5th Meeting: Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD text changes for

### 7.3.6 Coding unit syntax

7.3.7 Prediction unit syntax

### 7.4.7 Prediction unit semantics

Replace

### 7.3.6 Coding unit syntax

With

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 ) |  |
| } |  |
| } |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |

Replace

7.3.7 Prediction unit syntax

With

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### In 7.4.7 In Prediction unit semantics, delete the following sentence

When merge_flag[ x0 ][ y0 ] is not present (InferredMergeFlag is equal to 1), it is inferred to be equal to 1