| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F085 M20497 |
| --- | --- |

| Title: | Further study on compact representation of quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

Due to large block size transform and quantization used in the HEVC, carrying the quantization matrices in bitstream could lead to a significant overhead. In this document, a quantization matrix compression algorithm is proposed to achieve compact representation of quantization matrices for HEVC. Test results revealed that the algorithm provided up to 13-15x more compression when compared to the AVC quantization matrix compression method if all the compression tools are enabled. It is recommended to adopt the AVC quantization method into the test model, and start a CE on this subject to specify the compact quantization matrix representation format for efficient carriage of quantization matrices in HEVC.

# Introduction

In the current HEVC design the transform and quantization block sizes can go up to 32x32. The large block transforms can provide improved coding efficiency, but also lead to larger overhead for carrying the perceptual quantization matrices in the picture parameter sets. In HEVC there can be a total of 24 matrices (separate matrix for 4x4, 8x8, 16x16 and 32x32 four block sizes, intra/inter and Y, U, V components). It was reported in [3] that such an overhead can be roughly 10x of that of AVC if the AVC quantization matrix compression method is used. Therefore, it is desirable to design a compact representation format for encoding the quantization matrices of large block sizes.

In this contribution the quantization matrix compression algorithm proposed in [3] was further evaluated. The algorithm was integrated into the HM3.0 based AHG software, and tested by using the test vectors provided to the AHG group.

# Algorithm Description

The diagram of proposed algorithm is depicted in Figure 1. On the top of AVC method, the following additional tools are added to enhance the capability of quantization matrix compression:

- quantization matrix downsampling (down-sample)

- 135 degree symmetry processing (sym135)

- 45 degree symmetry processing with offset (sym45)

- unsigned exp-golomb coding (UEK)

The proposed algorithm is backward compatible to the AVC method, i.e. the additional compression tools can be enabled and disabled by high level flags in arbitrary manner, all the combinations of above tools are allowed. The detailed description of the algorithm can be found in [3].

Figure1.  Diagram of proposed quantization matrix compression algorithm

# Test Settings and Conditions

The quantization matrices used for testing are both asymmetry and symmetry ones provided to AHG by SONY. Attached text files qmatrix_asymmetry1.txt and qmatrix_symmetry1.txt contain quantization matrices used in testing.

The simulation is intended to demonstrate the compression ratio and associated quantization matrix compression errors of the proposed algorithm relative to the AVC method. The effect of quantization matrix compression errors on video quality is not evaluated in the tests.

# Experimental results

A summary of quantization matrix compression results is provided in Table 1. In the table the number of bits is total of bits consumed by encoding a total of 24 quantization matrices (intra/inter, Y, U, V for 4x4, 8x8, 16x16 and 32x32 block sizes), the compression ratio is computed against lossless AVC method, the average quantization matrix compression error is calculated by averaging compression errors over all the 24 matrices. As shown in Table 1, the proposed algorithm supports both lossless mode (i.e. AVC method when all the four compression tools are turned off) and lossy mode of 15 different compression levels.

The major observations are:

- using AVC method as is for encoding quantization matrices of HEVC can lead to significant overhead (26 – 28K bits observed in this particular case).

- for symmetry matrices a factor  of 1.8 lossless compression can be achieved.

- matrix down-sampling  appears an efficient compression tool that offers 2.5x compression without introducing significant  compression error

- up to 13-15x more compression can be achieved relative to AVC method if all the four tools are turned on in the proposed algorithm.

| Qmatx_compression_ID |  |  |  | Symmetry matrices |  |  | Asymmetry matrices |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down sample | UEK | sym45 | sym135 | Number of bits | Comp. ratio | Avg. error | Number of bits | Comp. ratio | Avg. error |
| 0 | 0 | 0 | 0 | 26408 | 1.00 | 0.00 | 28376 | 1.00 | 0.00 |
| 0 | 0 | 0 | 1 | 14432 | 1.83 | 0.00 | 14522 | 1.95 | 3.53 |
| 0 | 0 | 1 | 0 | 14996 | 1.76 | 5.06 | 13514 | 2.10 | 3.14 |
| 0 | 0 | 1 | 1 | 8396 | 3.15 | 5.06 | 7718 | 3.68 | 5.25 |
| 0 | 1 | 0 | 0 | 10010 | 2.64 | 3.78 | 10166 | 2.79 | 6.68 |
| 0 | 1 | 0 | 1 | 6146 | 4.30 | 3.53 | 6218 | 4.56 | 6.83 |
| 0 | 1 | 1 | 0 | 5876 | 4.49 | 7.91 | 5744 | 4.94 | 6.90 |
| 0 | 1 | 1 | 1 | 3896 | 6.78 | 7.52 | 3698 | 7.67 | 6.95 |
| 1 | 0 | 0 | 0 | 10682 | 2.47 | 0.43 | 11192 | 2.54 | 0.32 |
| 1 | 0 | 0 | 1 | 6224 | 4.24 | 0.43 | 6122 | 4.64 | 3.76 |
| 1 | 0 | 1 | 0 | 6620 | 3.99 | 4.73 | 5600 | 5.07 | 3.54 |
| 1 | 0 | 1 | 1 | 4028 | 6.56 | 4.73 | 3572 | 7.94 | 5.52 |
| 1 | 1 | 0 | 0 | 3884 | 6.80 | 3.96 | 3980 | 7.13 | 6.35 |
| 1 | 1 | 0 | 1 | 2792 | 9.46 | 3.47 | 2834 | 10.01 | 6.65 |
| 1 | 1 | 1 | 0 | 2558 | 10.32 | 7.29 | 2492 | 11.39 | 6.81 |
| 1 | 1 | 1 | 1 | 2006 | 13.16 | 6.73 | 1886 | 15.05 | 7.07 |

Table 1.  Summary of  quantization matrix compression results

# Conclusions and recommendations

Due to large block size transform and quantization used in the HEVC, carrying the quantization matrices can lead to significant overhead. It is recommended to adopt the AVC quantization matrix compression algorithm into the HEVC test model, and launch a CE on this subject to evaluate proposals in this category for specification of compact quantization matrix representation format for HEVC.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] M. Zhou, V. Sze, “Compact representation of quantization matrices for HEVC”. JCT-VC Document, : JCTVC-D024, Daegu, Korea, January, 2011

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).