| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F088 M20500 |
| --- | --- |

| Title: | CE9: Simplified AMVP design (SP06S1, SP06S2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou* Mahmut Sinangil* Vivienne Sze* Seungwook Park** Joonyoung Park** Byeongmoon Jeon** | Tel: Email:Tel: Email:Tel: Email: Tel: Email: | +1 214.480.3816 zhou@ti.com +1 214-480-3664 sinangil@ti.com +1 214-567-4005 sze@ti.com +82 10 3922 5896 Seungwook.park@lge.com |
| Source: | *Texas Instruments Inc; **LGE |  |  |

_____________________________

# Abstract

# In HM3.0 spatial MVP (motion vector predictor) scaling was introduced to the motion vector coding MVP (AMVP) list construction to improve the coding efficiency. It was reported that spatial MVP scaling can significantly increase the implementation cost of the AMVP list construction. This contribution proposes reducing the number of spatial MVP scaling from four to one to lower the AMVP cost. Simulation results revealed that this simplification does not introduce any coding loss when compared to the current design. However, the hardware implementation cost for the AMVP list can be reduced by almost 50% based on a hardware implementation study synthesized with 45 nm technology.

# Introduction

# In HM3.0 [2] spatial MVP (motion vector predictor) scaling was introduced to the motion vector coding MVP (AMVP) list construction to improve the coding efficiency. It was reported that spatial MVP scaling can significantly increase the implementation cost for the AMVP list construction. This contribution proposes a simplification method which can reduce the hardware implementation cost of the AMVP list construction by almost half without harming the coding efficiency.

# HM3.0 AMVP list construction

Current PU

A0

E

B0

C

D

Co-located PU

CR

1

2

H

Figure 1. Illustration of  the HM3.0 AMVP list construction

In HEVC the construction of the MVP list for motion vector coding (hereafter it is referred as AMVP list) is sophisticated. As shown in Figure 1, the construction of the AMVP list of each PU requires motion data from five spatial neighboring PUs (E, A0, C, B0, D in Figure 1) and two co-located temporal PUs (H and CR).

Figure 2. Illustration of spatial MVP candidates at each neighboring PU location

At each spatial neighboring PU location (i.e. E, A0, C, B0 or D in Figure 2), the following four MVP candidates as shown in Figure 2 can be derived, i.e.

- Motion vector from the same reference frame and same list (list0 or list1) as the current PU  (i.e. vec1)

- Or motion vector from the same reference frame but  different list (list0 or list1) from the current PU  (i.e. vec2)

- Or motion vector from the different reference frame but the same list as the current PU  (i.e. vec3)

- Or motion vector from the different reference frame and different list from the current PU (i.e. vec4)

The spatial MVP scanning order in HM3.0 is defined as follows:

- scan vec1 of PU E followed by PU A0

- scan vec2, vec3 and vec4 of PU E and followed by PU A0

- scan vec1 of PU C, PU B0 followed by PU D

- scan vec2, vec3 and vec4 of PU C, PU B0 followed by PU D

If none of above MVP candidates is available, the spatial MVP candidate at this neighboring PU location is treated as unavailable. For vec3 and vec4 because the current PU and its neighboring PU may have different temporal distances to their respective reference frame, motion vector scaling is needed to normalize the neighboring motion vectors to derive vec3 and vec4. Such a process is called spatial MVP scaling.

There are complexity issues with the AMVP list construction process described above because of complex and expensive spatial MVP scaling.

- Because there are up to 4 spatial MVP scaling each direction (list 0 or list 1), for hardware implementation it needs to instantiate a total of 16 multipliers (2 directions, up to 4 spatial MVP scaling each direction, and two components each MVP) for spatial MVP scaling to meet throughput requirement, which requires large hardware area

- The spatial MVP scaling is allowed down to 4x4 block size, which consumes lots of cycles especially for software solutions.

Therefore, it is desirable to reduce the number of spatial MVP scaling.

# Proposed algorithm

In the proposed algorithm, two changes are introduced:

- MVPs of same reference frame (vec1 and vec2) are scanned together, followed by the MVP scanning of different reference frames (vec3, vec4). (SP06S1)

- In addition, the maximum spatial MVP scaling is reduced to one, i.e. once the spatial MVP scaling is applied, it will no longer be used in the rest of AMVP list construction process even if the scaled MVP is the first MVP candidate in the list or it is a redundant MVP when compared to the first MVP candidate that is already in the list.  (SP06S2)

With the proposed changes, the AMVP list construction process is modified as follows (changes relative to HM3.0 are marked with yellow, SMVP_used is initialized to zero at beginning of AMVP list construction process of each PU).

Scanning left side neighboring PUs

- check PU E, if E has vec1 or vec2 it is added to the AMVP list, go to step 5. Otherwise, go to step 2 .

- check PU A0, if its has vec1 or vec2  it is added to the AMVP list, go to step 5. Otherwise, go to step 3.

- check PU E, if E has vec3 or vec4, it is added to the AMVP list, set SMVP_used to 1 and go to step 5. Otherwise, go to step 4

- check PU A0, if A0 has vec3 or vec4, it is added to the AMVP list and set SMVP_used to 1. Go to step 5.

Scanning upper side neighboring PUs

- check PU C, if C has vec1 or vec2  it is added to the AMVP list. If the AMVP list has two MVP candidates and they have equal value, remove the duplicated one from the AMVP list. Go to step 6 if the AMVP list has only one MVP candidate after pruning, otherwise go to step 11.

- check PU B0, if B0 has vec1 or vec2  it is added to the AMVP list. If the AMVP list has two MVP candidates and they have equal value, remove the duplicated one from the AMVP list. Go to step 7 if the AMVP list has only one MVP candidate after pruning, otherwise go to step 11.

- check PU D, if D has vec1 or vec2  it is added to the AMVP list. If the AMVP list has two MVP candidates and they have equal value, remove the duplicated one from the AMVP list. Go to step 8 if the AMVP list has only one MVP candidate after pruning, otherwise go to step 11.

- if SMVP_used equals to zero  check PU C, if C has vec3 or vec4 it is added to the AMVP list, go to step 11. Otherwise, go to step 9.

- if SMVP_used equals to zero  check PU B0, if B0 has vec3 or vec4 it is added to the AMVP list, go to step 11. Otherwise, go to step 10.

- if SMVP_used equals to zero  check PU D, if D has vec3 or vec4 it is added to the AMVP list. Go to step 11.

Scanning upper co-located temporal PUs & final AMVP list pruning

- check co-located PU H, if H has the temporal co-located motion vector, it is scaled and added to the AMVP list. Otherwise, check co-located PU CR, if CR has the temporal co-located motion vector, it is scaled and added to the AMVP list. Go to step 12

- if the AMVP list is empty, zero motion vector is added to the list. Otherwise, perform the pruning process to remove duplicated MVP candidate to form the final AMVP list.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The simulation results for SP06S1 and SP06S2 are summarized in table 1 and table 2, respectively. No loss has been observed by proposed simplification. For detailed results please refer to attached spreadsheets.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 103% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Class D | -0.1 | -0.4 | 0.5 | 0.0 | -0.2 | 0.0 |
| Class E | 0.0 | 0.2 | -0.3 | -0.1 | -0.4 | 0.4 |
| All | 0.0 | -0.1 | 0.1 | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 92% |  |  | 100% |  |  |

Table 1. BR-Rate increase relative to HM3.0 anchor for SP06S1

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.1 | 0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.4 | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 |
| Class D | -0.1 | -0.1 | 0.3 | 0.0 | 0.2 | 0.0 |
| Class E | 0.0 | 0.2 | 0.2 | 0.0 | -0.3 | 0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 93% |  |  | 98% |  |  |

Table 2. BR-Rate increase relative to HM3.0 anchor for SP06S2

# Complexity Analysis

Both HM3.0 AMVP list construction and the simplified one have been implemented in RTL and synthesized with 45 nm technology. As shown in Figure 3, for the HM3.0 design, in addition to a scale used for temporal MVP derivation, four additional scales are needed to support the spatial MVP scaling of five neighboring PUs running in parallel. The two left neighboring PUs can share a same scale. In total, five scales are needed for HM3.0 AMVP list construction in each direction (list 0 or list 1)

Figure 3. Diagram of HM3.0 AMVP list Implementation

With the proposed simplification, the number of scales can be significantly reduced. As shown in Figure 4, in the simplified design five left/upper spatial neighbors can share a single scale, the total number of scales are reduced from five to two for the AMVP list construction in each direction (list 0 or list 1)

Figure 4. Diagram of simplified HM3.0 AMVP list Implementation

Table 3 shows the implementation area comparison between the HM3.0 design and the proposed simplified design. The data revealed that with the simplification the implementation cost of the AMVP list can be significantly reduced. At 280 MHz, which is close to a typical frequency for low-power video cores, the gate count is almost cut by half.

Software complexity reduction is not quantified, but with the worst case number of spatial MVP scaling reduced from four to one, the software complexity shall be reduced accordingly.

Table 3. Area cost comparison between the HM3.0 and simplified AMVP implementation (45 nm technology)

# Conclusion and recommendations

The proposed simplification for the AMVP list construction significantly reduces the implementation cost without harming coding efficiency. It is recommended to adopt this proposal into HEVC working draft.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. and LGE may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

8.4.2.1.5 Derivation process for motion vector predictor candidates

…

Replace

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-21)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-22)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-23)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-24)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-25)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-26)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-27)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-28)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-29)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-30)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

with

set isScaledFlagLX  equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise, If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, set isScaledFlagLX  equal to 1,

- mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-31)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-32)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-33)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-34)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-35)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1,  RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ) and mvLY[ xBk ][ yBk ] is not identical to mvLXA,  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXB is equal to 0 and and isScaledFlagLX is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-36)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-37)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-38)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-39)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-40)