| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F089 M20501 |
| --- | --- |

| Title: | A study on simplification of spatial/temporal MVP scaling (CE9 SP01+SP06S2+SP07) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Park1 Joonyoung Park1 Byeongmoon Jeon1 Shun-ichi Sekiguchi2 Yusuke Itani2 Kazuo Sugimoto2 Minhua Zhou3 Mahmut Sinangil3 Vivienne Sze3 | Tel: Email: Tel: Email: Tel: Email: | +82 10 3922 5896 seungwook.park@lge.com Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp +1 214.480.3816 zhou@ti.com +1 214-480-3664 sinangil@ti.com + 1214-567-4005 sze@ti.com |
| Source: | LGE1, Mitsubishi Electric2, Texas Instruments Inc3 |  |  |

_____________________________

# Abstract

This contribution reports the evaluation results on the combinations of the following three simplification tools: 1) Disabling spatial MVP scaling process for 4x4 PUs in the AMVP list construction.  (SP01), 2) Scanning MVPs of same reference frame together followed by the MVP scanning of different reference frames, and reducing the maximum number of spatial MVP scaling to one in the AMVP list construction. (SP06S2) and 3) disabling the temporal MVP derivation for 4x4 PUs in both merge/skip MVP and AMVP list construction. (SP07).  Simulation results reveal that there is no coding loss when applying those three tools together. It is recommended to adopt those three tools in the HEVC test model to cut the AMVP list implementation cost by half and reduce the worst case decoder complexity for 4x4 blocks.

.

# Introduction

Spatial and temporal MVP scaling is the most complex part of the merge/skip MVP and AMVP list construction process and is reportedly expensive in both area and cycles [4]. Several simplification tools have been tested in CE9 to address the complexity concerns of spatial and temporal MVP derivation process. This contribution reports the evaluation results on the combinations of the following three simplification tools:

- Disabling spatial MVP scaling process for 4x4 PUs in the AMVP list construction.  (SP01)

- Scanning MVPs of same reference frame together followed by the MVP scanning of different reference frames, and reducing the maximum number of spatial MVP scaling to one in the AMVP list construction. (SP06S2)

- And disabling the temporal MVP derivation for 4x4 PUs in both merge/skip MVP and AMVP list construction. (SP07)

CE9 documents [3][4][5] have reported that those three tools won’t cause coding loss when applying them individually. This contribution further demonstrates that those three simplification tools can be combined together without harming coding quality. Applying those three tools together can reduce the implementation cost of the AMVP list by half [4] and reduce the worst case decoder complexity for 4x4 blocks [3][5].

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [1] are followed. .The simulation platform at Texas Instruments is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines. Mitsubishi Electric utilized a cluster system of 64bit Linux workstations equipped with various versions of Intel® Xeon® CPU. LGE utilized a group of stand-alone servers of Windows XP 64bit OS with Intel i7 875 & 950 CPU installed. The following table summarizes the tested combinations and how cross-check is arranged among three participants.

| Tool combination | Tester 1 | Tester2 |
| --- | --- | --- |
| SP01 + SP06S2 | TI | LGE |
| SP01 + SP06S2 + SP07 | TI | Mitsubishi Electric |

Table 1.  Summary of tested combinations

# Experimental results

The test results are summarized in Table 2 (SP01 + SP06S2) and Table 3 (SP01 + SP06S2 + SP07). No coding loss is observed for both combinations. The detailed results can be found in attached spreadsheets.

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 | -0.1 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | -0.1 | 0.1 | 0.0 | 0.0 | -0.3 |
| Class E | 0.1 | 0.4 | 0.3 | -0.1 | -0.1 | 0.2 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 2.  BD-rate increase relative to HM3.0 anchor for SP01 + SP06S2

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | -0.2 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | 0.0 | 0.0 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | -0.1 |
| Class D | -0.1 | -0.3 | 0.4 | 0.0 | -0.3 | -0.2 |
| Class E | 0.0 | 0.3 | -0.2 | -0.1 | -0.1 | 0.3 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.  BD-rate increase relative to HM3.0 anchor for SP01+SP06S2+SP07

# Conclusions and recommendations

The simulation results reveal that there is no coding loss when spatial/temporal MVP scaling is disabled for 4x4 PUs in both merge/skip MVP and AMVP list construction, as well as the maximum number of spatial MVP scaling is reduced to one for the AMVP list construction. It is recommended to adopt those three simplification tools (i.e. CE9 SP01, SP06S2, SP07) into the HEVC test model to simplify the current HEVC design.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] M. Zhou, “CE9:  Evaluation results on SP01, SP02, SP03 and SP14”, JCT-VC Document, JCTVC-F084, Tornio, 1T, July 2011

[4] M. Zhou, M. Sinagil, V. Sze, S. Park, J. Park, B. Joen “CE9:  Simplified AMVP design (SP06S1, SP06S2)”, JCT-VC Document, JCTVC-F088, Tornio, 1T, July 2011.

[5] Y. Itani, S.Sekiguchi, “CE9: Temporal vector restriction for small PUs (SP07/SP09/SP12)”, JCTVC-F169, Torino, IT, July 2011.

# Patent rights declaration(s)

LGE,  Mitsubishi Electric and Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

8.4.2.1.1 Derivation process for luma motion vectors for merge mode

8.4.2.1.4 Derivation process for luma motion vector prediction

8.4.2.1.5 Derivation process for motion vector predictor candidates

8.4.2.1.1 Derivation process for luma motion vectors for merge mode

Replace

…

- The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.7 is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

…

With

…

- If both nPSW and nPSH are equal to 4, availableFlagCol is set equal to 0. Otherwise, The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

…

8.4.2.1.4 Derivation process for luma motion vector prediction

Replace

…

- The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.7 is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

…

With

…

- If both nPSW and nPSH are equal to 4, availableFlagLXCol is set equal to 0. Otherwise, the derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

…

8.4.2.1.5 Derivation process for motion vector predictor candidates

…

Replace

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-1)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-3)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-4)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-7)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-10)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

with

Set isScaledFlagLX equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- If both nPSW and nPSH are equal to 4, set availableFlagLXA equal to 0. Otherwise, when availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- If availableFlagLXA is equal to 1, set isScaledFlagLX  equal to 1, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-11)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-12)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-13)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-14)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-15)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1,  RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ) and mvLY[ xBk ][ yBk ] is not identical to mvLXA,  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- If both nPSW and nPSH are equal to 4, set availableFlagLXB equal to 0. Otherwise, when availableFlagLXB is equal to 0 and and isScaledFlagLX is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-16)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-17)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-18)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-19)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-20)