| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F090 M20502 |
| --- | --- |

| Title: | Cross-verification of Panasonic’s proposal JCTVC-F470 Parsing Robustness for Merge/AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of Panasonic’s proposal JCTVC-F470 Parsing Robustness for Merge/AMVP. Both algorithm detail and source code have been carefully studied. It has been verified that the results provided by Panasonic are accurate. Due to significant increase in complexity further study is recommended for the merge/skip MVP list construction part of the proposal.

# Algorithm Study

It has been verified that the proposed algorithm contains the following tools:

- Ttruncated unary MVP index coding with fixed maximum length 5 and 3 for merge/skip and AMVP list coding, respectively. This fully decouples the motion data reconstruction from entropy decoding.

- Simplified CABAC/CAVLC coding for inter_pred_flag and ref_idx_lx

  - Adaptive context selection method for inter_pred_flag and ref_idx_lx (CABAC)
 It is changed that ctxIdx is fixed to 0

  - Adaptive combined coding method for inter_pred_flag and ref_idx_lx (CAVLC)
 It is changed that use_combined_inter_pred_ref is fixed to true in B slice

This change makes bitstream parsing more robust and simplifies design. No complexity implication.

- Modified merge/skip MVP list construction, after HM3.0 process

  - Adding additional bi-direction MVP candidates by combining high-priority MVPs

  - Adding additional bi-direction MVP candidates by  scaling high-priority to opposite list

  - Adding zero vector if number of valid MVP candidates in the list is still less than 5 after above steps

This tool adds complexity to compensate coding loss caused by step 1, and needs further trade-off study on complexity and coding efficiency.

- Modified AMVP list construction, after HM3.0 process if the list size is less than 3 and zero vector does not already exist in the list, a zero vector is appended to the list.

This tool does not have complexity impact but seems to be effective for quality improvement especially in class E sequence

The proposed algorithm resolves the parsing throughput issue and improves parsing robustness by making MVP index coding independent of MVP list size, and by coding inter_pred_flag and ref_idx_lx independent of neighbours. However, it introduces a total of 8 scales to compensate the loss. The merge/skip list construction part of this proposal has become quite complex because of additional scales and a much bigger number of MVPs for list pruning process. Therefore, it should be further studied for better quality and complexity trade-off..

It has been noticed that the derived TMVP can be scaled again to create additional bi-directional MVPs, this kind of cascaded (sequential) TMVP scaling can decrease throughput a lot, and should be avoided.

It has also been observed that when scaling a valid motion vector of one direction (i.e. list 0 or list 1) to another direction (i.e. list 1 or list 0) to create additional bi-directional MVPs, the scaled motion vector uses the same ref_idx as the valid motion vector.   Logically speaking using ref_idx = 0 for scaled motion vector should be a better choice.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [1]. The simulation platform at Texas Instruments is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies.

# Experimental Results

The test results are given in Table 1 to Table 3. All the results here perfectly match the ones provided by Panasonic. The anchor used here is HM3.0 without bug fix 146, the run time reported in the document may not be accurate. It has noticed that in Table 1 the average loss of decoupled motion data reconstruction and entropy decoding by using fixed size truncated unary coding matches results reported in JCTVC-F068 [3].

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.7 | 0.6 | 1.1 | 1.0 | 1.0 |
| Class B | 0.9 | 0.6 | 0.5 | 1.3 | 1.0 | 0.9 |
| Class C | 0.9 | 0.9 | 0.9 | 1.2 | 1.0 | 1.1 |
| Class D | 1.0 | 0.9 | 0.8 | 1.3 | 0.9 | 0.9 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.7 | 0.7 | 1.2 | 1.0 | 1.0 |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 | 0.8 | 0.3 | 1.6 | 1.3 | 1.1 |
| Class C | 1.0 | 1.0 | 0.8 | 1.4 | 1.0 | 0.9 |
| Class D | 1.1 | 0.9 | 1.2 | 1.5 | 0.9 | 0.9 |
| Class E | 2.7 | 2.5 | 2.6 | 3.7 | 3.4 | 3.3 |
| All | 1.4 | 1.2 | 1.1 | 1.9 | 1.5 | 1.4 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  |

Table 1: Average coding loss of straightforward decoupling method

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.5 | -0.4 | -0.4 | 0.1 | -0.3 |
| Class B | 0.0 | -0.2 | -0.2 | 0.0 | -0.1 | 0.0 |
| Class C | -0.1 | 0.0 | -0.2 | -0.1 | -0.1 | -0.2 |
| Class D | -0.3 | -0.2 | 0.0 | -0.2 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.2 | -0.2 | -0.2 | -0.1 | -0.2 |
| Enc Time[%] | 114% |  |  | 114% |  |  |
| Dec Time[%] | 97% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -1.1 | -1.2 | -1.1 | -1.1 | -1.0 |
| Class C | -0.5 | -0.8 | -0.9 | -0.7 | -0.7 | -1.0 |
| Class D | -0.3 | -0.7 | -0.1 | -0.3 | -0.2 | -0.4 |
| Class E | 0.3 | 0.1 | -0.8 | -0.7 | -0.4 | 0.1 |
| All | -0.2 | -0.7 | -0.8 | -0.7 | -0.6 | -0.7 |
| Enc Time[%] | 111% |  |  | 112% |  |  |
| Dec Time[%] | 91% |  |  | 103% |  |  |

Table 2: Average coding gain of proposed decoupling method from Panasonic

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.5 | -0.6 | -0.5 | -0.3 | -0.6 |
| Class B | -0.4 | -0.5 | -0.6 | -0.4 | -0.3 | -0.3 |
| Class C | -0.6 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
| Class D | -0.8 | -0.6 | -0.7 | -0.8 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.5 | -0.6 | -0.6 | -0.4 | -0.5 |
| Enc Time[%] | 113% |  |  | 110% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -1.2 | -1.5 | -1.4 | -1.5 | -1.4 |
| Class C | -0.9 | -1.0 | -1.1 | -1.1 | -1.2 | -1.1 |
| Class D | -0.8 | -0.8 | -0.4 | -0.8 | -0.8 | -0.5 |
| Class E | -0.9 | -1.4 | -1.3 | -1.2 | -1.6 | -1.5 |
| All | -0.9 | -1.1 | -1.1 | -1.1 | -1.3 | -1.1 |
| Enc Time[%] | 107% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 3: Average coding gain of Panasonic merge/skip list construction method (no decoupling)

Further cross-check was done for the modified algorithm which was aimed to address the complexity concerns expressed in section 1. In the modified algorithm, the maximum number of combined additional MVP candidates is limited to 5, additional scaled MVP candidates are avoided, instead, a bi-directional MVP candidate which does not need scaling is added, and up two zero MVPs are added at the end, this reduces the number of MVP candidates for final pruning process  from 24 to 12. The results are given in Table 4, which perfectly match the results from Panasonic.

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.4 | -0.1 | -0.3 | -0.1 | -0.2 |
| Class B | 0.0 | -0.2 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.2 | -0.2 | -0.2 | -0.2 |
| Class D | -0.2 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
| Enc Time[%] | 114% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -1.0 | -1.1 | -1.1 | -1.1 | -1.0 |
| Class C | -0.4 | -0.6 | -0.8 | -0.7 | -0.8 | -0.9 |
| Class D | -0.3 | -0.7 | 0.0 | -0.4 | -0.3 | -0.8 |
| Class E | 0.7 | 0.1 | -0.3 | -0.1 | -0.1 | 0.4 |
| All | -0.1 | -0.6 | -0.6 | -0.6 | -0.6 | -0.7 |
| Enc Time[%] | 112% |  |  | 110% |  |  |
| Dec Time[%] | 95% |  |  | 100% |  |  |

Table 3: Average coding gain of Panasonic modified merge/skip list construction method

The runtime increase is mainly due to the fact that now almost all the PUs have a merge/skip list of size 5, which was not the case in the current HM3.0 design. This increases the motion estimation time for merge mode estimation. But from HW design point of view, this is not true implementation cost increase because HW has to prepare logics to do motion estimation with 5 merge/skip MVP candidates in motion estimation anyway, even with the current HM3.0 design whose list size is 1, 2, 3, 4, or 5. The true complexity increase lies in the merge/skip list construction, the cost of final (2nd) pruning process due to adding additional MVP candidates should be further studied.  Apparently this part of complexity increase does not show up in the decoder runtime.

# Conclusions

It has been cross-verified that the results provided by Panasonic in [2] are accurate. It is recommended to further study the merge/skip MVP list construction part of proposal in CE, and adopt rest part of proposal into the HEVC test model to address parsing throughput concerns.

# References

[1] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2] T. Sugio, T. Nishi, “Parsing Robustness for Merge/AMVP” JCTVC-F470, 6th Meeting: Torino, IT, 14-22 July, 2011

[3] M. Zhou, V. Sze, Y. Matsuba,  “A study on HM3.0 parsing throughput issue” JCTVC-F068,  6th Meeting: Torino, IT, 14-22 July, 2011