| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F091 |
| --- | --- |

| Title: | Unifying binarizations of Intra modes in HE and LC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ehsan Maani, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 |  | ehsan.maani@am.sony.com (408) 352-4710 |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides a new method for binarization and coding of Intra prediction modes. Unlike the current Intra mode coding technique, the proposed approach uses the same binarization and adaptation in Low Complexity (LC) and High Efficiency (HE) and does not require any VLC or initialization tables. Using this approach 400 bytes VLC tables are removed and, on average, a compression gain of 0.3% and 0.1% were observed in HE and LC settings, respectively.

# Algorithm description

The currently reference software (HM3.0), uses fixed-length codes in High Efficiency (HE) settings and Huffman codes in Low Complexity (LC) settings. For the LC case, 8 sets of VLC tables (4 tables for 4x4 and 4 for the rest of PU sizes) and 8 sets of mode order initializations need to be stored in the ROM. These VLC and initialization tables have a combined size of 400 bytes (assuming 1 byte per stored number). The proposed approach, on the other hand, does not require any VLC or mode initialization tables, thus, reducing the required storage memory by 400 bytes. Furthermore, it utilizes the same binarization and adaptation scheme for both LC and HE and therefore harmonizes the Intra prediction mode coding of HE and LC. This proposal only affects the coding of remaining Intra modes and thus does not change the coding of MPM modes compared to HM3.0.

In the proposed scheme, one array of size 17 and one of size 34 hold the rank (order) of Intra prediction modes of 4x4 and 8x8/16x16/32x32 PU sizes, respectively.  Initially the rank of each mode is the same as its mode number. When an Intra prediction mode is selected and coded into the bit stream, and it is not one of Most Probable Modes (MPM), its rank is swapped with the mode that has exactly one lower rank. This process is carried out each time a mode is selected regardless of the coding profile (LC or HE). This adaptation scheme is the same as LC Intra mode coding in HM3.0 with the exception that the statistics of each Intra mode is kept separately. In other words, one rank is assumed for each mode (including MPM), and for the remaining modes, the rank of the current mode is compared to the rank of MPM. If the rank of current mode is larger than that of MPM, its value if reduced by one. In HM3.0, on the other hand, this process is done on the mode numbers themselves (rather their ranks), thus mixing mode statistics together.

Table 1.Comparison of derivation processes of Intra modes in HM3 and the proposed method.

| HM3.0 (LC only) | rem_intra_pred_mode = IntraPredModeTableD[rem_intra_pred_mode] for (cIdx = 0; cIdx < NumMPMCand; cIdx++) if (rem_intra_pred_mode]>= candModeList[cIdx]) rem_intra_pred_mode ++ IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode |
| --- | --- |
| Proposed (LC and HE) | for (cIdx = 0; cIdx < NumMPMCand; cIdx++) if (IntraPredModeTableE[rem_intra_pred_mode]>=IntraPredModeTableE[candModeList[cIdx]]) rem_intra_pred_mode ++ IntraPredMode[ xB ][ yB ] = IntraPredModeTableD[rem_intra_pred_mode] |

Binarization of the mode ranks (in both profiles) uses a generalized version of the Exponential-Golomb codes. The following tables demonstrate the construction of the codewords:

Table 2. Codewords for Intra mode coding of 4x4 PUs.

| Rank range | Prefix (Unary code) | Remainder (fixed length) |
| --- | --- | --- |
| 0-3 | 0 | xx (2 bits) |
| 4-7 | 10 | xx (2 bits) |
| 8-15 | 11 | xxx (3 bits) |

Table 3.  Codewords for Intra mode coding of 8x8, 16x16, and 32x32 PUs.

| Rank range | Prefix (Unary code) | Remainder (fixed length) |
| --- | --- | --- |
| 0-3 | 0 | xx (2 bits) |
| 4-11 | 10 | xxx (3 bits) |
| 12-19 | 110 | xxx (3 bits) |
| 20-35 | 111 | xxxx (4 bits) |

It should be noted that since these codewords are only used when modes other than MPM are selected, the maximum number of codewords needed for 4x4 and 8x8/16x16/32x32 are 16 and 33, respectively. However,  HM3.0 also allows for two MPM modes which reduces the number of remaining modes by one. Thus, whenever, the maximum number of possible modes is less than 16 for 4x4 PUs and 36 for 8x8/16x16/32x32 PUs (), the additional unused codewords are blocked from the last rank range (i.e., 8-15 range for 4x4 PUs and 20-35 range for 8x8/16x16/32x32 PUs) at the encoder and decoder.

In HE mode, in the current implementation, each bin of the prefix part of the codeword has its own context.

# Simulation Results

Simulations are performed according to common test conditions using a modified version of HM3.0 and the results are compared to HM3.0 anchors in Table 4.

Table 4. Results of the proposed binarization in Intra HE and LC.

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| Class B | -0.3 | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 |
| Class C | -0.3 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class E | -0.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Overall | -0.3 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).