| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F092 |
| --- | --- |

| Title: | Coding tree pruning based CU early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiho Choi Sang-Hyo Park Euee S. Jang | Tel: Email: | +82-2-2295-1086 aikiho@gmail.com voice@dmlab.hanyang.ac.kr esjang@hanyang.ac.kr |
| Source: | Hanyang university, Rep. of Korea |  |  |

_____________________________

# Abstract

In this proposal, coding tree pruning based CU early termination is suggested for reducing the encoding time in the HEVC test model software. The experimental results showed that the proposed method achieved about 42% reduction in encoding time compared to the HEVC test model 3.1 encoder with only a negligible loss of luma BD-bitrate (i.e., < 0.6%) and a small gain of chroma BD-bitrate (up to 0.8%)..

# Introduction

In determining the best CU size, the reference encoder of the HEVC tests every possible CU size in order to estimate the coding performance of each CU defined by the CU size. This causes major computational complexity within the encoding process, which should be overcome for the implementation of a fast encoder. This contribution proposes a simple tree-pruning algorithm that exploits the observation where the sub-tree computations can be skipped if the coding mode of the current node is sufficient (i.e., SKIP mode).

# Proposed encoder modifications

The CU is the basic unit of region splitting used for inter/intra prediction. The CU concept allows recursive splitting into four equally sized blocks, starting from the treeblock. In HEVC test model software, the key function for the CU processing is TEncCU::xCompressCU, which various prediction modes are evaluated as shown in the Figure 1.

For a fast CU depth decision by determining the CU depth early, we exploit the fact that no further processing of sub-trees is necessary when the current CU node selects SKIP mode as best prediction modes in current CU depth. The diagram of proposed algorithm is depicted in Figure 2.

Figure 1: Current CU processing in HEVC encoder

Figure 2: Proposed CU processing in HEVC encoder based on CU early termination

# Simulation Results

Experiments were conducted to determine the trade-offs between encoding time and coding performance offered by the proposed changes. HM 3.1 was used for the experiments [1]. The details of the encoding environment are same as JCTVC-E700 [2]. The experimental results summarized in Table 1.

Table 1 Results of the comparison between HM 3.1 and the proposed method with respect to total encoding time

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | -0.5 | -0.5 | 0.4 | -0.5 | -0.6 |
| Class B | 0.6 | -0.5 | -0.6 | 0.7 | -0.5 | -0.5 |
| Class C | 0.5 | -0.2 | -0.2 | 0.7 | -0.2 | -0.1 |
| Class D | 0.5 | -0.4 | -0.3 | 0.7 | -0.5 | -0.2 |
| Class E |  |  |  |  |  |  |
| Overall | 0.5 | -0.4 | -0.4 | 0.6 | -0.4 | -0.4 |
| Enc Time[%] | 62% |  |  | 52% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | -0.3 | -0.3 | 0.5 | -0.3 | -0.4 |
| Class C | 0.3 | -0.2 | -0.2 | 0.3 | -0.1 | 0.1 |
| Class D | 0.2 | -0.3 | -0.3 | 0.3 | -0.9 | -0.6 |
| Class E | 0.5 | -0.3 | -1.1 | 0.4 | -0.7 | -0.6 |
| Overall | 0.3 | -0.3 | -0.4 | 0.4 | -0.5 | -0.4 |
| Enc Time[%] | 63% |  |  | 54% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | -0.7 | -0.8 | 0.6 | -0.6 | -1.0 |
| Class C | 0.3 | -0.5 | -0.3 | 0.3 | -0.4 | 0.0 |
| Class D | 0.3 | -0.6 | -0.2 | 0.4 | -0.9 | -1.2 |
| Class E | 0.4 | -1.8 | -1.5 | 0.4 | -1.4 | -1.5 |
| Overall | 0.4 | -0.8 | -0.7 | 0.4 | -0.8 | -0.9 |
| Enc Time[%] | 62% |  |  | 56% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusion

In this contribution, we proposed a fast CU decision method for HEVC by early determination of the CU size based on coding tree pruning, which yielded a reduction in encoding time of approximately 42% when compared to the HM 3.1 encoder with only a negligible loss of BD-bitrate (i.e., < 0.6%). The experimental results showed that the proposed coding tree pruning method would be considered when designing a fast HEVC encoder.

# References

- http://hevc.kw.bbc.co.uk/trac/browser/jctvc-hm/tags/HM-3.1

- Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-E700, Jan. 2011

Hanyang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).