| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F099 |
| --- | --- |

| Title: | Performance report of temporal motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC corporation |  |  |

_____________________________

# Abstract

This contribution reports the performance of temporal motion vector prediction (TMVP), which is employed both for advanced motion vector prediction (AMVP) and merge mode in the current HEVC Working Draft. It is reportedly shown that TMVP with the default buffer compression achieves coding gains by 2.3%, 2.2%, 2.3%, 2.3%, 2.1% and 2.0% on average for RA-HE, RA-LC, LB-HE, LB-LC, LP-HE and LP-LC configurations, respectively. Ratios of BD-rate to the number of bits per reference SCU are from 0.15% to 0.17%.

# Introduction

In the current Working Draft (WD3) of HEVC [1], advanced motion vector prediction (AMVP) scheme is used for efficient compression of motion information. In AMVP, a temporal motion vector prediction (TMVP), where a temporally collocated motion vector is used as a candidate of the motion vector predictor, is employed. For TMVP, motion vectors should be stored into a buffer associated with each reference frame and the storage size can be significant due to fine granularity of motion vectors. A motion vector buffer compression method [2, 3] is therefore introduced into WD3 and thereby motion vectors are summarized on a 16x16 block basis by default. Some modifications were proposed at the last meeting and they are tested in CE1 [4].

This contribution reports the performance of the current TMVP with the default (16x) buffer compression. In addition to BD-rates and decoding/encoding times, a new metric is introduced to evaluate the balance between compression performance and memory consumption. The metric also helps evaluate effectiveness of temporal prediction of other information such as temporal QP prediction, which is described in JCTVC-F103 [5].

# Experiment

## Setup

The experiment was conducted with the HM 3.0 software (revision 823) under the common test conditions [6]. The performance of TMVP was evaluated by comparing HM 3.0 with the default setting against that with TMVP disabled. In order to disable TMVP, macro definitions in TypeDef.h were modified as follows.

#define MRG_NEIGH_COL 0
#define AMVP_NEIGH_COL 0

Our computing platform used for this experiment is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

It should be noted that individual decoding/encoding time can be significantly varied since the CPU have automated over-clocking functionality and I/O speed varies by 12 processes running in parallel.

## Results and analysis

Results are summarized in Table 1. More detailed results are shown in the attached spreadsheet. These results reveal that TMVP achieves high coding gains at the expense of several percentage of encoding/decoding time increase.

Meanwhile, TMVP needs buffer memory for storing motion data, although motion data are compressed on a 16x16 block basis by default. Here we analyze the balance between compression performance and memory consumption of TMVP. According to the analysis in JCTVC-E221 [7], 55 bits per block are consumed for storing motion information. Under the common conditions, 55/4=13.75 bits are consumed for each SCU on reference frame, since the SCU size is set to 8x8 and the block size for motion data storage is set to 16x16. Table 2 shows ratios of coding gain to bits per reference SCU, which are calculated by dividing the negative of the BD-Rate value by the number of bits consumed for each SCU.

Table 1: Summary results of HM 3.0 with TMVP vs. HM 3.0 without TMVP.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.1 | -2.5 | -2.9 | -3.3 | -2.6 | -2.9 |
| Class B | -2.0 | -1.4 | -1.4 | -1.6 | -1.1 | -1.2 |
| Class C | -2.1 | -2.1 | -2.2 | -2.0 | -2.0 | -2.0 |
| Class D | -2.0 | -1.8 | -1.9 | -2.0 | -1.8 | -1.8 |
| Class E |  |  |  |  |  |  |
| All | -2.3 | -1.9 | -2.1 | -2.2 | -1.8 | -1.9 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.0 | -2.0 | -1.8 | -2.2 | -1.5 | -1.4 |
| Class C | -2.4 | -2.4 | -2.5 | -2.5 | -2.3 | -2.2 |
| Class D | -2.2 | -1.9 | -2.4 | -2.2 | -1.3 | -1.2 |
| Class E | -2.8 | -2.4 | -1.8 | -2.5 | -1.3 | -1.2 |
| All | -2.3 | -2.1 | -2.1 | -2.3 | -1.6 | -1.5 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9 | -1.6 | -0.9 | -1.8 | -1.3 | -1.2 |
| Class C | -2.2 | -2.3 | -2.3 | -2.3 | -2.3 | -2.3 |
| Class D | -1.9 | -1.5 | -1.4 | -2.0 | -1.5 | -2.0 |
| Class E | -2.6 | -2.4 | -2.3 | -2.2 | -1.5 | -1.5 |
| All | -2.1 | -1.9 | -1.6 | -2.0 | -1.6 | -1.7 |
| Enc Time[%] | 109% |  |  | 107% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |

Table 2: Ratios of coding gain to bits per reference SCU.

|  | Y | U | V |
| --- | --- | --- | --- |
| RA.HE | 0.17 | 0.14 | 0.15 |
| RA.LC | 0.16 | 0.13 | 0.14 |
| LB.HE | 0.17 | 0.16 | 0.15 |
| LB.LC | 0.17 | 0.12 | 0.11 |
| LP.HE | 0.15 | 0.14 | 0.12 |
| LP.LC | 0.15 | 0.12 | 0.13 |

# Conclusion

In this contribution, the performance of TMVP in the WD3 was reported. The experimental results with HM 3.0 have shown that TMVP achieves 2.3%, 2.2%, 2.3%, 2.3%, 2.1% and 2.0% on average for RA-HE, RA-LC, LB-HE, LB-LC, LP-HE and LP-LC configurations, respectively. Ratios of coding gain to bits per reference SCU derived from these results are from 0.15% to 0.17%. It is suggested that effectiveness of temporal prediction of other information such as temporal QP prediction is evaluated by referring to results presented in this contribution.

# References

[1] T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[2] Y. Su and A. Segall, “On motion vector competition,” JCT-VC document JCTVC-C257, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 3rd meeting, Guangzhou, October 2010.

[3] Y. Su and A. Segall, “CE9: Reduced resolution storage of motion vector data,” JCT-VC document JCTVC-D072, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

[4] J. Jung, et al., “Description of Core Experiment CE1: Motion Data Storage Reduction,” JCT-VC document JCTVC-E701, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

[5] H. Aoki, et al., “CE4 Subtest2: QP prediction based on intra/inter prediction (test 2.4.b),” JCT-VC document JCTVC-F103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

[6] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

[7] E. Francois, et al., “On memory compression for motion vector prediction,” JCT-VC document JCTVC-E221, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.