| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F101 |
| --- | --- |

| Title: | CE3: Reports on Bi/Uni filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81-3-5435-3308 Kenji.Kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of Bi/Uni filter which is studied in core experiment (CE) 3[1]. To improve coding efficiency, the Bi/Uni filter switch a set of filter coefficients by either prediction mode is bi-prediction or not. The detail is shown in [2].

In this contribution, Bi/Uni filter is modified slightly from [2]. The changes is filter coefficients and number of tap that is 7 tap, 8 tap and 8 tap for bi-prediction quarter pel, half pel and uni-prediction.

With the proposed method, 0.4% gain can be obtained compared with CE3 anchor. (-0.5%, -1.1%, +0.1%, -0.2%, -0.1% and -0.3% for RA_HE, RA_LC, LD_HE, LD_LC, LDP_HE and LDP_LC)

# Introduction

This Bi/Uni filter is basically the same as Bi/Single filter [2]. The basic technique of Bi/Uni filter was introduced in [2, 3]. We show the filter coefficients in Table 1. The filter coefficients have 6 bit accuracy.

In [2], we applied 5 tap for uni-prediction. But, in this proposal, we changed the tap length because test condition was added for Low delay P-only. For this low delay P-only case, 5-tap filter for uni-prediction was always used. It can not avoid some loss. So, we modified the number of tap as shown in Table 1. The quarter pel filter for bi-pred is reduced to 7 tap. This aims to reduce complexity in worst case.

Table 1: Filter coefficients

| Prediction mode | Sub pel | Tap length | Filter coefficients |
| --- | --- | --- | --- |
| Uni-prediction | 1/2 pel | 8 | -1 , 4 , -10 , 39 , 39, -10, 4, -1 |
|  | 1/4 pel | 8 | -1 , 4 , -10 , 57 , 19, -7, 3, -1 |
| Bi-prediction | 1/2 pel | 8 | -2 , 5 , -12 , 41 , 41, -12, 5, -2 |
|  | 1/4 pel | 7 | -2 , 6 , -13 , 62 , 15 , -5 , 1 , 0 |

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 3.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in JCTVC-E700 [4].

# Test results

Simulation results are shown in Table 2 for random access case.

Table 2: Simulation result for random access case

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.5 | 0.5 | 0.2 | 0.7 |
| Class B | -0.1 | 0.1 | 0.2 | -0.3 | 0.2 | 0.2 |
| Class C | -0.8 | -0.1 | -0.3 | -1.6 | -0.1 | -0.2 |
| Class D | -1.6 | -0.7 | -0.8 | -3.3 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.1 | -0.1 | -1.1 | -0.1 | 0.0 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Simulation results are shown in Table 3 for low delay case.

Table 3: Simulation result for low delay case

|  | Low delay |  |  | Low delayLC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.6 | 0.1 | 0.2 | 0.0 | -0.3 |
| Class C | -0.1 | 0.3 | 0.2 | -1.0 | 0.1 | 0.0 |
| Class D | -0.1 | 0.0 | 0.7 | -1.3 | 0.2 | -0.1 |
| Class E | 0.6 | 0.9 | 0.3 | 1.7 | -0.5 | 0.4 |
| All | 0.1 | 0.5 | 0.3 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Simulation results are shown in Table 4 for low delay case.

Table 4: Simulation result for low delay case

|  | Low delay P |  |  | Low delay LCP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.4 | -0.3 | -0.3 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 |
| Class D | 0.0 | -0.1 | -0.3 | 0.3 | 0.0 | -0.7 |
| Class E | -0.2 | 0.0 | -0.6 | -1.3 | -0.8 | -0.6 |
| All | -0.1 | -0.1 | -0.4 | -0.3 | -0.3 | -0.4 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

# Theoretical complexity analysis

We investigated theoretical complexity analysis using CE3’s excel sheet. The summary of complexity analysis is shown in Table 5 and Table 6. For uni prediction, we can see that complexity for operation is decreasing.

Table 5: Summary of complexity analysis (multiplication case)

| Method |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  | multiple | multiple |  |
| Sheet |  |  | Anchor | Proposal | Ratio (%) |
| Single | Operations | Worst case / pixel | 48.75 | 48.75 | 100% |
|  |  | Average of operations / pixel | 24.08 | 24.08 | 100% |
|  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  | Average of bytes / pixel | 2.48 | 2.48 | 100% |
| Bi | Operations | Worst case / pixel | 97.50 | 88.00 | 90% |
|  |  | Average of operations / pixel | 48.17 | 45.92 | 95% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 4.95 | 4.95 | 100% |
| Total | Operations | Worst case / pixel | 97.50 | 88.00 | 90% |
|  |  | Average of operations / pixel | 36.12 | 35.00 | 97% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 3.71 | 3.71 | 100% |

Table 6: Summary of complexity analysis (add/shift case)

| Method |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  | add/shift | add/shift |  |
| Sheet |  |  | Anchor | Proposal | Ratio (%) |
| Single | Operations | Worst case / pixel | 60.00 | 60.00 | 100% |
|  |  | Average of operations / pixel | 30.10 | 30.10 | 100% |
|  | Bandwith | Worst case / pixel | 7.56 | 7.56 | 100% |
|  |  | Average of bytes / pixel | 2.48 | 2.48 | 100% |
| Bi | Operations | Worst case / pixel | 120.00 | 118.00 | 98% |
|  |  | Average of operations / pixel | 60.21 | 61.67 | 102% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 4.95 | 4.95 | 100% |
| Total | Operations | Worst case / pixel | 120.00 | 118.00 | 98% |
|  |  | Average of operations / pixel | 45.15 | 45.89 | 102% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 3.71 | 3.71 | 100% |

# Conclusion

We tested a Bi/Uni filter which is one of tools in CE3. It is reported that the coding efficiency are -0.5%, -1.1%, +0.1%, -0.2%, -0.1% and -0.3% for RA_HE, RA_LC, LD_HE, LD_LC, LDP_HE and LDP_LC. And we investigated also theoretical complexity analysis. We reported that computation cost at worst case decreases. Encoding time and decoding time is measured using same machine platform. It is reported that the simulation time is slightly decreases.

# References

[1] “Core Experiment 3: Motion Compensation Interpolation”, JCTVC-E703, Mar 2011

[2] “CE3 : Results on Bi/Single MC interpolation filter” , JCTVC-E128, Mar 2011

[3] K. Kondo and T. Suzuki, “TE3 subset4: Results on Bi/Single filter switching in FIF” JCTVC-C164, 3rd. JCT-VC Meeting, Guangzhou, CN, 7-15 Oct, 2010.

[4] F. Bossen Common HM test conditions and software reference configurations JCTVC-E700, 5th. JCTVC, Meeting, Geneva, Mar. 2011.

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).