| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F103 |
| --- | --- |

| Title: | CE4 Subtest 2: QP prediction based on intra/inter prediction (test 2.4.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC corporation |  |  |

_____________________________

# Abstract

This contribution presents the detailed description and the results of a QP prediction method based on intra/inter prediction (test 2.4.b) for CE4 subtest 2. The method predicts QP of the current block from spatially/temporally neighbouring blocks directed by its intra/inter prediction. This concept is naturally derived from the fact that similar texture has similar perceptual sensitivity for human visual system. It is reportedly shown that QP prediction based on inter prediction provides BD-rate gains by 0.43%, 0.44%, 0.74% and 0.89% for RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively, when MinCUDQPSize is set to 8x8. Combined with QP prediction based on intra prediction (test 2.3.g), more BD-rate gains are achieved by 0.40%, 0.37%, 0.59%, 0.63%, 0.83% and 1.02% for AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively. The proposed method definitely provides gains for other MinCUDQPSize settings, even when MinCUDQPSize is set to 64x64. The additional computational complexity is negligible and the coding gain relative to memory requirement by temporal QP prediction is at least comparable to that of temporal MV prediction.

# Introduction

In the current common conditions [1], fixed quantization scheme is used, where all quantization parameter (QP) values are fixed within each frame. However, in real-world applications, adaptive quantization scheme is generally employed for subjective quality improvement and rate control purposes. For subjective quality improvement, perceptually-adaptive quantization technique is widely used, where each QP value is determined according to estimated perceptual sensitivity for human visual system on each area. When perceptually-adaptive quantization technique is used, QP varies frequently according to texture and thereby bit amount for coding QPs can be increased. CE4 Subtest 2 [2] was therefore established at the 5th JCT-VC meeting in order to discuss how to improve coding efficiency of QPs.

In the current WD of HEVC (WD3) [3], directional intra prediction and motion-compensated inter prediction are employed and they significantly help improve its coding efficiency, since neighbouring pixels are generally highly correlated both spatially and temporally in natural images. Supposing that similar texture has similar perceptual sensitivity for human visual system, the best QP value applied for a subimage should be same as that applied for the reference subimage for predicting it. QP prediction based on intra prediction is therefore proposed in JCTVC-E198 [4] and JCTVC-E215 [5], and additionally employing inter prediction is also proposed in JCTVC-E215 [5]. The former is listed in CE4 subtest 2 as the test 2.3.g/2.4.a and the latter is also listed as the test 2.4.b. This contribution focuses on the test 2.4.b.

# Algorithm description

The concept of the proposed QP prediction can be simply described as follows.

If a subimage PD is well-predictable from another subimage PS, the QP variation applied for the subimage PD should be the same as that applied for the subimage PS,

where QP variation means QP offset value derived by some perceptual QP adaptation algorithm like MPEG-2 Test Model 5 Step 3 or its variants [5, 6].

The proposed method leverages information both of intra prediction and inter prediction to derive PS. Prediction is performed on a PU basis and QP is assigned to a set of CUs, which is referred to as quantization group of coding units in WD3 [3] and hereafter denoted as QU in this contribution. Therefore, it should be defined which PU is employed for QP prediction. For simplification, the proposed method employs the top-left-most PU (TLPU) of each QU for QP prediction as shown in Figure 1.

Figure 1: PU employed for QP prediction.

The proposed QP prediction switches between QP prediction based on intra prediction (spatial QP prediction) and that based on inter prediction (temporal QP prediction) according to prediction mode of the TLPU. If a TLPU is intra-predicted, spatial QP prediction shown in Figure 2 is employed for the QU. Otherwise, temporal QP prediction shown in Figure 3 is employed. Detailed algorithm description of spatial QP prediction is described in JCTVC-F159 [7] and therefore omitted here. In this contribution, we focus on temporal QP prediction.

First let us suppose the unidirectional prediction in order to simplify the explanation. Let xTL be the luma location of the top-left luma sample of the current QU relative to the top-left luma sample of the current picture, WTL be the width of the current TLPU, HTL be the height of the current TLPU, vTL be the motion vector of the current TLPU, and FRef be the reference frame indicated by the reference index of the current TLPU. The reference luma location xRef on the frame FRef is derived as follows.

xRef = xTL + (WTL/2, HTL/2) + vTL

This luma location specifies the center of the reference block for motion compensation of the current TLPU. For the proposed inter QP prediction, the QU covering the luma location (xQRef, yQRef) is employed as the reference QU. Let Q(F, x) be the luma quantization parameter of the QU covering a luma location x of the frame F, QSlice(F, x) be the value of SliceQPY of the slice covering a luma location x of the frame F and FCurr be the current frame. The predicted QP value QPred of the current QU is derived as follows.

QPred = Q(FRef, xRef) + QSlice(FCurr, xTL) – QSlice(FRef, xRef)

The offset term by slice QPs accounts for the inter-frame QP adaptation.

When the bidirectional prediction is used for the TLPU, the predicted QP value QPred of the current QU is achieved simply by averaging two predicted QP values as follows.

QPred = (QPredL0 + QPredL1 + 1) / 2

Modification of semantics for derivation of quantization parameter of the current WD is described in Appendix A. No syntax change is introduced by the proposed method.

Figure 2: QP prediction based on intra prediction.

Figure 3: QP prediction based on inter prediction.

# Experiments

## Setup

The proposed QP prediction algorithm has been implemented onto the base software for CE4 Subtest 2 [2]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 2 test conditions [2], with all applicable MinCUDQPSize values, i.e. 8x8, 16x16, 32x32 and 64x64.

Our computing platform used for the experiments is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

It should be noted that individual decoding/encoding time can be significantly varied since the CPU have automated over-clocking functionality and I/O speed varies by 12 processes running in parallel.

## Results

To evaluate gains achieved by introducing inter QP prediction compared with the test 2.4.a employing intra QP prediction, BD-Rates and processing time ratios against test 2.4.a, which employs only intra QP prediction, were computed. Tables 1-5 show the summary of experimental results. Tables 2, 3, 4 and 5 show the summary of experimental results with MinCUDQPSize = 8x8, 16x16, 32x32 and 64x64, respectively. The brief summary of them is shown in Table 1. Note that results for all intra settings are omitted in Tables 1-5 since there is no difference between test 2.4.a (with intra QP prediction) and test 2.4.b (with intra and inter QP prediction). In addition, BD-Rates and processing time ratios against CE4 anchor were computed to evaluate total gains by intra and inter QP prediction. Results are shown in Tables 6-10. More detailed results are shown in the attached spreadsheets.

These results are summarized as follows.

- QP prediction based on inter prediction definitely improves coding efficiency in all configurations using inter prediction. Additional complexity introduced by employing inter prediction is negligible.

- When QP is signalled at 8x8-CU level, its coding gain over the QP prediction based on intra sample prediction is from 0.43 % (for RA-HE) to 0.89 % (for LB-LC). Even when QP is signalled at 64x64-CU level, coding gain is observed from 0.16 % (for RA-HE) to 0.48 % (for LB-HE) and the code amount of delta QPs is significantly reduced by up to 56.8 %.

- Total coding gain of QP prediction based on intra and inter sample prediction over the WD3 anchor is from 0.37 % (for AI-LC) to 1.02 % (for LB-LC) when QP is signalled at 8x8-CU level. Gains can be achieved even when QP is signalled at 64x64-CU level.

Table 1: Brief summary results of test 2.4.b vs. 2.4.a.

| MinCUDQP | Y BD-rates |  |  |  | dQP code amount |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HE | RA LC | LB HE | LB LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -0.43 | -0.44 | -0.74 | -0.89 | -14.7% | -14.8% | -18.8% | -19.4% |
| 16x16 | -0.36 | -0.40 | -0.74 | -0.84 | -20.5% | -20.4% | -27.1% | -26.5% |
| 32x32 | -0.25 | -0.25 | -0.65 | -0.66 | -27.8% | -27.2% | -40.6% | -37.8% |
| 64x64 | -0.16 | -0.16 | -0.48 | -0.43 | -36.7% | -35.3% | -56.8% | -50.7% |

Table 2: Summary results of test 2.4.b vs. 2.4.a with MinCUDQPSize = 8x8.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.4 | -0.6 | -0.2 | -13.4% | -0.3 | -0.3 | -0.5 | -13.4% |
| Class B | -0.4 | -0.4 | -0.4 | -13.8% | -0.4 | -0.4 | -0.4 | -13.1% |
| Class C | -0.5 | -0.6 | -0.6 | -15.7% | -0.5 | -0.7 | -0.5 | -16.0% |
| Class D | -0.5 | -0.7 | -0.6 | -16.3% | -0.6 | -0.7 | -0.5 | -17.3% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.4 | -0.6 | -0.5 | -14.7% | -0.4 | -0.5 | -0.5 | -14.8% |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.8 | -0.7 | -16.8% | -0.7 | -0.6 | -0.6 | -15.9% |
| Class C | -0.7 | -0.6 | -0.9 | -18.7% | -0.8 | -0.8 | -0.8 | -19.3% |
| Class D | -0.7 | -1.0 | -1.1 | -19.5% | -0.9 | -1.0 | -1.1 | -21.3% |
| Class E | -1.0 | -0.6 | -1.3 | -21.4% | -1.4 | -1.5 | -0.9 | -23.0% |
| All | -0.7 | -0.8 | -1.0 | -18.8% | -0.9 | -0.9 | -0.8 | -19.4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 3: Summary results of test 2.4.b vs. 2.4.a with MinCUDQPSize=16x16.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.3 | -0.4 | -0.1 | -16.9% | -0.3 | -0.3 | -0.4 | -16.6% |
| Class B | -0.4 | -0.4 | -0.5 | -18.9% | -0.3 | -0.4 | -0.3 | -17.6% |
| Class C | -0.4 | -0.5 | -0.4 | -21.5% | -0.5 | -0.5 | -0.5 | -22.1% |
| Class D | -0.4 | -0.4 | -0.4 | -24.9% | -0.5 | -0.5 | -0.6 | -25.8% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.4 | -0.4 | -0.4 | -20.5% | -0.4 | -0.4 | -0.4 | -20.4% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.4 | -23.0% | -0.6 | -0.7 | -0.8 | -21.2% |
| Class C | -0.6 | -0.6 | -0.7 | -25.4% | -0.8 | -0.8 | -0.8 | -25.4% |
| Class D | -0.7 | 0.0 | -0.6 | -31.6% | -0.9 | -0.6 | -0.8 | -31.7% |
| Class E | -1.1 | -0.9 | -1.2 | -30.2% | -1.3 | -1.1 | -1.1 | -29.8% |
| All | -0.7 | -0.5 | -0.7 | -27.1% | -0.8 | -0.8 | -0.8 | -26.5% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |

Table 4: Summary results of test 2.4.b vs. 2.4.a with MinCUDQPSize = 32x32.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | 0.0 | -0.3 | -22.8% | -0.2 | -0.2 | -0.2 | -22.7% |
| Class B | -0.3 | -0.3 | -0.3 | -26.1% | -0.3 | -0.3 | -0.3 | -23.9% |
| Class C | -0.2 | -0.4 | -0.2 | -28.1% | -0.3 | -0.2 | -0.2 | -28.5% |
| Class D | -0.3 | -0.2 | -0.2 | -34.5% | -0.3 | -0.2 | -0.3 | -34.8% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.3 | -0.2 | -0.3 | -27.8% | -0.3 | -0.3 | -0.3 | -27.2% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.8 | -0.7 | -34.8% | -0.5 | -0.6 | -0.4 | -30.8% |
| Class C | -0.5 | -0.4 | -0.4 | -36.3% | -0.5 | -0.6 | -0.8 | -34.9% |
| Class D | -0.6 | -0.6 | 0.1 | -46.7% | -0.6 | -0.8 | -0.8 | -44.7% |
| Class E | -1.1 | -1.1 | -1.5 | -48.0% | -1.2 | -1.1 | -1.0 | -44.3% |
| All | -0.7 | -0.7 | -0.6 | -40.6% | -0.7 | -0.7 | -0.7 | -37.8% |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |

Table 5: Summary results of test 2.4.b vs. 2.4.a with MinCUDQPSize = 64x64.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.3 | -0.3 | -0.3 | -32.6% | -0.2 | -0.2 | -0.1 | -30.9% |
| Class B | -0.2 | -0.2 | -0.2 | -36.1% | -0.2 | -0.2 | -0.2 | -33.2% |
| Class C | -0.1 | 0.0 | -0.1 | -35.3% | -0.1 | -0.1 | -0.1 | -35.0% |
| Class D | -0.1 | -0.1 | -0.2 | -42.8% | -0.2 | -0.1 | 0.0 | -42.6% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -36.7% | -0.2 | -0.2 | -0.1 | -35.3% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.3 | -53.2% | -0.3 | -0.3 | -0.4 | -45.3% |
| Class C | -0.3 | -0.1 | -0.2 | -47.7% | -0.3 | -0.4 | -0.1 | -44.5% |
| Class D | -0.3 | 0.1 | 0.0 | -57.4% | -0.3 | -0.2 | 0.3 | -53.9% |
| Class E | -1.1 | -1.5 | -0.6 | -74.4% | -1.0 | -1.3 | -0.4 | -63.9% |
| All | -0.5 | -0.4 | -0.3 | -56.8% | -0.4 | -0.5 | -0.2 | -50.7% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 6: Brief summary results of test 2.4.b vs. CE4 anchor.

| MinCUDQP | Y BD-rates |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -0.40 | -0.37 | -0.59 | -0.63 | -0.83 | -1.02 |
| 16x16 | -0.19 | -0.17 | -0.44 | -0.48 | -0.79 | -0.91 |
| 32x32 | -0.06 | -0.06 | -0.28 | -0.29 | -0.64 | -0.70 |
| 64x64 | -0.02 | -0.02 | -0.17 | -0.17 | -0.45 | -0.45 |
| MinCUDQP | dQP code amount |  |  |  |  |  |
|  | AI HE | AI LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 | -10.8% | -10.0% | -18.3% | -18.7% | -20.3% | -21.2% |
| 16x16 | -10.1% | -9.3% | -23.0% | -23.0% | -28.2% | -27.8% |
| 32x32 | -7.9% | -7.6% | -29.2% | -28.8% | -41.2% | -38.6% |
| 64x64 | -5.4% | -5.5% | -37.3% | -36.0% | -57.1% | -51.0% |

Table 7: Summary results of test 2.4.b vs. CE4 anchor with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.2 | -0.2 | -6.2% | -0.2 | -0.2 | -0.2 | -5.9% |
| Class B | -0.3 | -0.3 | -0.3 | -9.9% | -0.3 | -0.3 | -0.3 | -9.0% |
| Class C | -0.4 | -0.4 | -0.4 | -11.7% | -0.4 | -0.4 | -0.4 | -11.1% |
| Class D | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.8% |
| Class E | -0.8 | -0.8 | -0.8 | -17.0% | -0.6 | -0.6 | -0.6 | -14.9% |
| All | -0.4 | -0.4 | -0.4 | -10.8% | -0.4 | -0.4 | -0.4 | -10.0% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.5 | -0.8 | -0.3 | -15.6% | -0.4 | -0.3 | -0.5 | -15.7% |
| Class B | -0.6 | -0.5 | -0.6 | -17.6% | -0.5 | -0.6 | -0.6 | -17.1% |
| Class C | -0.7 | -0.7 | -0.8 | -20.3% | -0.8 | -0.8 | -0.7 | -21.0% |
| Class D | -0.6 | -0.6 | -0.6 | -19.8% | -0.8 | -0.9 | -0.8 | -21.3% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.6 | -0.7 | -0.6 | -18.3% | -0.6 | -0.6 | -0.7 | -18.7% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.7 | -18.3% | -0.8 | -0.7 | -0.6 | -17.7% |
| Class C | -0.9 | -0.7 | -1.0 | -20.9% | -1.0 | -0.8 | -1.1 | -22.0% |
| Class D | -0.8 | -1.0 | -1.0 | -20.9% | -1.1 | -0.9 | -1.4 | -23.0% |
| Class E | -1.1 | 0.3 | -0.3 | -22.3% | -1.4 | -1.5 | -1.2 | -23.7% |
| All | -0.8 | -0.6 | -0.8 | -20.3% | -1.0 | -0.9 | -1.0 | -21.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |

Table 8: Summary results of test 2.4.b vs. CE4 anchor with MinCUDQPSize = 16x16.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | -0.1 | -5.1% | -0.1 | -0.1 | -0.1 | -4.9% |
| Class B | -0.2 | -0.2 | -0.2 | -9.5% | -0.1 | -0.1 | -0.1 | -8.5% |
| Class C | -0.2 | -0.2 | -0.1 | -10.5% | -0.2 | -0.2 | -0.2 | -10.3% |
| Class D | -0.1 | -0.1 | -0.1 | -8.7% | -0.1 | -0.1 | -0.1 | -9.0% |
| Class E | -0.5 | -0.5 | -0.5 | -18.7% | -0.4 | -0.4 | -0.4 | -15.6% |
| All | -0.2 | -0.2 | -0.2 | -10.1% | -0.2 | -0.2 | -0.2 | -9.3% |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 104% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.4 | -0.3 | -0.3 | -18.4% | -0.4 | -0.4 | -0.4 | -18.3% |
| Class B | -0.4 | -0.6 | -0.5 | -21.8% | -0.4 | -0.5 | -0.4 | -20.6% |
| Class C | -0.5 | -0.5 | -0.5 | -24.8% | -0.6 | -0.6 | -0.7 | -25.6% |
| Class D | -0.4 | -0.5 | -0.5 | -27.2% | -0.6 | -0.6 | -0.7 | -28.3% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.5 | -23.0% | -0.5 | -0.5 | -0.5 | -23.0% |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.4 | -24.2% | -0.7 | -0.8 | -0.9 | -22.6% |
| Class C | -0.7 | -0.6 | -0.9 | -27.0% | -0.9 | -0.9 | -0.8 | -27.3% |
| Class D | -0.7 | -0.1 | -0.6 | -32.5% | -0.9 | -0.8 | -0.9 | -32.9% |
| Class E | -1.2 | -1.2 | -0.7 | -30.8% | -1.3 | -1.2 | -1.1 | -30.4% |
| All | -0.8 | -0.7 | -0.6 | -28.2% | -0.9 | -0.9 | -1.0 | -27.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 101% |  |  |  |

Table 9: Summary results of test 2.4.b vs. CE4 anchor with MinCUDQPSize = 32x32.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.1 | -3.2% | 0.0 | 0.0 | 0.0 | -3.2% |
| Class B | -0.1 | -0.1 | 0.0 | -7.4% | -0.1 | 0.0 | 0.0 | -6.7% |
| Class C | 0.0 | 0.0 | 0.0 | -8.1% | 0.0 | 0.0 | 0.0 | -8.3% |
| Class D | 0.0 | 0.0 | 0.0 | -5.7% | 0.0 | 0.0 | 0.0 | -6.5% |
| Class E | -0.2 | -0.2 | -0.2 | -17.7% | -0.2 | -0.2 | -0.2 | -15.5% |
| All | -0.1 | -0.1 | 0.0 | -7.9% | -0.1 | -0.1 | -0.1 | -7.6% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.1 | -0.5 | -23.7% | -0.3 | -0.3 | -0.2 | -23.6% |
| Class B | -0.3 | -0.3 | -0.4 | -27.9% | -0.3 | -0.3 | -0.3 | -25.8% |
| Class C | -0.3 | -0.4 | -0.2 | -29.9% | -0.3 | -0.2 | -0.3 | -30.4% |
| Class D | -0.3 | -0.3 | -0.3 | -35.5% | -0.3 | -0.3 | -0.3 | -36.0% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -29.2% | -0.3 | -0.3 | -0.3 | -28.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.7 | -0.5 | -35.6% | -0.6 | -0.6 | -0.6 | -31.8% |
| Class C | -0.5 | -0.5 | -0.5 | -37.2% | -0.6 | -0.4 | -0.7 | -35.9% |
| Class D | -0.5 | -0.4 | -0.4 | -47.1% | -0.6 | -0.9 | -0.9 | -45.3% |
| Class E | -1.1 | -1.0 | -0.4 | -48.2% | -1.2 | -1.0 | -1.3 | -44.6% |
| All | -0.6 | -0.6 | -0.5 | -41.2% | -0.7 | -0.7 | -0.8 | -38.6% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

Table 10: Summary results of test 2.4.b vs. CE4 anchor with MinCUDQPSize = 64x64.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.1 | -1.7% | 0.0 | 0.0 | 0.0 | -1.8% |
| Class B | 0.0 | 0.0 | 0.0 | -5.1% | 0.0 | 0.0 | 0.0 | -4.7% |
| Class C | 0.0 | 0.0 | 0.0 | -6.1% | 0.0 | 0.0 | 0.0 | -6.8% |
| Class D | 0.0 | 0.0 | 0.0 | -1.8% | 0.0 | 0.0 | 0.0 | -2.1% |
| Class E | -0.1 | -0.1 | 0.0 | -15.0% | -0.1 | -0.1 | -0.1 | -14.9% |
| All | 0.0 | 0.0 | 0.0 | -5.4% | 0.0 | 0.0 | 0.0 | -5.5% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.3 | -0.5 | -0.3 | -33.0% | -0.2 | -0.2 | -0.2 | -31.5% |
| Class B | -0.2 | -0.2 | -0.2 | -37.0% | -0.2 | -0.2 | -0.2 | -34.2% |
| Class C | -0.1 | -0.1 | -0.2 | -36.2% | -0.1 | -0.1 | -0.1 | -36.0% |
| Class D | -0.1 | -0.1 | -0.2 | -43.1% | -0.2 | -0.1 | 0.0 | -42.9% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -37.3% | -0.2 | -0.2 | -0.1 | -36.0% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.4 | -53.5% | -0.4 | -0.4 | -0.4 | -45.7% |
| Class C | -0.3 | -0.1 | -0.3 | -48.1% | -0.3 | -0.5 | -0.2 | -45.0% |
| Class D | -0.3 | 0.2 | -0.1 | -57.6% | -0.3 | -0.3 | 0.3 | -54.1% |
| Class E | -0.9 | -1.9 | -1.3 | -74.6% | -1.0 | -1.1 | -0.8 | -64.0% |
| All | -0.4 | -0.5 | -0.4 | -57.1% | -0.4 | -0.5 | -0.3 | -51.0% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

# Remarks

## Analysis on memory requirement

While computational complexity of the proposed QP prediction is negligible, temporal QP prediction (TQPP) introduces additional buffer for storing QPs of reference frames for temporal prediction. Here we analyze the balance between compression performance and memory requirement of TQPP. Since temporal MV prediction (TMVP) has been adopted into WD3, comparing the performance of TQPP with that of TMVP should result in a good indication for judging whether temporal QP prediction should be adopted into HEVC or not.

To evaluate the balance between compression performance and memory requirement, ratio of coding gain to bits per reference SCU [8], which is calculated by dividing the negative of the luma BD-Rate value by the number of bits consumed for each SCU on reference frame, is employed. As for TQPP, the value is calculated as follows:

RTQP = B  (6  2log2MinCUDQPSize-Log2MinCUSize),

where B is BD-Rate value and 6 means the number of bits consumed for storing each QP.

Calculated ratio values, namely relative coding gains, are shown in Table 11. Those for TMVP are quoted from JCTVC-F099 [8] and values for TQPP are calculated from results shown in Tables 1. As shown here, coding gain relative to memory requirement of TQPP is less than those of TMVP when delta QP is signalled at 8x8 block level. However, when delta QP is signalled at more than 8x8 block level, relative coding gain of TQPP exceeds that of TMVP. When delta QP is signalled at 64x64 block level, the relative coding gain of TQPP attains up to about 30 times of that of TMVP, for the LB-HE configuration. Although it does not mean that TQPP is up to 30 times better than TMVP, it indicates that introducing TQPP should be at least considered. It should be noted that, in these results, the default (16x) buffer compression is applied to TMVP. If some buffer compression scheme is also applied to TQPP, its relative coding gain is expected to increase more.

Table 11: Ratios of coding gain to bits per reference SCU.

|  | TMVP | TQPP |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | 16x16 | 32x32 | 64x64 |
| RA.HE | 0.17 | 0.07 | 0.24 | 0.67 | 1.76 |
| RA.LC | 0.16 | 0.07 | 0.27 | 0.67 | 1.68 |
| LB.HE | 0.17 | 0.12 | 0.49 | 1.74 | 5.07 |
| LB.LC | 0.17 | 0.15 | 0.56 | 1.77 | 4.57 |

## Remarks on usability of temporal QP prediction

While experimental results have shown remarkable performance of temporal QP prediction, there may be some doubts whether temporal QP prediction should be effective in any cases. In real applications, some rate control would be applied since generated bitstreams must conform to some constraint imposed on transmission channel. Although the proposed temporal QP prediction introduces offset by Slice QP and it provides some adaptability to rate control, there definitely are cases where more strict rate control are needed and QP for each CU significantly varies within a slice. For example, at early frames of the whole sequence to be encoded, rate control tends to be unstable and thereby CU QP can be unstable. Supposing such cases, switching mechanism for enabling/disabling temporal QP prediction is better to be provided.

We therefore suggest introducing a flag to control availability of temporal QP prediction, for example, into PPS. The alternative proposed modification of the WD3 text with the flag, namely temporal_qp_prediction_enabled_flag, is described in Appendix B. Since the current HM has no rate control mechanism and the QP assignment in the common test conditions is perfectly stable, no additional experimental results are given here. It should be noted that fixing temporal_qp_prediction_enabled_flag equal to 1 corresponds to the method described in this contribution (test 2.4.b) and fixing temporal_qp_prediction_enabled_flag equal to 0 conrresponds to the method described in JCTVC-F159 (test 2.3.g/2.4.a) [7].

# Conclusions

This contribution has presented the detailed description and the results of a QP prediction method based on intra/inter prediction (test 2.4.b) for CE4 subtest 2. The concept of the proposed method is naturally derived from the fact that similar texture has similar perceptual sensitivity for human visual system. Experimental results show that QP prediction based on inter prediction provides BD-rate gains by 0.43%, 0.44%, 0.74% and 0.89% for RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively, when MinCUDQPSize is set to 8x8. Moreover, combined with QP prediction based on intra prediction (test 2.3.g/2.4.a), it provides more BD-rate gains by 0.40%, 0.37%, 0.59%, 0.63%, 0.83% and 1.02% for AI-HE, AI-LC, RA-HE, RA-LC, LB-HE and LB-LC configurations, respectively. The proposed method definitely provides gains for other MinCUDQPSize settings, even when MinCUDQPSize is set to 64x64. The additional computational complexity is negligible and the coding gain relative to memory requirement by temporal QP prediction is at least comparable to that of temporal MV prediction. It is therefore recommended to adopt the proposed QP prediction based on intra/inter prediction into HM and to consider introducing some buffer compression scheme over the proposed QP prediction.

# References

[1] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

[2] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[3] T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC document JCTVC-E603, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[4] M. Kobayashi and M. Shima, “Sub-LCU level delta QP signaling,” JCT-VC document JCTVC-E198, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[5] H. Aoki, et al., “Prediction-based QP derivation,” JCT-VC document JCTVC-E215, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

[6] K. Sato, et al., “On LBS and Quantization,” JCT-VC document JCTVC-D308, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

[7] H. Aoki, M. Kobayashi, et al., “CE4 Subtest 2: QP prediction based on intra prediction (test 2.3.g),” JCT-VC document JCTVC-F159, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

[8] H. Aoki, et al., “Performance report of temporal motion vector prediction,” JCT-VC document JCTVC-F099, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

NEC corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

The modified or added portions are highlighted with yellow or green colour.

Note: This modification includes semantics change by QP prediction based on intra prediction, which is described in JCTVC-F159 [7]. The portions described in [7] are highlighted with yellow colour and the portions introduced in this contribution are highlighted with green colour.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, spatial quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, temporal quantisation parameter prediction process as specified in subclause 8.5.x.3 is invoked with the output being the predicted quantisation parameter QPY, PRED.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Default quantisation parameter prediction process

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Spatial quantisation parameter prediction process

Inputs to this process are

–	a variable intraPCMFlag specifying if top-left coding unit of the current quantisation group of coding units is coded in PCM mode.

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- intraPCMFlag is equal to 1.

- intraPredMode is equal to 2 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 0 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 1 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-6.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is more than 0 and less than 18 and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is equal to or more than 18 and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.

### 8.5.x.3		Temporal quantisation parameter prediction process

Inputs to this process are

–	a luma location ( xQ, yQ ) specifying the top-left luma sample of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the top-left prediction unit of the current quantisation group of coding units, nPSW and nPSH,

–	luma motion vectors mvL0 and mvL1 of the top-left prediction unit of the current quantisation group of coding units,

–	reference indices refIdxL0 and refIdxL1 of the top-left prediction unit of the current quantisation group of coding units,

–	prediction list utilization flags predFlagL0 and predFlagL1 of the top-left prediction unit of the current quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

For X being replaced by 0 or 1, when predFlagLX is equal to 1, the uni-predicted quantisation parameter PredQPLX is derived in the following ordered steps.

- The variables xQRefLX and yQRefLX specifying the center of the reference block used for motion compensation of the top-left prediction unit of the current quantisation group of coding units are derived as follows.

xQRefLX = xQ + nPSW/2 + ( mvLX[ 0 ] >> 2 )					(8-xx3)
yQRefLX = yQ + nPSH/2 + ( mvLX[ 1 ] >> 2 )						(8-xx4)

- Let variables RefQPLX and SliceQPLX be set as follows.

- RefQPLX is set equal to the luma quantisation parameter of the quantisation group of coding units covering luma location (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- SliceQPLX is the initial value of quantisation parameter of the slice covering luma location  (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- The uni-predicted quantisation parameter PredQPLX is derived as follows.

PredQPLX = RefQPLX + SliceQPY – SliceQPLX,					(8-xx5)

where SliceQPY is the initial value of quantisation parameter of the current slice.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

QPY,PRED = PredQPL0									(8-xx6)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

QPY,PRED = PredQPL1									(8-xx7)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

QPY,PRED = ( PredQPL0 + PredQPL1 + 1 ) >> 1						(8-xx8)

Appendix B: Proposed modification of HEVC Working Draft 3 with switching control of availability of temporal QP prediction

The modified or added portions over WD3 are highlighted with yellow or green colour. The portions described in Appendix A are highlighted with yellow colour and the portions introduced in this section are highlighted with green colour.

### B.1. Change of subclause 7.3.2.2 “Picture parameter set RBSP syntax”

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| max_cu_qp_delta_depth | u(4) |
| temporal_qp_prediction_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### B.2. Change of subclause 7.4.2.2 “Picture parameter set RBSP semantics”

temporal_qp_prediction_enabled_flag equal to 1 specifies that temporal QP prediction is enabled in derivation process for quantisation parameters. temporal_qp_prediction_enabled_flag equal to 0 specifies that temporal QP prediction is disabled in derivation process for quantisation parameters. When temporal_qp_prediction_enabled_flag is not present, temporal_qp_prediction_enabled_flag shall be inferred to be equal to 0.

### B.3. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-5)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-6)

### B.4. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, spatial quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, if temporal_qp_prediction_enabled_flag is equal to 1, temporal quantisation parameter prediction process as specified in subclause 8.5.x.3 is invoked with the output being the predicted quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the predicted quantisation parameter QPY, PRED.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-7)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-8)

### 8.5.x.1		Default quantisation parameter prediction process

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Spatial quantisation parameter prediction process

Inputs to this process are

–	a variable intraPCMFlag specifying if top-left coding unit of the current quantisation group of coding units is coded in PCM mode.

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- intraPCMFlag is equal to 1.

- intraPredMode is equal to 2 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 0 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 1 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-6.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is more than 0 and less than 18 and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is equal to or more than 18 and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.

### 8.5.x.3		Temporal quantisation parameter prediction process

Inputs to this process are

–	a luma location ( xQ, yQ ) specifying the top-left luma sample of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the top-left prediction unit of the current quantisation group of coding units, nPSW and nPSH,

–	luma motion vectors mvL0 and mvL1 of the top-left prediction unit of the current quantisation group of coding units,

–	reference indices refIdxL0 and refIdxL1 of the top-left prediction unit of the current quantisation group of coding units,

–	prediction list utilization flags predFlagL0 and predFlagL1 of the top-left prediction unit of the current quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

For X being replaced by 0 or 1, when predFlagLX is equal to 1, the uni-predicted quantisation parameter PredQPLX is derived in the following ordered steps.

- The variables xQRefLX and yQRefLX specifying the center of the reference block used for motion compensation of the top-left prediction unit of the current quantisation group of coding units are derived as follows.

xQRefLX = xQ + nPSW/2 + ( mvLX[ 0 ] >> 2 )					(8-xx3)
yQRefLX = yQ + nPSH/2 + ( mvLX[ 1 ] >> 2 )						(8-xx4)

- Let variables RefQPLX and SliceQPLX be set as follows.

- RefQPLX is set equal to the luma quantisation parameter of the quantisation group of coding units covering luma location (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- SliceQPLX is the initial value of quantisation parameter of the slice covering luma location  (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- The uni-predicted quantisation parameter PredQPLX is derived as follows.

PredQPLX = RefQPLX + SliceQPY – SliceQPLX,					(8-xx5)

where SliceQPY is the initial value of quantisation parameter of the current slice.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

QPY,PRED = PredQPL0									(8-xx6)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

QPY,PRED = PredQPL1									(8-xx7)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

QPY,PRED = ( PredQPL0 + PredQPL1 + 1 ) >> 1						(8-xx8)