| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F104 |
| --- | --- |

| Title: | On Intra Smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | yongjoon.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

The intra smoothing of HM filters the 4N-1 neighbouring samples to increase prediction accuracy where N is the size of PU. It is observed that at most half (2N-1 or 2N+1) of samples out of the filtered 4N-1 neighbouring samples are used for prediction. Therefore the other half (2N or 2N-2) of the samples are filtered and never used for prediction. The filtering for the unused samples is obviously a useless waste of complexity and this waste always happens in the current MDIS (Mode Dependent Intra Smoothing). In this contribution, a look-up table is introduced to filter only the samples which could be actually used for prediction. By the proposed look-up table, the waste can be minimized and the number of filtering operation for intra smoothing is reduced by half compared to the current MDIS of HM.

# Problem

In the intra smoothing of HM [1], the 4N-1 neighbouring samples around the current PU are filtered for prediction using [1 2 1] filter before the prediction process where N is the size of PU. Fig. 1 represents the samples to be filtered for prediction (The marked samples).

- Fig. 1 The neighboring samples to be filtered in AIS

However, it is observed that only a part of samples out of the 4N-1 neighboring samples are used for prediction depending on the intra prediction angle. Fig. 2 illustrates the case when the intra prediction mode is 16. The marked samples are filtered while only the left neighboring samples are used in the prediction process.

- Fig. 2 An example showing the filtered samples and the samples used for prediction

Therefore, more than half of the neighboring samples are filtered and never used for prediction. This filtering for the unused samples is obviously a useless waste of complexity and this waste always happens in the current MDIS.

# Proposed method

From the investigation, it is observed that the samples which are actually used for prediction depends on prediction direction as shown in Fig. 3.

Fig. 3 The samples used for prediction according to prediction direction

The samples used for prediction according to intra prediction mode can be categorized into 3 groups as shown in Table 1. In this table, N specifies the prediction size and p[x, y] specifies neighbouring samples where p[0, 0] is the top-left sample of the current block.

Table 1. Group of samples used for prediction according to intra pred. mode

| Group number | Intra pred. mode | Pred. direction | Samples used for prediction |
| --- | --- | --- | --- |
| 0 | 5, 6, 12, 13, 22, 23, 24, and 25 | Down-left | p[x, -1], x = 0 .. 2N-1 |
| 1 | 3, 4, 7, 10, 11, 14, 15,18, 19, 20, 21, 26, 27, 28, and 29 | Down-right | p[-1, y ], y = N-1 … 0 p[-1, -1], p[x, -1], x = 0 .. N-1 |
| 2 | 8, 9, 16, 17, 30, 31, 32, and 33 | Up-right | p[-1, y], y = 0 .. 2N-1 |

From this observation, the samples used for prediction for each intra prediction mode can be indicated by using the group number. Therefore, the filtering operation for the unused samples can be easily eliminated by using the following look-up table.

Table 2. Proposed LUT

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group | - | - | - | 1 | 1 | 0 | 0 | 1 | 2 | 2 | 1 | 1 | 0 | 0 | 1 | 1 | 2 |
| intraPredMode | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| Group | 2 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |

Fig. 4 summarizes the proposed method. In the proposed method, the samples which will not be used for prediction are not filtered by the proposed look-up table. The relevant draft text is provided at the end of this document.

Fig. 4 The proposed method (LUT-based filtering)

# Simulation result

The proposed method is implemented in the HM3 software as follows.

  - Encoder : No change has been made because the current encoder filters the neighbouring samples and the filtered samples are shared by all the intra prediction modes which are tested. This is the best way of doing it.

  - Decoder : The proposed method is implemented such that only the samples indicated by the proposed LUT are filtered depending on its intra prediction mode if the MDIS filter type is 1.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The simulation results show that the proposed method does not affect on the coding efficiency of the HM. The encoding time is exactly same as the HM3 anchor because no change has been made by the proposed method at the encoder. The decoding time reduction is not significant because the portion of the intra smoothing is relatively small compared to the whole decoding process. However it is obvious that the proposed method reduces the filtering operation for intra smoothing by half compared to the current MDIS of HM.

# Conclusion

In this contribution, a LUT-based filtering method is proposed to minimize the useless filtering operation which should be done in the current HM. It is asserted that the filtering of the samples which will never be used for prediction is a useless waste of complexity and should be eliminated. The proposed method reduces the number of filtering operations for the intra smoothing by half compared to the current MDIS of HM. Therefore we recommend the proposed LUT to be adopted in the next version of HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E603, “WD3: Working Draft 3 of High-Efficiency Video Coding,” 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

##### Annex – draft text

The provided text is based on JCTVC-E603_d8.doc document and the proposed changes are marked in yellow.

##### 8.3.3.1.2 Filtering process of neighbouring samples

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	filtered samples pF[ x, y ],. with x, y = -1..2*nS-1.

Table 8-2 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 0 |
| 9 | 1 | 1 | 1 | 1 | 0 |
| 10 | 0 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 1 | 1 | 0 |
| 12 | 0 | 0 | 1 | 1 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 |
| 14 | 0 | 0 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 1 | 0 |
| 17 | 0 | 0 | 1 | 1 | 0 |
| 18-33 | 0 | 0 | 0 | 1 | 0 |
| 34 | 0 | 0 | 0 | 0 | 0 |
| 35 | n/a | n/a | n/a | n/a | n/a |

Table 8-6 – Specification of intraFilterMap[ IntraPredMode ]

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraFilterMap | - | - | - | 1 | 1 | 0 | 0 | 1 | 2 | 2 | 1 | 1 | 0 | 0 | 1 | 1 | 2 | 2 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| intraFilterMap | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | - | - |

Filtered sample array pF[ x, y ] with x = -1..nS*2-1 and y = -1..nS*2-1  are derived as follows:

–	When intraFilterType[ nS ][ IntraPredMode ] is equal to 1, the following applies:

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]		(8-1)

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]		(8-2)

pF[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-3)

pF[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2	(8-4)

pF[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-21)

- If intraFilterMap[ IntraPredMode ] is equal to 0,

pF[ -1, y ] = p[ -1, y ] for y = nS*2-1..-1

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]

pF[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2

- If intraFilterMap [ IntraPredMode ] is equal to 1,

pF[ -1 , y ] = p[ -1, y ] for y = nS*2-2.. nS

pF[ x, -1 ] = p[ x, -1 ] for x = nS.. nS*2-1

pF[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS-1.. 0

pF[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2

pF[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*-1

- If intraFilterMap [ IntraPredMode ] is equal to 2,

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]

pF[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0

pF[ x, -1 ] = p[ x, -1 ] for x = -1..nS*2-1