| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F105 |
| --- | --- |

| Title: | On MVP list pruning process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | yongjoon.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the AMVP process, ‘identical’ MVP (Motion Vector Predictor) candidates are removed from the MVP list. This contribution proposes to allow ‘different’ MVP candidates to be removed from the MVP list if the difference between motion vectors is smaller than or equal to a threshold value. Various threshold values are tested and the threshold value is determined to 5 considering overall coding efficiency. The proposed technique changes only one condition in the AMVP process and this minor change results in 0.2% BD-rate reduction for the random access and low delay high efficiency configurations and 0.1% BD-rate reduction for the random access and low delay low complexity configurations.

# Introduction

AMVP (Advanced Motion Vector Coding) [1] is a technique to improve the accuracy of MVP (Motion Vector Predictor) by using the MVP index that specifies the best MVP in terms of the R-D optimization.

In the AMVP process, MVP candidates are derived from neighboring prediction unit partitions and a MVP list is constructed. Among the motion vectors in the MVP list, redundant motion vectors which have the same value could exist. Since the redundant motion vectors have the same value, they only make the number of MVP candidates increased, which results in the unnecessary bit increase for the MVP index. Therefore AMVP has the MVP list pruning process which removes identical motion vectors from the MVP list.

Even though the MVP list pruning process minimizes the bits for the MVP index, the current process might not be the optimal in terms of the R-D optimization perspective in the following reason. : In the current process, different motion vectors cannot be removed from the list even though the impact of using all of them is negligibly small. However, if the impact of using all the different MVP candidates is negligibly small, it is better to remove some of them because the bits for the MVP index can be saved by removing them. For instance, if there are two motion vectors in the list and the impact of using both of them is negligibly small, it is better to remove one of them because the MVP index does not need to be sent to decoder if we remove one of them. Similar cases could be happen when there are more than two motion vectors. Therefore, if we consider some different MVP candidates as redundant and remove them in terms of RD optimization perspective, the number of MVP candidates can be reduced so the bits for the MVP index can be reduced further. In this sense, there is still a room for improvement in the current MVP list pruning process.

# Proposed method

In the MVP list pruning process, the pruning condition is modified so that the MVP candidates are removed from the MVP list if the motion vectors have the difference smaller than or equal to a threshold value with other motion vectors in the MVP list.

The following pseudo codes summarize the change by the proposed method. It is supposed that mvA has lower order than mvB in the MVP list.

HM

- if( mvA.x == mvB.x && mvA.y == mvB.y )

  - remove mvB from the MVP list

Proposed

- if( |mvA.x - mvB.x| + |mvA.y - mvB.y| < threshold )

  - remove mvB from the MVP list

The relevant draft text is provided at the end of this document.

# Experimental results

This proposed scheme is implemented into HM3.0 software. Various threshold values (1 to 10) are tested to find the optimal threshold value. The common test conditions [2] are used. Table 1 shows the overall BD-rate reduction for all threshold values for each coding configurations. Detailed results can be found in the accompanying excel sheet.

Table 1. Overall BD-rate reduction for various threshold values

|  | RAHE |  |  | RALC |  |  | LDHE |  |  | LDLC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Threshold | BD-Y | BD-U | BD-V | BD-Y | BD-U | BD-V | BD-Y | BD-U | BD-V | BD-Y | BD-U | BD-V |
| 1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.1 | -0.1 | -0.1 | -0.2 |
| 2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 | -0.4 |
| 3 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | -0.3 |
| 4 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | -0.2 | -0.2 | -0.2 | -0.1 | -0.3 | -0.3 |
| 5 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.3 |
| 6 | -0.2 | -0.2 | -0.1 | 0.0 | -0.1 | 0.0 | -0.2 | -0.1 | -0.1 | -0.1 | -0.3 | -0.3 |
| 7 | -0.1 | -0.2 | -0.1 | 0.0 | -0.1 | -0.1 | -0.2 | -0.1 | -0.2 | -0.1 | -0.1 | -0.3 |
| 8 | -0.1 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 |
| 9 | -0.1 | -0.2 | -0.1 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 | -0.3 |
| 10 | -0.1 | -0.2 | -0.1 | 0.0 | 0.0 | 0.1 | -0.2 | 0.1 | 0.0 | -0.1 | -0.1 | -0.3 |

Fig. 1. BD-Y results for various threshold values

The best threshold values for each configuration are as follows.

- Random Access High Efficiency	: threshold 5

- Random Access Low Complexity	: threshold 5

- Low Delay High Efficiency		: threshold 4

- Low Delay Low Complexity		: threshold 3

The best threshold value is determined to 5 considering overall coding efficiency for all the configurations. The following figures show the results for threshold 5.

Fig. 2. BD rate (%) for Random Access High Efficiency configuration

Fig. 3. BD rate (%) for Random Access Low Complexity configuration

Fig. 4. BD rate (%) for Low Delay High Efficiency configuration

Fig. 5. BD rate (%) for Low Delay Low Complexity configuration

Fig.6 summarizes the results for threshold 5.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.3 | -0.2 | -0.1 | -0.2 | 0.1 |
| Class B | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class D | -0.2 | -0.2 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | -0.1 | -0.1 | -0.1 | -0.5 |
| Class C | -0.2 | -0.1 | -0.1 | -0.2 | 0.0 | -0.2 |
| Class D | -0.2 | -0.4 | 0.0 | -0.2 | -0.1 | -0.1 |
| Class E | -0.2 | 0.0 | -0.3 | -0.1 | 0.0 | -0.6 |
| All | -0.2 | 0.0 | -0.1 | -0.1 | -0.1 | -0.3 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Fig. 6 Results for threshold 5

# Conclusion

This contribution proposes to allow different MVP candidates removed from the MVP list if the difference between motion vectors is smaller than or equal to a threshold value, which is different from the current design where only the identical motion vectors can be removed from the MVP list.

The proposed scheme changes only one condition in the AMVP process and this minor change results in 0.2% BD-rate reduction for the random access and low delay high efficiency configurations and 0.1% BD-rate reduction for the random access and low delay low complexity configurations. Therefore, we recommend the proposed technique to be adopted in the next version of HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E603, “WD3: Working Draft 3 of High-Efficiency Video Coding,” 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- JCTVC-E700, “Common test conditions and software reference configurations”, 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

##### Annex – draft text

In the current draft, the MVP list pruning process is described with only one sentence. However, we think that this sentence does not describe the process specifically. Therefore the current description for the process should be replaced for the specific description as follows.

From

If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

To

The pruning process for motion vector predictor list in subclause 8.4.2.1.8 is invoked with the mvpListLX and the number of elements NumMVPCand(LX) within the mvpListLX as the inputs and with the output being the modified mvpListLX and the modified number of elements NumMVPCand(LX) within the mvpListLX. 8.4.2.1.8 Pruning process for motion vector predictor list Inputs to this process are (with N being replaced by A,or B) - the motion vector predictor candidate list mvpListLX, - the number of elements NumMVPCand( LX ) within the mvpListLX Outputs of this process are - the modified motion vector predictor candidate list mvpListLX, - the modified number of elements NumMVPCand( LX ) within the mvpListLX If numMVPCand( LX ) is greater than 0, the following applies: - The variable listIdx is initially set to 0. - The variable numMvpCand is set equal to NumMVPCand(LX). - The following procedure is conducted with the variable array of motion vectors mvList as follows. mvList[ listIdx ][ 0 ] = mvpListLX[ 0 ][ 0 ] mvList[ listIdx ][ 1 ] = mvpListLX[ 0 ][ 1 ] listIdx++ for( i = 1; i < numMvpCand; i++ ) { for( j = listIdx – 1; j >= 0; j-- ) { if(mvpListLX[ i ][ 0 ] == mvList[ j ][ 0 ] && mvpListLX[ i ][ 1 ] == mvList[ j ][ 1 ] ) break } if( j < 0 ) { mvList[ listIdx ][ 0 ] = mvpListLX[ i ][ 0 ] mvList[ listIdx ][ 1 ] = mvpListLX[ i ][ 1 ] listIdx++ } } for( i = 0; i < listIdx; i++ ) { mvpListLX[ i ] = mvList[ i ] } NumMVPCand( LX ) = listIdx

Based on the above description, the process is modified to reflect the proposed change as follows. The change by the proposed method is marked in yellow.

The pruning process for motion vector predictor list in subclause 8.4.2.1.8 is invoked with the mvpListLX and the number of elements NumMVPCand(LX) within the mvpListLX as the inputs and with the output being the modified mvpListLX and the modified number of elements NumMVPCand(LX) within the mvpListLX. 8.4.2.1.8 Pruning process for motion vector predictor list Inputs to this process are (with N being replaced by A,or B) - the motion vector predictor candidate list mvpListLX, - the number of elements NumMVPCand( LX ) within the mvpListLX Outputs of this process are - the modified motion vector predictor candidate list mvpListLX, - the modified number of elements NumMVPCand( LX ) within the mvpListLX If numMVPCand( LX ) is greater than 0, the following applies: - The variable listIdx is initially set to 0. - The variable numMvpCand is set equal to NumMVPCand(LX). - The following procedure is conducted with the variable array of motion vectors mvList as follows. mvList[ listIdx ][ 0 ] = mvpListLX[ 0 ][ 0 ] mvList[ listIdx ][ 1 ] = mvpListLX[ 0 ][ 1 ] listIdx++ for( i = 1; i < numMvpCand; i++ ) { for( j = listIdx – 1; j >= 0; j-- ) { if(mvpListLX[ i ][ 0 ] == mvList[ j ][ 0 ] && mvpListLX[ i ][ 1 ] == mvList[ j ][ 1 ] ) diff = Abs( mvpListLX[ i ][ 0 ] – mvList[ j ][ 0 ] ) + Abs( mvpListLX[ i ][ 1 ] – mvList[ j ][ 1 ] ) if( diff <= 5 ) break } if( j < 0 ) { mvList[ listIdx ][ 0 ] = mvpListLX[ i ][ 0 ] mvList[ listIdx ][ 1 ] = mvpListLX[ i ][ 1 ] listIdx++ } } for( i = 0; i < listIdx; i++ ) { mvpListLX[ i ] = mvList[ i ] } NumMVPCand( LX ) = listIdx