| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F106 |
| --- | --- |

| Title: | CAVLC coding for intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a new remaining mode coding with CAVLC for luma intra prediction. In HM 3.0, the index mapping tables for intra remaining modes represent the accumulated distribution of intra remaining modes. However, those tables are not enough to reflect the content’s characteristics because they don’t consider the dependency of remaining modes on the values of MPM candidates. In the proposed method, the orders of index mapping process and remaining mode calculation process are switched. By switching them, the tables can represent the distribution of prediction modes instead of the distribution of remaining modes. The proposed change brings 0.2% BD-rate reduction for intra low complexity configurations.

# Introduction

According to the number of intra prediction modes and MPM candidates, four index mapping tables for CAVLC are defined in HM 3.0. These tables represent accumulated distribution of intra remaining modes for efficient coding by reflecting content’s characteristics. However, they are not enough to reflect content’s characteristics with the distribution of intra remaining modes, because the values of remaining mode depend on the values of MPM candidates. Therefor in this contribution, we propose a new intra remaining mode coding method so that the index mapping tables for CAVLC can reflect the distribution of intra prediction modes instead of the distribution of remaining modes.

# Proposed method

In order to make the index mapping table for intra remaining mode represent the distribution of prediction modes, the encoding/decoding process should be changed as below.

<Encoder changes>

Fig. 2-1. Encoding and Writing of intra remaining mode (a) in HM 3.0 (b) in proposed algorithm

To encode an intra luma prediction mode for CAVLC in HM 3.0, a remaining mode is obtained from prediction mode and MPM candidates, as depicted in figure 2-1(a).  Afterwards, remaining mode is mapped to a table index using an inverse index mapping table. The table index represents the rank of the remaining mode which is the same as a codeword index. The codeword index is then converted into codeword by VLC table, and finally the codeword is transmitted.

On the other hand, in the proposed method, an intra luma prediction mode is directly converted into table index by inverse index mapping table, as depicted in figure 2-1(b). In other words, table index represents the rank of the prediction mode. Afterwards, the table index is converted into a codeword index using MPM candidates. This converting process is depicted in figure 2-2. It is similar with conversion between prediction mode and remaining mode in HM 3.0.

Fig. 2-2.  Conversion of table index into codeword index

<decoder changes>

Fig. 2-3. Parsing and decoding of intra remaining mode (a) in HM 3.0 (b) in proposed algorithm

Figure 2-3 and figure 2-4 illustrate the parsing and decoding process of an intra remaining mode and the conversion between codeword index and table index in the proposed method, respectively. The parsing and decoding process depicted in figure 2-3 is a straightforward one because it is simply the inverse process of encoding.

Fig.2-4. conversion of codeword index into table index

The proposed method is able to reflect the content’s characteristics which can be considered as an improvement compared to the conventional method by having index mapping tables represent the distribution of prediction modes. Because the tables are not dependent on the number of MPM candidates, we do not have to use table according to the number of MPM candidates. We can use one unified table. At decoder side, an inverse index mapping table is required, because table indexes of MPM candidates are used in the conversion between a codeword index and a table index. Table 2-1 tabulates the number of index mapping tables needed and the number of entries for each table.

Table 2-1. The number of index mapping tables needed and the number of entries for each table

| # of intra prediction modes | HM 3.0 | Proposed |
| --- | --- | --- |
| 17 | Two index mapping tables are needed. One is with 15 entries; the other is with 16 entries. | One index mapping table and one inverse index mapping table with 17 entries are needed. |
| 34 | Two index mapping tables are needed. One is with 32 entries; the other is with 33 entries. | One index mapping table and one inverse index mapping table with 34 entries are needed. |

In addition, even for intra-coded PU which does not transmit the remaining mode, the index mapping table is updated because the table is based on the prediction mode, not on the remaining mode.

# Simulation result

The proposed method has been implemented on top of HM 3.0, and the performance was measured under LC-IN, LC-RA and LC-LD configuration. Common test condition was used.

Table 3-1. Performance in LC-IN

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | -0.1 |
| Class B | -0.2 | -0.2 | -0.2 |
| Class C | -0.2 | -0.2 | -0.2 |
| Class D | -0.1 | -0.1 | -0.1 |
| Class E | -0.2 | -0.1 | -0.1 |
| All | -0.2 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3-2. Performance in LC-RA

|  |  | Random access LoCo |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | 0.0 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 |
| Class D | -0.1 | -0.2 | -0.2 |
| Class E |  |  |  |
| All | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3-3. Performance in LC-LD

|  | Low delay LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | -0.1 | -0.4 |
| Class C | -0.1 | 0.0 | -0.2 |
| Class D | 0.0 | 0.4 | 0.2 |
| Class E | 0.0 | 0.9 | -1.0 |
| All | 0.0 | 0.2 | -0.3 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

The proposed method gives 0.2%, 0.1% and 0.0% BD bit rate reduction under LC-IN, LC-RA and LC-LD configurations. Encoding and decoding times are very similar to that of given by the anchor which suggests that the proposed method does not cause additional complexity. The detailed results for each sequence and QP are attached in the excel sheet.

# Conclusion

In this contribution, new remaining mode coding method with CAVLC for luma intra prediction is proposed. It brings 0.2% BD bit-rate reduction by reflecting the distribution of intra modes. We recommend to adopt this into HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E603, “WD3: Working Draft 3 of High-Efficiency Video Coding,” 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

Annex 1. Proposed draft

### 8.3.1 Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

–	a variable array intraModeTable

–	a variable array invIntraModeTable

Output of this process is the variable IntraPredMode[ xB ][ yB ] , value-updated intraModeTable and value-updated invIntraModeTable.

…

<text skipped>

…

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables intraPredModeN are further modified to be equal to Intra_DC when intraPredModeN is equal to Intra_Planar.

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode3[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode9[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ] and NumMPMCand is set equal to 2.

- The value of k is derived as follows:

- If intraPredModeNum is equal to 17, the variable k is set equal to 0.

- Otherwise, k is set equal to 1.

- If intraPredModeNum is not equal to 3, rankCandModeList[ x] are derived as follows:

- If NumMPMCand is equal to 1, invIntraModeTable[ k ] [ candModeList[ 0 ] ] is assigned to rankCandModeList[ 0 ].

- Oterwise, both invIntraModeTable[ k ] [ candModeList[ x ] ] are assigned to the ranks of candidate modes list with the smaller of the two rank at rankCandModeList[ 0 ] and the larger at rankCandModeList[ 1 ].

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

- If intraPredModeNum is equal to 3 or entropy_coding_mode_flag is equal to 1,

    - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

    - for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candModeList[cIdx])

IntraPredMode[ xB ][ yB ]++

- Otherwise, the following applies,

    - The variable rank_intra_luma_pred_mode is set equal to rem_intra_luma_pred_mode.

    - for (cIdx = 0 ; cIdx < NumMPMCand; cIdx++)

if (rank_intra_luma_pred_mode >= rankCandModeList[cIdx] )

rank_intra_luma_pred_mode ++

    - IntraPedMode[ xB ] [yB ] is set equal to IntraModeTable[ k ] [rank_intra_luma_pred_mode] .

- When IntraPredMode[ xB ][ yB ] is equal to Intra_DC and planar_flag_luma is equal to 1, IntraPredMode[ xB ][ yB ] is further modified to be equal to Intra_Planar.

- If intraPredModeNum is not equal to 3 and entropy coding_mode_flag is equal to 0, the following applies:

  - The value of a variable counterNum is set to 0. The variable array intraModeTable[k] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[k], rem_intra_luma_pred_mode and counterNum as inputs.

  - The variable array invIntraModeTable[ k ] is updated by invoking process in subclause 9.2.X with invIntraModeTable[k], rem_intra_luma_pred_mode as inputs.

#### 9.2.4.6 Parsing process for rem_intra_luma_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element rem_intra_luma_pred_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, a variable puSize specifying the size of the current prediction unit, NumMPMCand. and a variable array intraModeTable.

Outputs of this process are the syntax element rem_intra_luma_pred_mode. and value-updated intraModeTable.

The value of rem_intra_luma_pred_mode is derived as follows.

–	Based on the input puSize, the value of a variable intraPredModeNum indicating the number of intra prediction modes for the given size of prediction unit is obtained according to Error: Reference source not found.

–	If intraPredModeNum is equal to 3, read one bit and assign its value to rem_intra_luma_pred_mode. No further step is carried out. Othewise,

–	The values of variable k and vlcNum areis obtained as follows

  - If intraPredModeNum is equal to 17

    - If NumMPMCand is equal to 1, set vlcNum equal to 16. and k equal to 0.

    - Otherwise, set vlcNum equal to 17. and k equal to 1.

  - If intraPredModeNum is equal to 34

    - If NumMPMCand is equal to 1, set vlcNum equal to 18. and k equal to 2.

    - Otherwise, set vlcNum equal to 19. and k equal to 3.

  - The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNumrem_intra_luma_pred_mode as output.

  - The value of rem_intra_luma_pred_mode is set equal to intraModeTable[k][codeNum].

The value of a variable counterNum is set to 0. The variable array intraModeTable[k] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[k], codeNum and counterNum as inputs.

Annex 2. IntraModeTables and VLC tables

Initialisation of IntraModeTables and inverse of IntraModeTables

const UInt  g_auiIntraModeTableD17[17]={2, 3, 9, 15, 11, 7, 8, 4, 6, 0, 1, 12, 10, 16, 14, 5, 13};

const UInt  g_auiIntraModeTableE17[17]={9, 10, 0, 1, 7, 15, 8, 5, 6, 2, 12, 4, 11, 16, 14, 3, 13};

const UInt  g_auiIntraModeTableD34[34]={2, 0, 1, 30, 29, 8, 16, 21, 22, 15, 12, 9, 11, 3, 4, 7, 31, 5, 6, 32, 17, 20, 10, 19, 23, 13, 24, 28, 14, 27, 18, 26, 25, 33};

const UInt  g_auiIntraModeTableE34[34]={1, 2, 0, 13, 14, 17, 18, 15, 5, 11, 22, 12, 10, 25, 28, 9, 6, 20, 30, 23, 21, 7, 8, 24, 26, 32, 31, 29, 27, 4, 3, 16, 19, 33};

VLC Tables

const UInt huff17_2[2][17]=

{

{1, 7, 5, 13, 12, 9, 8, 6, 5, 4, 3, 2, 0, 15, 14, 3, 2},

{1, 6, 1, 14, 11, 10, 9, 8, 7, 6, 5, 4, 1, 0, 31, 30, 0},

};

const UInt lengthHuff17_2[2][17]=

{

{1, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 6, 6, 6, 6},

{1, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 6, 6, 0},

};

const UInt huff34_2[2][34]=

{

{1, 7, 0, 12, 8, 4, 27, 26, 19, 15, 13, 11, 10, 7, 5, 4, 47, 46, 45, 44, 43, 41, 40, 37, 36, 29, 25, 24, 13, 12, 85, 84, 57, 56 },

{1, 0, 4, 13, 4, 28, 24, 22, 15, 13, 11, 10, 63, 62, 60, 59, 58, 51, 47, 46, 43, 42, 41, 40, 28, 25, 24, 123, 122, 101, 100, 59, 58, 0 }

};

const UInt lengthHuff34_2[2][34]=

{

{1, 4,  4,  5,  5,  5,  6,  6,  6,  6,  6,  6,  6,  6,  6,  6,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  8,  8,  8, 8 },

{1, 3,  4,  5,  5,  6,  6,  6,  6,  6,  6,  6,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  8,  8,  8,  8,  8,  8, 0 }

};