| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F107 |
| --- | --- |

| Title: | Redundancy removal of residual information for CAVLC in merge 2Nx2N |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea | Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In current HM, skip mode and merge mode for 2Nx2N PU size (2Nx2N_MRG) share the same motion representation method. The only difference between two modes is whether the residual information is sent or not. Hence, no_residual_data_flag in CABAC coding is inferred to be equal to 0 when CU is coded with 2Nx2N_MRG mode (in S/W, not in W/D). However there is no such consideration for CAVLC coding. In a similar spirit with CABAC case, we propose a change in cbp_and_split_transform syntax. Applying this change brings 0.1% BD-rate reduction for both random access and low delay configurations.

# Introduction

In Geneva meeting, Merge skip has been adopted [1]. In Merge skip, the motion information is fully derived from the neighboring block signaled by the merge index and the residual information is not transmitted. Hence, two modes, 2Nx2N_MRG mode and skip mode, share the same motion representation method. The only difference between two modes is whether the residual information is sent or not. To avoid redundancy between two modes in HM 3.0, no_residual_data_flag syntax is is inferred to be equal to 0 instead of being transmitted when the entropy coding mode is CABAC. It is included in S/W, but it’s not included in W/D [2]. However, there is no such consideration when the entropy coding mode is CAVLC. In CAVLC, cbp_and_split_transform syntax represents the joint coding between coded block flags and split_transform_flag. We propose to resolve mismatch between S/W and W/D and also propose to make a change in cbp_and_split_transform syntax with same spirit of CABAC case.

# Proposed method

As mentioned in the above section, we propose to make a change in cbp_and_split_transform syntax for CAVLC coding so that the residual data always exists in 2Nx2N_MRG mode. For cbp_and_split_transform in the proposed method, the value with all coded block flags being 0 is temporally disabled when CU is coded with 2Nx2N_MRG and trafoDepth is zero.

<Changes in encoder side>

Tables 2-1 and 2-2 tabulate the examples of ranking table (inverse index mapping table) for cbp_and_split_transform syntax. In normal case, the value of table index is assigned as the value of codeword index. When CU is coded with 2Nx2N_MRG mode and trafoDepth is zero, the value of table index is converted to codeword index in the last column so that all-cbf-zero case is disabled. In table 2-1, the case of table index being 4 is marked as ‘NA’ when CU is coded with 2Nx2N_MRG mode and trafoDepth is zero. Hence the table indexes larger than 4 are converted into the codeword indexes reduced by 1 respectively. The same applies to table 2-2.

Table 2-1.  An example of cbp_and_split_transform ranking table (=inverse index mapping table) when flag pattern equals to 11, 13 or 15, and CU is inter-coded.

| Luma | CbCr | Split | table index (=codeword index in normal case) | codeword index in 2Nx2N_MRG with zero trafoDepth case |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 4 | NA |
| 0 | 0 | 1 | NA | NA |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 5 | 4 |
| 1 | 0 | 0 | 3 | 3 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 2 | 2 |
| 1 | 1 | 1 | 6 | 5 |

Table 2-2.  An example of cbp_and_split_transform ranking table (=inverse index mapping table) when flag pattern equals to 14, CU is inter-coded and trafoDepth is zero.

| Luma | Cb | Cr | table index (=codeword index in normal case) | codeword index in 2Nx2N_MRG |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 3 | NA |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 4 | 3 |
| 1 | 0 | 0 | 2 | 2 |
| 1 | 0 | 1 | 5 | 4 |
| 1 | 1 | 0 | 7 | 6 |
| 1 | 1 | 1 | 6 | 5 |

<changes in decoder side>

When CU is coded with 2Nx2N_MRG and trafoDepth equals to 0, let A be the value of codeword index for cbp_and_split_transform syntax, and B be the value of table index of all-cbf-zero case. If A is smaller than B, then table index for the codeword index is assigned to be A. Otherwise, if A is equal to B or bigger than B, table index is assigned to be A+1.

# Simulation result

The proposed method has been implemented on top of HM 3.0, and the performance was measured under LC-RA and LC-LD configuration. Common test condition was used.

Table 3-1. Performance for LC-RA

|  |  | Random access LoCo |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | 0.1 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 |
| Class D | -0.1 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3-2. Performance for LC-LD

|  | Low delay LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | -0.2 | -0.6 |
| Class C | -0.2 | -0.1 | -0.3 |
| Class D | -0.1 | 0.0 | -0.2 |
| Class E | -0.1 | -0.5 | -0.8 |
| All | -0.1 | -0.2 | -0.4 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

On average, the proposed method achieves 0.1% BD bit rate reduction under LC-RA and LC-LD configurations. Encoding times are 100% and 101%, and decoding times are 100% and 100%, relative to that of the anchors so that it can be concluded that the proposed method does not cause additional complexity. The detailed results for each sequence and QP are attached in the excel sheet.

# Conclusion

In this contribution, there are two recommendations. 1) One recommendation is to resolve the mismatch between s/w and working draft mentioned above, it can be resolved by inserting one conditional statement into the working draft. 2) Second recommendation is to remove the redundancy of residual information in 2Nx2N_MRG mode for CAVLC coding. It brings 0.1% BD bit-rate reduction and it enables to keep consistency between CAVLC and CABAC. It is recommended to adopt the proposed method into HM.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E481, “BoG report of CE9: MV Coding and Skip/Merge operations”, 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- JCTVC-E603, “WD3: Working Draft 3 of High-Efficiency Video Coding,” 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011.

Annex : Proposed text

Recommendation 1 :

Replace

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | u(1) ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |

with

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA && ! (PartMode == PART_2Nx2N && merge_flag[x0][y0] ) ) |  |
| no_residual_data_flag | u(1) ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |

Recommendation 2 :

Replace

9.2.4.7 Parsing process for cbp_and_split_transform

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element cbp_and_split_transform in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, a variable cbpVlcNumIdx, trafoDepth and a variable array cbpSplitTransTable.

Outputs of this process are the syntax element cbp_and_split_transform and value-updated cbpVlcNumIndex and cbpSplitTransTable.

The value of cbp_and_split_transform is derived as follows.

- Obtain a 4-bit value flagPattern accoding to subclause Error: Reference source not found.

- Derive the value of a variable k according to Error: Reference source not found.

- Set the value of a variable n as

n = (PredMode = = MODE_INTRA ? 0 : 1) 	(9-1)

- Obtain a value of vlcNum as follows

  - If flagPattern is equal to 8,

vlcNum =  (n = = 0 ? cbpVlcNumTable[cbpVlcNumIdx]:11) 	(9-2)

  - Otherwise, if flagPattern is equal to 11, 13 or 15 and n is equal to 0, set vlcNum to 15;

  - Otherwise, set vlcNum to 14.

- If vlcNum is euqal to 14, obtain the value of a variable cMax according to Table 9-1.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum and cMax as inputs and the variable codeNum as output.

- Set the value of cbp_and_split_transform equal to cbpSplitTransTable[k][n][codeNum]

- If flagPattern is not equal to 10 or 12, a variable counterNum is obtained according to Error: Reference source not found and the variable array cbpSplitTransTable[k][n] is updated by invoking process Error: Reference source not found with cbpSplitTransTable[k][n], codeNum and counterNum as inputs.

…

Table 9-1 – Derivation of cMax from flagPattern and n

|  | flagPattern |  |  |  |
| --- | --- | --- | --- | --- |
| n | 9 | 10, 12 | 11,13,15 | 14 |
| 0 | 3 | 3 | - | 7 |
| 1 | 2 | 3 | 6 | 7 |

with

9.2.4.7 Parsing process for cbp_and_split_transform

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element cbp_and_split_transform in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, a variable cbpVlcNumIdx, trafoDepth, a variable array cbpSplitTransTable and the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Outputs of this process are the syntax element cbp_and_split_transform and value-updated cbpVlcNumIndex and cbpSplitTransTable.

The value of cbp_and_split_transform is derived as follows.

- Obtain a 4-bit value flagPattern accoding to subclause Error: Reference source not found.

- Derive the value of a variable k according to Error: Reference source not found.

- Set the value of a variable n as

n = (PredMode = = MODE_INTRA ? 0 : 1) 	(9-3)

- Obtain a value of vlcNum as follows

  - If flagPattern is equal to 8,

vlcNum =  (n = = 0 ? cbpVlcNumTable[cbpVlcNumIdx]:11) 	(9-4)

  - Otherwise, if flagPattern is equal to 11, 13 or 15 and n is equal to 0, set vlcNum to 15;

  - Otherwise, set vlcNum to 14.

- If vlcNum is euqal to 14, obtain the value of a variable cMax according to Table 9-1.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum and cMax as inputs and the variable codeNum as output.

The variable 2Nx2N_MRG_ZeroTrafoDepth is derived as follow.

- If PredMode is equal to MODE_INTER, PartMode is equal to PART_2Nx2N, merge_flag[ xP ][ yP ] is equal to 1 and trafoDepth is equal to 0, set 2Nx2N_MRG_ZeroTrafoDepth to 1

- Otherwise, set 2Nx2N_MRG_ZeroTrafoDepth to 0.

- If 2Nx2N_MRG_ZeroTrafoDepth is equal to 1, the following applies.

  - The variable codeNumCbfZero is derived as follows,

codeNumCbfZero=0

While (cbpSplitTransTable[k][n][ codeNumCbfZero]!=0) codeNumCbfZero++

  - If codeNum is equal to or bigger than codeNumCbfZero,

codeNum ++

- Set the value of cbp_and_split_transform equal to cbpSplitTransTable[k][n][codeNum]

- If flagPattern is not equal to 10 or 12, a variable counterNum is obtained according to Error: Reference source not found and the variable array cbpSplitTransTable[k][n] is updated by invoking process Error: Reference source not found with cbpSplitTransTable[k][n], codeNum and counterNum as inputs.

…

Table 9-2 – Derivation of cMax from flagPattern and n

|  |  |  | flagPattern |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| n | 9 | 10, 12 | 11,13,15 (2Nx2N_MRG_ZeroTrafoDepth =0) | 15 (2Nx2N_MRG_ZeroTrafoDepth =1 | 14 (2Nx2N_MRG_ZeroTrafoDepth =0) | 14 (2Nx2N_MRG_ZeroTrafoDepth =1) |
| 0 | 3 | 3 | - | - | 7 | - |
| 1 | 2 | 3 | 6 | 5 | 7 | 6 |