| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F108 |
| --- | --- |

| Title: | Cross-verification results of MediaTek’s Luma Intra Prediction Mode Coding (JCTVC-F062) by LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for MediaTek’s luma intra prediction mode coding in JCTVC-F062.

# Observation

MediaTek provided their source codes and algorithm descriptions. We have investigated them and make sure that there is no mismatch between the source code and algorithm descriptions.

# Cross-verification results

The code was successfully compiled using Microsoft® Visual Studio® 2008. Simulations were conducted according to the common test condition of JCTVC-E700. Encoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Core™ i7 CPU 950 @3.07GHz, 12GB of RAM. Decoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU X5355 @2.66GHz, 16GB of RAM. In the simulation, -o encoder option (YUV writing) was used to confirm that a reconstructed sequence matches its decoded sequence, and -o decoder option (YUV writing) was not used. The simulation succeeded and produced as same BD BR rate as proponent provided. The encoding and decoding times also show very similar aspect. The spreadsheets which contain the detailed data of the simulation are accompanied. The computing platform for this cross-check experiment is Window XP 64 bits on Intel i7 core.

Table 2-1. Performance for HE-IN condition

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.0 |
| Class B | -0.3 | 0.0 | 0.1 |
| Class C | -0.3 | -0.1 | -0.1 |
| Class D | -0.3 | -0.2 | -0.1 |
| Class E | -0.5 | 0.0 | 0.0 |
| All | -0.4 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |