| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F109 |
| --- | --- |

| Title: | Remaining mode redundancy removal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

When parsing the syntax rem_intra_luma_pred_mode, if the syntax prev_intra_luma_pred_flag (mpm_flag) is equal to 0, the possible number of intra prediction modes is 3 (64x64), and the two MPM candidates have different intra prediction mode, the value of rem_intra_luma_pred_mode can be inferred without signaling since the possible number of intra prediction mode is three and the two MPM candidates have the two modes out of the three. However, the current HM always signals rem_intra_luma_pred_mode even though it can be inferred in that case. Therefore the current HM has a redundancy in signaling of the syntax rem_intra_luma_pred_mode. This contribution proposes to add a condition when parsing rem_intra_luma_pred_mode not to waste bits for the syntax element.

# Introduction

In the current HM, different PU sizes have different number of intra prediction modes as shown in Table 1.

Table.1 Intra prediction mode number by PU size

| PU size | intraPredModeNum |
| --- | --- |
| 4x4 | 17 |
| 8x8 | 34 |
| 16x16 | 34 |
| 32x32 | 34 |
| 64x64 | 3 |

When intraPredModeNum is 3 (64x64), the parsing process for the remaining mode is as follows.

9.2.4.6 Parsing process for rem_intra_luma_pred_mode If intraPredModeNum is equal to 3, read one bit and assign its value to rem_intra_luma_pred_mode.

However, when the two MPM candidates have different intra prediction modes, the remaining mode can be inferred without parsing rem_intra_luma_pred_mode since the possible number of intra prediction modes is three. Therefore the current HM has a redundancy in signaling of rem_intra_luma_pred_mode when the PU size is 64x64 and the two MPM candidates have different intra prediction modes.

# Proposed Methods

In the proposed method, the syntax rem_intra_luma_pred_mode is not signaled when all of the following conditions are satisfied.

- The variable intraPredModeNum is equal to 3

- The syntax element prev_intra_luma_pred_flag is equal to 0

- The variable NumMPMCand is equal to 2

, where intraPredModeNum indicates the allowed number of intra prediction modes for the current PU, and NumMPMCand indicates the number of MPM candidates.

When the conditions are satisfied, the syntax rem_intra_luma_pred_mode can be inferred without signaling it. Therefore the condition for parsing rem_intra_luma_pred_mode is modified as shown in Table 2.

Table.2 Prediction unit syntax table for the proposed method

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| Else |  |
| if( NumRemCand > 1 ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| ︙ |  |
| } |  |
| } |  |

In Table 2, the variable NumRemCand is derived as follows.

- NumRemCand = intraPredModeNum - NumMPMCand

For the proposed method, the yellow marked sentence is added in the semantic of the following syntaxes.

prev_intra_luma_pred_flag[ x0 ][ y0 ], mpm_idx[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When prev_intra_luma_pred_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause Error: Reference source not found. When rem_intra_luma_pred_mode[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

# Experimental Results

The proposed method was implemented on the HM 3.0 software and the test condition of [1] is followed. Table 1 shows the results for the proposed method. The performance is almost same as the anchor since 64x64 PU is not frequently selected.

Table.3 BD rate (%) reduction of the proposed method

|  | Intra only HE |  |  | Intra only LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This contribution points out the redundancy in signaling of the remaining mode for intra prediction and proposes a way to resolve it. A condition is just added when parsing rem_intra_luma_pred_mode not to waste the bit for the syntax element. Therefore it is recommended to adopt the proposed method into HM to remove the redundancy.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.