| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F111 |
| --- | --- |

| Title: | Intra prediction mode coding with CAVLC on SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents a CAVLC joint coding scheme for intra prediction mode on SDIP. When SDIP is enabled, split mode and four kinds of partitioning type exist for intra prediction. Similar CAVLC joint coding scheme and counters as used for inter mode coding was also applied to intra mode coding. The simulation result shows that the proposed method achieves 0.1% bit rate reduction for INLC configuration without any encoder and decoder complexity changes.

# Introduction

When SDIP is enabled, intra prediction modes currently include split flag, 2Nx2N, NxN, 2NxhN and hNx2N. According to the current SDIP anchor software, intra mode information is sent with two types by the size of CU as shown is Figure.1.

If the current CU has larger size than the minimum CU size, firstly the split flag is sent. If split flag is 1, no more mode information is sent; otherwise, a flag which indicates whether SDIP partition is used is sent; if the SDIP flag is 0, the partition type is determined to 2Nx2N and no more information is sent; otherwise a flag which indicates which partition type is used between 2NxhN and hNx2N.

If the current CU has the minimum CU size, firstly partition type flag is sent, If this flag is 0, the partition type is determined to 2Nx2N and no more information is sent; otherwise the SDIP flag is sent; if the SDIP flag is 0, the partition type is determined to Nx2N and no more information is sent; otherwise a flag which indicates which partition type is used between 2NxhN and hNx2N.

Therefore, regardless of the CU size, 3 flags are transmitted for signaling intra mode coding in SDIP. In this contribution, similar CAVLC joint coding scheme and counters as used for inter mode coding was also applied to intra mode coding.

Figure.1 intra mode signaling in SDIP

# Proposed Methods

According to the proposal, split flag and SDIP flag, SDIP direction flag are grouped and coded together with a symbol intra_cu_split_pred_part_mode. Unary codes are used and mapping between symbol and codeword index is adaptive. The adaptation method is the same as used for inter mode coding. Such adaptation is applied separately at each CU depth. SDIP is enabled for 32x32, 16x16 and 8x8 CU sizes, therefore corresponding three VLC tables are used for intra mode coding.

Initial VLC tables are as shown in Table 1 and these tables are initialized at each slice. If SDIP mode is selected from the VLC table, additional flag is transmitted so that the partition type is determined between 2NxhN and hNx2N.

Table.1 VLC table for intra mode coding

| index | mode | codeword |  | index | mode | codeword |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | SPLIT | 1 |  | 0 | 2Nx2N | 1 |
| 1 | 2Nx2N | 01 |  | 1 | NxN | 01 |
| 2 | 2NxhN | 00 |  | 2 | 2NxhN | 00 |
|  | hNx2N |  |  |  | hNx2N |  |

(a) CU size > min                        (b) CU size = min

# Experimental Results

The proposed method was implemented on the HM3.0-SDIP software (rev.866) and the performance was measured under Low Complexity intra only configuration. Common test condition is used for measuring the performance of the proposed method besides the SDIP option.

Table.2 shows the experimental results in terms of BD rate reduction. The average gains for INLC configuration is 0.1%. There are no noticeable encoder and decoder complexity changes.

Table.2 BD rate (%) reduction of the proposed method

|  | Intra only LC |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | -0.2 | -0.2 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | -0.2 | -0.4 | -0.4 |
| All | -0.1 | -0.1 | -0.2 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

# Proposed change to WD text

7.3.5	Coding tree syntax

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress ) { |  |
| if( !entropy_coding_mode_flag ){ |  |
| if( slice_type == I ) |  |
| intra_cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| else |  |
| cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| } |  |
| else if( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| ︙ |  |
| } |  |

intra_cu_split_pred_part_mode[ x0 ][ y0 ] specifies split_coding_unit_flag and when the coding unit is not split PartMode of a coding unit in I slice. The array indices x0 and y0 specify the location ( x0, y0 ) of the top-left luma sample of the coding unit relative to the top-left luma sample of the picture.

Table 7-1 – Specification of intra_cu_split_pred_part_mode when log2CUSize is greater than Log2MinCUSize

| - intra_cu_split_pred_part_mode | - split_coding_unit_flag | - SDIP_flag | - PredMode | intra_part_mode |
| --- | --- | --- | --- | --- |
| - 0 | - 1 | - 0 | - - | - - |
| - 1 | - 0 | - 0 | - MODE_INTRA | PART_2Nx2N |
| - 2 | - 0 | - 1 | - MODE_INTRA | PART_2NxhN |
|  |  |  | - MODE_INTRA | PART_hNx2N |

Table 7-2 – Specification of intra_cu_split_pred_part_mode when log2CUSize is equal to Log2MinCUSize

| - intra_cu_split_pred_part_mode | - split_coding_unit_flag | - SDIP_flag | - PredMode | intra_part_mode |
| --- | --- | --- | --- | --- |
| - 0 | - 0 | - 0 | - MODE_INTRA | - PART_2Nx2N |
| - 1 | - 0 | - 0 | - MODE_INTRA | PART_NxN |
| - 2 | - 0 | - 1 | - MODE_INTRA | PART_2NxhN |
|  |  |  | - MODE_INTRA | PART_hNx2N |

7.3.6	Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( entropy_coding_mode_flag && ( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) ) |  |
| pred_type | u(v) ¦ ae(v) |
| if(intra_SDIP_enabled_flag && PredMode == MODE_INTRA && log2CUSize < 6 && (PartMode == PART_NxN ¦ ¦ log2CUSize > Log2MinCUSize) ) { |  |
| if ( entropy_coding_mode_flag ) |  |
| SDIP_flag | u(1) ¦ ae(v) |
| if( SDIP_flag ) { |  |
| if( log2CUSize == 4 ) { |  |
| SDIP_TU_split[0] | u(1) ¦ ae(v) |
| SDIP_TU_split[1] | u(1) ¦ ae(v) |
| SDIP_TU_split[2] | u(1) ¦ ae(v) |
| SDIP_TU_split[3] | u(1) ¦ ae(v) |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| SDIP_direction | u(1) ¦ ae(v) |
| } |  |
| } |  |
| ︙ |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

9.2.4.6	Parsing process for intra_cu_split_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element intra_cu_split_pred_part_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, splitPredPartModeTable.

Output of this process is the syntax element intra_cu_split_pred_part_mode.

The value of intra_cu_split_pred_part_mode is derived as follows.

- If log2CUSize is greater than Log2MinCUSize, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 2 as inputs and the variable codeNum as output.

- Otherwise, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 5 as inputs and the variable codeNum as output.

- The syntax element intra_cu_split_pred_part_mode is derived as

intra_cu_split_pred_part_mode = splitPredPartModeTable[Log2MaxCUSize – log2CUSize][codeNum]

- If intra_cu_split_part_mode has a value of 2, PartMode is determined as follows

- Read one bit and assign its value to a variable b

- If b is equal to 1, set PartMode to PART_2NxhN

- Otherwise, set PartMode to PART_hNx2N.

The variable array splitPredPartModeTable[Log2MaxCUSize – log2CUSize] is updated by invoking process Error: Reference source not found with splitPredPartModeTable[Log2MaxCUSize – log2CUSize], codeNum and a value of 3 as inputs.

# Conclusion

In this document, intra prediction mode coding with CAVLC was proposed. The proposed scheme shows 0.1% bit-rate reduction without causing any encoder or decoder complexity changes. It is recommended to adopt the proposed method into HM.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).