| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F112 |
| --- | --- |

| Title: | CE1 : Results on a selection of the representative motion data (A.3, A.7, A.9 and A.11) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Park Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | +82 10 3922 5896 seungwook.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution reported the results of A.3, A.7, A.9 and A.11, which represents a various combination of two positions; one for a motion data storage compression and the other for a temporal motion data. It is reported that A.3 provides 0.1%, 0.2%, 0.1% and 0.2%, A.7 provides 0.4%, 0.6%, 0.5% and 0.6%, A.9 provides 0.3%, 0.5%, 0.3% and 0.5%, and A.11 provides 0.3%, 0.5%, 0.3% and 0.5% for HE-RA, HE-LD, LC-RA and LC-LD respectively.

# Introduction & proposed tests

In the last meeting, there are some contributions regarding a change of a representative position for motion data storage reduction and for a temporal motion vector predictor [1]. They have shown some promising coding gain (about 1%) by a simple change of a representative block position, but the decision was deferred due to their interaction with technologies regarding MV coding and skip/merge operation.

In this proposal, we have tested 4 kinds of combination; A.3, A.7, A.9 and A.11 (refer to table 1). And representative positions for MDSR and TMVP are depicted in Figure 1 and 2.

Figure 1: Description of the positions for MDSR.

Figure 2: Description of the positions for TMVP.

Table 1. Test configurations

| Experiment | MVSR (position) | Prediction (TMVP position) | Cross-check |
| --- | --- | --- | --- |
| A.3 | C3 | HM3.0 | JCTVC-F327 (Toshiba) |
| A.7 | C3 | C0 | JCTVC-FXXX (I2R) |
| A.9 | BR | C3 | JCTVC-F081 (TI) |
| A.11 | C3 | C3 | JCTVC-F266 (Orange Lab) |

# Experimental results

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [2] are followed. The detail results can be founded in attached spreadsheets.

Table 2. Overall BD-rate reduction of A.3, A.7, A.9 and A.11

| Experiment | AVG | HE-RA | HE-LD | LC-RA | LC-LD |
| --- | --- | --- | --- | --- | --- |
| A.3 | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 |
| A.7 | 0.5 | 0.4 | 0.6 | 0.5 | 0.6 |
| A.9 | 0.4 | 0.3 | 0.5 | 0.3 | 0.5 |
| A.11 | 0.4 | 0.3 | 0.5 | 0.3 | 0.5 |

# Conclusion

All test points have provided a slight coding loss about 0.1% ~ 0.5% compared with HM3.0. Main reason of this coding loss could be not using H position for TMVP. Despite the coding loss, a case of A.11 provides a unified position between MDSR and TMVP. It could be beneficial to calculate the position of a representative block for MDSR and TMVP (refer to Annex). So we recommend to continue CE to achieve a unified and simplified solution with no coding loss.

# Patent rights declaration(s)

LG Electronics  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- S. Park, J. Park and B. Jeon, “Modifications of temporal mv memory compression and temporal mv predictor,” JCT-VC Document, JCTVC-E059, Geneva, CH, 16-23 March, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

##### Annex – draft text

For A.3 (C3, HM3.0) combination,

Replace

- 8.4.2.1.6 Derivation process for temporal luma motion vector prediction

…

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-1)

yPRb = yP + nPSH	(8-2)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-3)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-4)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

with

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW 	(8-5)

yPRb = yP + nPSH 	(8-6)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( ( xPRb >> 4 ) << 4 ) + 8, ( ( yPRb >> 4 ) << 4 ) + 8) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = xP + ( nPSW >> 1 ) - 1	(8-7)

yPCtr = yP + ( nPSH >> 1 ) - 1	(8-8)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( ( xPCtr >> 4 ) << 4 ) + 8, ( ( yPCtr >> 4 ) << 4 ) + 8) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

For A.7 (C3, C0) combination,

Replace

- 8.4.2.1.6 Derivation process for temporal luma motion vector prediction

…

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW 	(8-9)

yPRb = yP + nPSH	(8-10)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-11)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-12)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

with

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + ( nPSW >> 1 ) - 1	(8-13)

yPRb = yP + ( nPSH >> 1 ) - 1	(8-148)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( ( xPRb >> 4 ) << 4 ) + 8, ( ( yPRb >> 4 ) << 4 ) + 8) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

For A.9 (BR, C3) combination,

Replace

- 8.4.2.1.6 Derivation process for temporal luma motion vector prediction

…

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-15)

yPRb = yP + nPSH	(8-16)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-17)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-18)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

with

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = ( xP + ( nPSW >> 1 ) 	(8-19)

yPRb = ( yP + ( nPSH >> 1 ) 	(8-207)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( ( xPRb >> 4 ) << 4 ) + 15, ( ( yPRb >> 4 ) << 4 ) + 15) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

For A.11 (C3, C3) combination,

Replace

- 8.4.2.1.6 Derivation process for temporal luma motion vector prediction

…

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-21)

yPRb = yP + nPSH	(8-22)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-23)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-24)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

with

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom center luma position of the current prediction unit is defined by

xPRb = xP + (nPSW >> 1 )	(8-25)

yPRb = yP + (nPSH >> 1)	(8-26)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4 + 8, ( yPRb >> 4 ) << 4  + 8) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.