#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength are initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-428)

    - bStrength[1][k][m] is set equal to bSVer.

    - Decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m], an array dS of size (8) and a variable d as outputs.

    - dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7, and diff[1][k][m] is set equal to d.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-428)

    - bStrength[0][k][m] is set equal to bSHor.

    - Decision process for luma block edge in subclause 8.6.1.4.1 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m], an array dS of size (8) and a variable d as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7, and diff[0][k][m] is set equal to d.

- For xDk set equal to xC+( k << 3 ), k=0..nD - 1, the following applies:

- For yDm set equal to yB+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - dS[i] is set equal to dSample[1][k][(m<<3)+i] for i=0..7.

    - Filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, the boundary filtering strength bStrength[1][k][m], the decision dEdge[1][k][m], and the array of size (8), dS as inputs and the modified luma picture buffer as outputs.

- For yDm set equal to yC+( m << 3 ), m=0..nD - 1, the following applies:

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD – 1) << 3 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

    - dS[i] is set equal to dSample[0][m][ (k << 3) + i ] for i = 0..7.

    - Filtering process for luma block edge in subclause 8.6.1.4.2 is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bStrength[0][k][m], the decision dEdge[0][k][m], and the array of size (8), dS, xPOS, dSL[m][], dEL[m], bSL[m], and tCL[m], as inputs and the modified luma picture buffer as output.

- The elements of the two dimensional array of size (3)x(nD), dSL are set as follows. dSL[m][0], dSL[m][1], and dSL[m][2], are set equal to dS[5], dS[6] and dS[7].

- The elements of the array of size (nD), dEL are set as follows. dEL[m] is set equal to dEdge[0][k][m].

- The elements of the array of size (nD), bSL are set as follows. bSL[m] is set equal to bStrength[0][k][m].

- The elements of the array of size (nD), tC are set as follows. tCL[m] is set equal to tc.

The filtering process for chroma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( Max( log2CUSize, 4 ) – 4 ).

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD – 1, the following applies:

- For yDm set equal to ( yC / 2)+( m << 2 ), m=0..nD – 1, the following ordered steps apply:

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ] 		(8-430)

- Filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1, the boundary filtering strength bSVer and the luma sample difference diff[1][k][m] as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSVer and the luma sample difference diff[1][k][m] as inputs and the modified chroma picture buffer as output.

- For yDm set equal to ( yC / 2 )+( m << 2 ), m=0..nD – 1, the following applies:

- For xDk set equal to ( xC / 2 )+( k << 2 ), k=0..nD – 1, the following ordered steps apply:

- If xDk is equal to 0, the parameter xPOS is set equal to 1. If xDk is equal to xB+( ( nD*2 – 1) << 2 ) xPOS is set equal to 2. Otherwise xPOS is set to 0.

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ] 		(8-431)

- Filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] and the luma sample difference diff[0][k][m]  as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause 8.6.1.4.3 is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2 and the boundary filtering strength bSHor, xPOS, bSL[m] and tCL[m] and the luma sample difference diff[1][k][m]  as inputs and the modified chroma picture buffer as output.

##### Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable dE containing a decision,

–	one-dimensional array of size (8), dS containing decisions,

–	a variable bSL,

–	a variable tCL,

Output of this process is:

–	modified reconstruction of the picture.

Let s’ represent the luma sample array recPictureL of the current picture.

A variables β is specified as Table 8-11 with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Table 8-11 with luma quantization parameter ( qPL + 4 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Table 8-11 with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-432)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-433)

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, the decision dS[k], the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’		(8-435)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’		(8-436)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -3 and 4 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 4 respectively. Otherwise ks and ke are set to 0 and 7 respectively.

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-437)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-438)

- If xPOS is less than 0 and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[k+3], the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’		(8-459)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-460)

- If dE is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke , the following ordered steps apply:

- The filtering process for a luma sample specified in subclause 8.6.1.4.5 is invoked with sample values pi,k, qi,k with i = 0..3, decision dS[k], the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

- The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’		(8-461)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-462)

##### Decision process for chroma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	a variable dE containing a decision,

–	one-dimensional array of size (8), dS containing decisions.

Let s’ represent the chroma sample array recPictureL of the current picture.

A variables β is specified as Table 8-13 with chroma quantization parameter qPC as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Table 8-13 with chroma quantization parameter ( qPC + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Table 8-13 with chroma quantization parameter qPC as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..1 and k = 1,2 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-447)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-448)

- The variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 |	(8-449)

- The variable dE is set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause 8.6.1.4.4 is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam

- The variable dE is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-435)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-436)

- The variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 |	(8-437)

- The variable dE is set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause 8.6.1.4.4 is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

##### Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variables β is derived as follows:

–	If current slice is an intra coded slice, β is specified as Table 8-13 with luma quantization parameter qPL as input.

–	otherwise, β is set equal to 2* qPL

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Table 8-13 with luma quantization parameter ( qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Table 8-13 with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-442)

pi = s’[ xC + xB – i – 1, yC + yB + k ]		(8-443)

- If bS is greater than 1 and d is less than β, the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-444)

s’[ xC + xB – 1, yC + yB + k ] = p0’		(8-445)

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -1 and 2 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 2 respectively. Otherwise ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-446)

pi = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-447)

- If xPOS is less than 0, and if bSL, is greater than 1 and d is less than β, for each sample location ( xC + xB - 1, yC + yB ), the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSL and the variable tCL as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-452)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-445)

- If bS is greater than 1 and d is less than β, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke, the following ordered steps apply:

- The filtering process for a sample specified in subclause 8.6.1.4.6 is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-448)

s’[ xC + xB +k, yC + yB – 1 ] = p0’		(8-449)

##### Decision process for a luma sample

[Ed: (WJ) no filtering when bS is equal to 0]

Inputs of this process are:

–	sample values, pi and qi with i = 0..2,

–	a variable bS specifying the boundary filtering strength,

–	variables d, β and tC.

Output of this process is:

–	a variable dSam containing a decision

When the variable bS is not equal to 0, the following applies:

–	If d is less than ( β >> 2 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, dSam is set equal to 1.

–	Otherwise, dSam is set equal to 0.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dSam containing a decision,

–	a variable tC.

Output of this process is:

–	number of filtered samples nD,

–	filtered sample values, pi’ and qi’ with i = 0..nD – 1,

Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nD is set equal to 3:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-450)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-451)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-452)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-453)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-454)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-455)

–	Otherwise, the following weak filtering applies while nD is set equal to 2:

= Clip3( -tC, tC, ( 13*( q0 – p0 ) + 4*( q1 – p1 ) - 5*( q2 – p0 ) + 16 ) >> 5 )	(8-456)

p0’ = Clip1Y( p0 +  )		(8-457)

q0’ = Clip1Y( q0 -  )		(8-458)

p1’ = Clip1Y( p1 + /2 )		(8-459)

q1’ = Clip1Y( q1 – /2 )		(8-460)

Table 8-11 – Derivation of threshold variables β and tC from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

##### Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable bS specifying the boundary filtering strength.

–	variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

When the variable bS is greater than 1, the filtered sample values p0’ and q0’ are derived by

= Clip3( -tC, tC, ( ( 12 * ( q0 – p0 )   +  3 *(p1 – q1) + 16 ) >> 5 ) )	(8-461)

p0’ = Clip1C( p0 +  )		(8-462)

q0’ = Clip1C( q0 -  )		(8-463)