| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F121 |
| --- | --- |

| Title: | Intra Chroma LM Mode with Reduced Line Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Mei Guo, *Xun Guo, #Yu-Wen, Huang, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | mei.guo@mediatek.com xun.guo@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM3.0, when a chroma prediction unit (PU) is coded with intra LM mode, two lines of reconstructed pixels above the corresponding luma PU are used to derive the correlation parameters between the luma component and chroma component. When the current luma PU is located beneath the upper boundary of the largest coding unit (LCU), two bottom lines of reconstructed luma pixels in the upper LCU are required. Therefore, two line buffers are needed in hardware implement. This contribution proposes to use only one line from the upper LCU in LM mode for PUs located beneath the upper LCU boundary, and keep the current method for PUs in other positions. Using this method, no additional line buffer is needed for intra LM mode since one line buffer is also needed for luma intra prediction. It is reported that there is almost no change of BD-rate and running time with this method.

# Introduction

In HM3.0, the chroma component has six intra prediction modes: DM mode which is the best intra prediction mode of the corresponding luma PU, LM mode which achieves the prediction of chroma PU from the reconstructed pixels of corresponding luma PU [1], vertical mode, horizontal mode, DC mode and planar mode. Among them, LM mode generates the prediction of chroma pixels using a linear equation as follows.

,                         (1)

where indicates the prediction of chroma pixels and indicates the reconstructed pixels in corresponding luma PU. Parameters  and are derived from causal reconstructed pixels around the luma and chroma PUs.

In YUV420 sampling, the sampling ratio of chroma components is half of that of luma component both horizontally and vertically, and has 0.5 pixel phase difference in vertical direction. Thus, for chroma LM mode, the reconstructed luma pixels are down-sampled in vertical direction and sub-sampled in horizontal direction to match the size and phase of chroma component, as follows.

(2)

After that, the linear least square solution between causal reconstructed data of down-sampled luma pixels and causal chroma pixels is used to derive the model parameters  and  using the following equations:

and            (3)

,                             (4)

whereand indicate reconstructed chroma pixels and down-sampled luma pixels around the target PU, respectively; and indicates the total number of samples involved in the calculation of the parameters. As shown in Figure 1, the left and above causal pixels marked as gray circles are involved in the calculation to keep the total number as power of 2. For a target N×N chroma PU, when both left and above causal pixels are available, the number of involved pixels is 2N. When only left or above causal pixels are available, the number of involved pixels is N.

Figure 1 Locations of the samples used for the derivation of α and β

In LM mode, two lines of reconstructed luma pixels above the current PU, if they are available, are required to calculate the parameters α and β. If the current block is just located beneath the upper boundary of the LCU, two bottom lines of the upper LCU are needed to be stored in the buffer. However, all the other intra prediction modes in HM3.0 only need to store one bottom line from the upper LCU. The increased line buffer will take additional cost for the on-chip memory (SRAM) in hardware implementation. In order to remove this additional line buffer, the proposed method is described in the following section.

# Algorithm Description

We propose to use only one reconstructed (bottom) line from the upper LCU in the derivation of α and β for the PUs located beneath the LCU boundaries. That is, in vertical direction, no down-sampling filter is applied to the reconstructed luma lines from the upper LCU. And in horizontal direction, sub-sampling method is still used. In order to reduce the noises in luma component, a simple horizontal (1/4, 2/4, 1/4) filter in Equation (5) can be applied to the luma line of the upper LCU before sub-sampling in horizontal direction.

RecL[2x,2y+1]=(RecL[2x−1,2y+1]+2∗RecL[2x,2y+1]+RecL[2x+1,2y+1]+2)>>2(5)

Figure 2 shows the luma pixels used in LM mode for the PUs beneath the LCU boundary. Only one line from the upper LCU is utilized in the derivation of α and β. The down-sampling of above casual pixels is shown as Equation (6).

Re {cL'[x,y]=RecL[2x,2y+1]                              (6)

Figure 2 Locations of the pixels used for the derivation of α and β beneath LCU boundary.

Note that the proposed algorithm is only applied to the PUs located beneath the upper LCU boundaries. For other PUs, the same method is used as that in HM3.0.

# Simulation results

Our simulations are based on HM3.0 and the recommended test conditions in JCTVC-E700 [2]. We tested our method both without and with horizontal smooth filtering. Table 1 and Table 2 gives the coding performance and running time of the proposed method without and with horizontal smooth filtering, respectively. On average, the coding performance and running time are quite similar to that of anchor.

Table 1. Experimental results of the proposed method without horizontal smooth filtering

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.03 | 0.69 | 0.27 | 0.02 | 0.75 | 0.37 |
| Class B | 0.01 | 0.15 | 0.07 | 0.01 | 0.08 | 0.04 |
| Class C | 0.02 | 0.13 | 0.15 | 0.02 | 0.10 | 0.12 |
| Class D | 0.01 | 0.06 | 0.02 | 0.00 | 0.06 | 0.05 |
| Class E | -0.01 | -0.05 | 0.09 | 0.00 | 0.02 | -0.01 |
| All | 0.01 | 0.20 | 0.12 | 0.01 | 0.20 | 0.12 |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 2. Experimental results of the proposed method with horizontal smooth filtering

|  | HE-AI |  |  | LC-AI |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.03 | 0.31 | 0.21 | 0.02 | 0.36 | 0.08 |
| Class B | 0.01 | 0.08 | 0.04 | 0.01 | 0.03 | 0.04 |
| Class C | 0.02 | 0.01 | 0.05 | 0.01 | 0.02 | -0.01 |
| Class D | 0.02 | -0.02 | 0.02 | 0.00 | 0.05 | 0.02 |
| Class E | -0.01 | -0.08 | 0.02 | 0.00 | 0.00 | -0.02 |
| All | 0.01 | 0.07 | 0.07 | 0.01 | 0.09 | 0.02 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Conclusion

This contribution proposed a method to reduce the line buffer for intra chroma prediction with reconstructed luma pixels (intra chroma LM mode). For the PUs located beneath the upper LCU boundaries, only one reconstructed luma line from the upper LCU is used to generate the prediction for chroma PUs. By using this method, no additional luma line buffer is needed for chroma intra prediction since the same one line buffer is needed for luma intra prediction. Experimental results show that there is almost no change for both coding performance and running time.

# References

- J. Chen, V. Seregin, W.-J. Han(Samsung), J. Kim, J. Moon(LGE), “CE6.a.4:Chroma intra prediction by reconstructed luma samples”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E266, 5th Meeting: Geneva, Switzerland, 16-23 March, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, Switzerland, 16-23 March, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).