Proposed Specification for Working Draft

#### Intra sample prediction

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable intraPredMode specifying the luma intra prediction mode,

–	a variable nS specifying the prediction size.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is:

–	the predicted samples predSamples[ x, y ], with x, y =0..nS-1.

The nS*4+1 neighbouring samples p[ x, y ] that are constructed luma samples prior to the deblocking filter process, with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y=-1, are derived as follows.

–	The luma location ( xBN, yBN ) is specified by

xBN = xB + x 		(8-14)

yBN = yB +y 		(8-15)

–	The derivation process for neighbouring locations in subclause XXX is invoked for sample location with ( xBN, yBN ) as input and the treeblock address tbAddrN as output.

–	Each sample p[ x, y ] with x = -1, y= -1..nS*2-1  and x = 0, y = -1 is derived as follows

- If any of the following condition is true, the sample p[ x, y ] is marked as “not available for intra prediction”

  - tbAddrN is not available

  - the coding unit covering ( xBN, yBN ) is not coded as intra mode and constrained_intra_pred_flag is equal to 1

- Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the sample at the location ( xBN, yBN ) inside the treeblock tbAddrN is assigned to p[ x, y ]

When at least one sample p[ x, y ] with x = 1, y = 1..nS*2 1 and x = 0..nS*2 1, y = 1 is marked as “not available for intra prediction,” the reference sample substitution process for intra sample prediction in subclause 8.3.3.1.1 is invoked with the samples p[ x, y ] with x = 1, y = 1..nS*2 1 and x = 0..nS*2 1, y = 1 as input and the modified samples p[ x, y ] with x = 1, y = 1..nS*2 1 and x = 0..nS*2 1, y = 1 as output.

Table 8-4 specifies the value for intraPredMode and the associated names.

Table 8-4 – Specification of intraPredMode and associated names

| intraPredMode | Name of intraPredMode |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33) | Intra_Angular |

Depending on intraPredMode, the following ordered steps apply:

- When cIdx is equal to 0, filtering process of neighbouring samples specified in 8.3.3.1.2 is invoked with the sample array p and the prediction size nS as the inputs and the output is a (2*nS+1)x(2*nS+1) filtered sample array pF.

- One of the intra prediction modes specified in subclause 8.3.3.1.2 to 8.3.3.1.6 is invoked with the sample array pF and the prediction size nS as the inputs and the output is the predicted sample array predSamples.

##### Reference sample substitution process for intra sample prediction

Inputs to this process are the reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

The values of the samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are modified as follows:

- If all samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “not available for intra prediction,” the value ( 1 << ( BitDepthY - 1 ) ) is substituted for the values of all samples p[ x, y ].

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

- Let the variables q and r be initially set to -1.

- The variable q is modified as follows:

  - If x0 is equal to -1, a reference sample p[ x, y ] marked as “available for intra prediction” is identified by searching sequentially among p[ x, y ] starting from x = -1, y = y0+1 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0-1, y = -1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = x0-1..-1, y = -1, the search is continued among p[ x, y ] sequentially starting from x = -1, y = 0 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

- The variable r is modified as follows:

  - If x0 is equal to -1, a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = -1, y = y0-1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = -1, y = y0-1..-1, the search is continued among p[ x, y ] sequentially starting from x = 0, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0+1, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

- The value of the sample p[ x0, y0 ] is modified as follows:

  - If q is not equal to -1 and r is not equal to -1, the value ( ( q + r + 1 ) >> 1 ) is substituted for the value of p[ x0, y0 ].

  - Otherwise, if q is not equal to -1, the value of q is substituted for the value of p[ x0, y0 ].

  - Otherwise (q is equal to -1 and r is not equal to -1), the value of r is substituted for the value of p[ x0, y0 ].

All samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “available for intra prediction.”

##### Filtering process of neighbouring samples

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	filtered samples pF[ x, y ],. with x, y = -1..2*nS-1.

Table 8-5 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 2 | 2 | 2 |
| 4 | 0 | 1 | 2 | 2 | 2 |
| 5 | 0 | 2 | 2 | 2 | 0 |
| 6 | 1 | 2 | 2 | 2 | 0 |
| 7 | 0 | 2 | 2 | 2 | 0 |
| 8 | 0 | 1 | 2 | 2 | 0 |
| 9 | 1 | 1 | 2 | 2 | 0 |
| 10 | 0 | 0 | 2 | 2 | 0 |
| 11 | 0 | 0 | 2 | 2 | 0 |
| 12 | 0 | 0 | 2 | 2 | 0 |
| 13 | 0 | 0 | 2 | 2 | 0 |
| 14 | 0 | 0 | 2 | 2 | 0 |
| 15 | 0 | 0 | 2 | 2 | 0 |
| 16 | 0 | 0 | 2 | 2 | 0 |
| 17 | 0 | 0 | 2 | 2 | 0 |
| 18-33 | 0 | 0 | 0 | 2 | 0 |

Filtered sample array pF[ x, y ] with x = -1..nS*2-1 and y = -1..nS*2-1  are derived as follows:

–	If intraFilterType[ nS ][ IntraPredMode ] is equal to 1, the following applies:

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]		(8-17)

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]		(8-18)

pF[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-19)

pF[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2	(8-20)

pF[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-21)

–	Otherwise, intraFilterType[ nS ][ IntraPredMode ] is equal to 2, the following applies:

pF[ -1, nS*2-1 ] = p[ -1, nS*2-1 ]		(8-22)

pF[ nS*2-1, -1 ] = p[ nS*2-1, -1 ]		(8-23)

pF1[ -1, y ] = ( p[ -1, y+1 ] + 2*p[ -1, y ] + p[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-24)

pF1[ -1, -1] = ( p[ -1, 0 ] + 2*p[ -1, -1] + p[ 0, -1 ] + 2) >> 2	(8-25)

pF1[ x, -1 ] = ( p[ x-1, -1 ] + 2*p[ x, -1 ] + p[ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-26)

pF[ -1, y ] = ( pF1[ -1, y+1 ] + 2*pF1[ -1, y ] + pF1[ -1, y-1 ] + 2 ) >> 2 for y = nS*2-2..0	(8-27)

pF[ -1, -1] = ( pF1 [ -1, 0 ] + 2*pF1 [ -1, -1] + pF1 [ 0, -1 ] + 2) >> 2	(8-28)

pF[ x, -1 ] = ( pF1 [ x-1, -1 ] + 2* pF1 [ x, -1 ] + pF1 [ x+1, -1 ] + 2 ) >> 2 for x = 0..nS*2-2	(8-29)

##### Specification of Intra_Vertical prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 0.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

predSamples[ x, y ] = p[ x, -1 ], with x, y = 0..nS-1	(8-30)

–	If nS is equal to or larger than 32

- for(y=0; y<nS;y++)

gradientVal = p[-1,y]-p[-1,-1]

predSamples[0,y] += (gradientVal + 1)>>1

predSamples[1,y] += (gradientVal + 2)>>2

predSamples[2,y] += (gradientVal + 4)>>3

predSamples[3,y] += (gradientVal + 8)>>4

predSamples[4,y] += (gradientVal + 8)>>4

–	Otherwise,

- for(y=0; y<nS;y++)

gradientVal = p[-1,y]-p[-1,-1]

predSamples[0,y] += (gradientVal + 1)>>1

predSamples[1,y] += (gradientVal + 2)>>2

predSamples[2,y] += (gradientVal + 4)>>3

##### Specification of Intra_Horizontal prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

predSamples[ x, y ] = p[ -1, y ], with x, y = 0..nS-1	(8-31)

–	If nS is equal to or larger than 32

- for(x=0; x<nS;x++)

gradientVal = p[x,-1]-p[-1,-1]

predSamples[x,0] += (gradientVal + 1)>>1

predSamples[x,1] += (gradientVal + 2)>>2

predSamples[x,2] += (gradientVal + 4)>>3

predSamples[x,3] += (gradientVal + 8)>>4

predSamples[x,4] += (gradientVal + 8)>>4

–	Otherwise,

- for(x=0; x<nS;x++)

gradientVal = p[x,-1]-p[-1,-1]

predSamples[x,0] += (gradientVal + 1)>>1

predSamples[x,1] += (gradientVal + 2)>>2

predSamples[x,2] += (gradientVal + 4)>>3

##### Specification of Intra_DC prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 2.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as follows.

–	If all samples p[ x, -1 ], with x=0..nS-1 and p[ -1, y ], with y=0..nS-1 are marked as “available for intra prediction,” the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by

predSamples[ x, y ] = , with x, y = 0..nS-1	(8-32)
where k=log2(nS)

–	Otherwise, If all samples p[ x, -1 ], with x=0..nS-1 are marked as “not available for intra prediction” and all samples p[ -1, y ], with y=0..nS-1 are marked as “available for intra prediction,” the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by

predSamples[ x, y ] = , with x, y = 0..nS-1	(8-33)
where k=log2(nS)

–	Otherwise, If all samples p[ x, -1 ], with x=0..nS-1 are marked as “available for intra prediction” and all samples p[ -1, y ], with y=0..nS-1 are marked as “not available for intra prediction,” the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by

predSamples[ x, y ] = , with x, y = 0..nS-1	(8-34)
where k=log2(nS)

–	Otherwise, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by

predSamples[ x, y ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-35)

[Ed: (WJ) current software implementation seems different. It uses division operation and may have a bug when both above and left samples are not available]

##### Specification of Intra_Angular prediction mode

[Ed: (WJ) provided text from JCTVC-C042 was modified. It still needs to be improved]

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is not equal to 0, 1, and 2.

Table 8-6 specifies the mapping table between intraPredMode and the rearranged intra prediction order intraPredOrder.

Table 8-6 – Specification of intraPredOrder

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredOrder | - | - | - | 1 | 5 | 13 | 17 | 21 | 29 | 33 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| intraPredMode | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredOrder | 31 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 |

Figure 8-2 illustrates the total 33 intra angles and Table 8-7 specifies the mapping table between intraPredOrder and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-7 – Specification of intraPredAngle

| intraPredOrder | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | -32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 |
| intraPredOrder | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| intraPredAngle | 32 | -26 | -21 | -17 | -13 | -9 | -5 | -2 | - | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-8 further specifies the mapping table between intraPredOrder and the inverse angle parameter invAngle.

Table 8-8 – Specification of invAngle

| intraPredOrder | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | -256 | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 |
| intraPredOrder | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| invAngle | -315 | -390 | -482 | -630 | -910 | -1638 | -4096 | - |

The reference pixel array refMain[ x ], with x=-nS..2*nS is specified as follows.

–	If intraPredOrder is less than 18,

refMain[ x ] = p[ -1+x, -1 ], with x=0..nS		(8-36)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1, -1+( ( x*invAngle+128 )>>8 ) ], with x=( nS*intraPredAngle ) >>5..-1	(8-37)

- Otherwise,

refMain[ x ] = p[ -1+x, -1 ], with x=nS+1..2*nS		(8-38)

Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=0..nS		(8-39)

- If intraPredAngle is less than 0,

refMain[ x ] = p[ -1+( ( x*invAngle+128 )>>8 ), -1 ], with x=( nS*intraPredAngle ) >>5..-1	(8-40)

- Otherwise,

refMain[ x ] = p[ -1, -1+x ], with x=nS+1..2*nS		(8-41)

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived by the following procedures.

–	The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(8-42)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(8-43)

–	Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5	(8-44)

- Otherwise, the value of the prediction samples predSamples[ x, y ] is derived by

predSamples[ x, y ] = refMain[ x+iIdx+1 ]

- If IntraPredMode is equal to 5

- For(y=0;y<nS;y++)

predSamples[0,y]=(12*predSamples[0,y]+3*p[-1,y+2]+p[-1,y+3]+8)>>4

- If IntraPredMode is equal to 6

- For(y=0;y<nS;y++)

predSamples[0,y]=(8*predSamples[0,y]+8*p[-1,y+1]+8)>>4

predSamples[1,y]=(12*predSamples[1,y]+4*p[-1,y+2]+8)>>4

predSamples[2,y]=(14*predSamples[2,y]+2*p[-1,y+3]+8)>>4

predSamples[3,y]=(15*predSamples[3,y]+p[-1,y+4]+8)>>4

- If IntraPredMode is equal to 8

- For(x=0;x<nS;x++)

predSamples[x,0]=(12*predSamples[x,0]+3*p[x+2,-1]+p[x+3,-1]+8)>>4

- If IntraPredMode is equal to 9

- For(y=0;y<nS;y++)

predSamples[x,0]=(8*predSamples[x,0]+8*p[x+1,-1]+8)>>4

predSamples[x,1]=(12*predSamples[x,1]+4*p[x+2,-1]+8)>>4

predSamples[x,2]=(14*predSamples[x,2]+2*p[x+3,-1]+8)>>4

predSamples[x,3]=(15*predSamples[x,3]+p[x+4,-1]+8)>>4

- If IntraPredMode is equal to 12, 22, and 23

- iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(8-42)

- iFact = ( ( y + 1 )*intraPredAngle ) && 31

- If iFact is not equal to 0, the value of the prediction samples pRefSamples[ -1, y ] is derived by

pRefSamples[ -1, y ] = ( ( 32 – iFact )*refMain[ iIdx ] + iFact*refMain[ iIdx+1] + 16 ) >> 5	(8-44)

- Otherwise, the value of the prediction samples pRefSamples[ -1, y ] is derived by

pRefSamples[ -1, y ] = refMain[ iIdx ]

- For(y=0;y<nS;y++)

predSamples[0,y]+=((p[-1,y]-pRefSamples[-1,y])+2)>>2

- If IntraPredMode is equal to 13

- For(y=0;y<nS;y++)

predSamples[0,y]=(12*predSamples[0,y]+2*p[-1,y+1]+2*p[-1,y+2]+8)>>4

- If IntraPredMode is equal to 16, 30, and 31

- iIdx = ( ( x + 1 )*intraPredAngle ) >> 5		(8-42)

- iFact = ( ( x + 1 )*intraPredAngle ) && 31

- If iFact is not equal to 0, the value of the prediction samples pRefSamples[ x, -1 ] is derived by

pRefSamples[ x, -1 ] = ( ( 32 – iFact )*refMain[ iIdx ] + iFact*refMain[ iIdx+1] + 16 ) >> 5	(8-44)

- Otherwise, the value of the prediction samples pRefSamples[ x, -1 ] is derived by

pRefSamples[ x, -1 ] = refMain[ iIdx ]

- For(x=0;x<nS;x++)

predSamples[x,-1]+=((p[x,-1]-pRefSamples[x,-1])+2)>>2

- If IntraPredMode is equal to 17

- For(x=0;x<nS;x++)

predSamples[x,0]=(12*predSamples[x,0]+2*p[x+1,-1]+2*p[x+2,-1]+8)>>4

- If IntraPredMode is equal to 24

- For(y=0;y<nS;y++)

predSamples[0,y]=(12*predSamples[0,y]+p[-1,y+1]+3*p[-1,y+2]+8)>>4

- If IntraPredMode is equal to 25

- For(y=0;y<nS;y++)

predSamples[0,y]=(8*predSamples[0,y]+6*p[-1,y+1]+2*p[-1,y+2]+8)>>4

- If IntraPredMode is equal to 32

- For(x=0;x<nS;x++)

predSamples[x,0]=(12*predSamples[x,0]+p[x+1,-1]+3*p[x+2,-1]+8)>>4

- If IntraPredMode is equal to 33

- For(x=0;x<nS;x++)

predSamples[x,0]=(8*predSamples[x,0]+6*p[x+1,-1]+2*p[x+2,-1]+8)>>4