Text for Working Draft

### Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if (IntraPredMode[ x0 ][ y0 ] != 34) && (IntraPredModeC[ x0 ][ y0 ] = = 2) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Prediction unit semantics

prev_intra_luma_pred_flag[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode of the ith prediction unit for luma samples with index i = 0 when IntraSplitFlag is equal to 0 or i =0..3 when IntraSplitFlag is equal to 1. When prev_intra_luma_pred_flag[ i ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit.

planar_flag_chroma[ x0 ][ y0 ] specifies whether the value of IntraPredModeC[ x0 ][ y0 ] should be changed to 34. planar_flag_chroma[ x0 ][ y0 ] is decoded from the bitstream only when both the following conditions are satisfied:

- IntraPredMode[ x0 ][ y0] is not equal to 34

- IntraPredModeC[ x0 ][ y0] is equal to 2

Otherwise, planar_flag_chroma[ x0 ][ y0 ] is inferred to be equal to 0. If planar_flag_chroma[ x0 ][ y0 ] is equal to 1, IntraPredModeC[ x0 ][ y0] is set equal to 34.

## Decoding process for coding units coded in intra prediction mode

### 8.3.1Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the number of luma intra prediction modes intraPredModeNum depending on log2TrafoSize.

Table 8-1 – Specification of intraPredModeNum

| log2TrafoSize | intraPredModeNum |
| --- | --- |
| 2 | 18 (modes numbered 0-16 and 34) |
| 3 | 35 (modes numbered 0-33 and 34) |
| 4 | 35 (modes numbered 0-33 and 34) |
| 5 | 35 (modes numbered 0-33 and 34) |
| 6 | 4 (modes numbered 0-3 and 34) |

Table 8-2 specifies the table used for re-ordering intra prediction modes.

Table 8-2 – Specification of unifyIntraPredMode[value]

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| unifyIntraPredMode[ value ] | 2 | 3 | 0 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| Value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| unifyIntraPredMode[ value ] | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| Value | 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| unifyIntraPredMode[ value ] | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-3 – Specification of mapIntraPredMode5 and mapIntraPredMode9

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode5[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 2 | 0 | 3 | 3 | 0 | 0 | 2 | 2 | 0 | 0 | 3 |
| mapIntraPredMode9[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| mapIntraPredMode5[ value ] | 3 | 0 | 4 | 5 | 5 | 2 | 2 | 2 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 0 |
| mapIntraPredMode9[ value ] | 0 | 0 | 4 | 5 | 5 | 2 | 2 | 6 | 6 | 7 | 4 | 8 | 8 | 3 | 3 | 9 | 9 |
| value | 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mapIntraPredMode5[ value ] | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mapIntraPredMode9[ value ] | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8-4 specifies the table used for converting mode indices of intra prediction modes to intra prediction modes.

Table 8-4 – Specification of ivsUnifyIntraPredMode[value]

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ivsUnifyIntraPredMode[ value ] | 2 | 34 | 0 | 1 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| ivsUnifyIntraPredMode[ value ] | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| Value | 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ivsUnifyIntraPredMode[ value ] | 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

ntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 33, 34 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

3

18

10

19

4

20

11

21

0

22

12

23

5

24

25

13

6

2: DC mode

34: Planar mode

26

14

27

7

28

15

29

1

30

16

31

8

32

17

33

9

IntraPredMode[ xB ][ yB ] is derived as follows.

–	The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

–	For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to 2

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- intraPredModeN=unifyIntraPredMode[intraPredModeN]

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 6 then candIntraPredModeN is set equal to mapIntraPredMode5[ intraPredModeN ]

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode3[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode9[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- If both candIntraPredModeN are not available, then the value 0 is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ] and NumMPMCand is set equal to 2.

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

- IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candModeList[cIdx]) IntraPredMode[ xB ][ yB ]++

- IntraPredMode = ivsUnifyIntraPredMode[IntraPredMode]