| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F124 |
| --- | --- |

| Title: | Extended Mode-Dependent Coefficient Scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *#Xin Zhao, *Xun Guo, *M. Guo, +Shawmin Lei, #Siwei Ma, #Wen Gao, *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #School of EECS, Peking University, Institute of Digital Media, Room 2641, Science Building #2, No.5, Yiheyuan Rd., Haidian District, Beijing, China, 100871 +No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | {x.zhao, xun.guo, mei.guo, shawmin.lei}@mediatek.com {swma, wgao}@pku.edu.cn |
| Source: | #MediaTek Inc., *Peking University |  |  |

_____________________________

# Abstract

In this contribution, an extended mode-dependent coefficient scanning (MDCS) method is proposed. Based on the MDCS in HM 3.0, this proposal makes two extensions. First, a modified MDCS is applied to larger block sizes including 16×16 and 32×32, in which the horizontal scanning order and vertical scanning order are different from the current ones. Second, MDCS is also applied for scanning of the absolute values in CABAC, which only uses zigzag scanning now. It is reported that, with both modifications, average 0.3% and 0.7% BD-rate reductions are reported for AI-HE and AI-LC configurations, respectively. It is also reported that the running time of the proposed method is almost the same with that of anchor.

# Introduction

In the 4th JCTVC meeting held in Daegu, a MDCS method is proposed [1] and adopted into the HM software. For Intra coded coefficients, MDCS utilizes three scanning patterns including pure horizontal, vertical and zigzag scanning as illustrated in Figure 1.

Figure 1 Three scanning patterns in HM3.0: zigzag (left), horizontal (middle) and vertical (right).

The selection of the scanning patterns for each transform unit (TU) size is based on intra prediction mode, as shown in table 1. MDCS is only used for TU sizes of 4x4 and 8x8.

Table 1 LUT of scanning patterns for intra luma block in HM 3.0

| Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 2x2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

For CABAC coding, the absolute values of an N×N block is further split into sub-blocks of size 4×4. For each sub-block, elements are scanned in a reversed zigzag order. An example for scanning the absolute values of an 8x8 block is illustrated in Figure 2.

Figure 2 Sub-block scanning of absolute levels of an 8x8 block in CABAC.

# Proposed methods

Two extensions are proposed in this contribution. First, a modified MDCS is applied to 16x16 and 32x32 intra blocks, in which the horizontal scanning order and vertical scanning order are different from the current ones. Second, MDCS is also applied to coding absolute values in CABAC.

## Scanning for block sizes of 16x16 and 32x32

To retrieve scanning patterns for large block sizes including 16x16 and 32x32, an image correlation model is assumed as below,

,					(1)

where  is the variance of signal samples, (x0, y0) and (x1, y1) are the coordinates of sample s0 and s1; ρx and ρy are correlation coefficients for horizontal and vertical directions, respectively. This image model has been applied in [2] for analyzing the covariance matrix of directional prediction residue.

For an n×n block X predicted along vertical direction, as figure 3 states,

| p1 | p2 | ··· | pn |
| --- | --- | --- | --- |
| x1,1 | x1,2 | ··· | x1,n |
| x2,1 | x2,2 | ··· | x2,n |
| ··· | ··· | ··· | ··· |
| xn,1 | xn,2 | ··· | xn,n |

Figure 3 Intra vertical prediction of an n×n block.

the residual block Y is calculated as

.		 (2)

Equation 2 can be alternatively written as

.		(3)

n

n×n

To perform transform on Y, the following arithmetic is implemented,

,						(4)

where C and R are the n×n column and row transform matrix, respectively. Similar to (3), equation (4) can be alternatively written as

,	    (5)

where B is actually the Kronecker product of C and RT, i.e., [3]. With (3) and (5), we have

.  					   (6)

Let us denote A=B·P,  and , we have

,							(7)

therefore the variance of the element in  can be calculated as

,					(8)

If both C and RT are restricted to be a DCT matrix, A is a fixed matrix. Given the image correlation defined in (1), the variances of the transform coefficient can be calculated with (8). For example, for ρx=0.4, ρy=0.9 and n=8, the calculated variance value for each element in 8×8 is,

| 8.087 | 6.312 | 4.876 | 3.601 | 2.744 | 2.213 | 1.901 | 1.737 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.068 | 1.614 | 1.247 | 0.921 | 0.702 | 0.566 | 0.486 | 0.444 |
| 0.712 | 0.555 | 0.429 | 0.317 | 0.241 | 0.195 | 0.167 | 0.153 |
| 0.341 | 0.266 | 0.206 | 0.152 | 0.116 | 0.093 | 0.080 | 0.073 |
| 0.215 | 0.168 | 0.130 | 0.096 | 0.073 | 0.059 | 0.051 | 0.046 |
| 0.156 | 0.122 | 0.094 | 0.069 | 0.053 | 0.043 | 0.037 | 0.033 |
| 0.127 | 0.099 | 0.076 | 0.056 | 0.043 | 0.035 | 0.030 | 0.027 |
| 0.112 | 0.088 | 0.068 | 0.050 | 0.038 | 0.031 | 0.026 | 0.024 |

Given the variances of each transform coefficient, the corresponding scanning order can be retrieved by sorting the positions in a descending order of the variance values.

It is noted that blocks with high ρy values tend to select vertical prediction as the optimal prediction direction, in this contribution, extensive simulations have been performed, and the best performance locates at “ρx=0.45, ρy=0.99” for n=16 and “ρx=0.6, ρy=0.99” for n=32. The scanning orders for vertical prediction on both 16×16 and 32×32 are available in the Appendix A. The scanning order for horizontal prediction is the transposed version of the vertical one. In the proposed implementation, the selection of scanning order for 16×16 and 32×32 is based on Table 2.

Table 2 Proposed LUT of scanning patterns for intra luma block

| Mode TU | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 | 0 |
| 16x16 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 2x2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Scanning absolute values in CABAC

For coding the absolute values of an N×N block with N larger than 4, the “4×4 sub-block” partition in HM 3.0 is maintained. However, for each sub-block, elements are scanned and coded using a reversed 4×4 scanning pattern defined in MDCS, with the same scanning order index used for scanning the significance flag. For example, three proposed scanning patterns for scanning the absolute values of an 8×8 block is illustrated in Figure 4.

Zigzag: 0					Horizontal: 1			Vertical: 2

Figure 4 Proposed sub-block scanning patterns for absolute levels of an 8x8 block in CABAC.

# Simulations and results

The proposed extended MDCS is implemented on top of HM 3.0. Simulations are performed under all intra configurations defined in [3]. Since two extensions are proposed in this contribution, which include (A) scanning for large block sizes and (B) scanning of absolute values in CABAC, three experiments are performed, including “A”, “B”, and “A+B”. The BD-rate reductions for the three experiments are reported in Table 3, 4 and 5, respectively.

Table 3 Experimental results of A

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | -0.3% | -0.7% | -0.6% |
| Class B | -0.1% | -0.6% | -0.6% | -0.9% | -1.4% | -1.4% |
| Class C | -0.3% | -0.4% | -0.4% | -0.5% | -0.8% | -0.8% |
| Class D | -0.1% | -0.3% | -0.2% | -0.4% | -0.5% | -0.4% |
| Class E | -0.5% | -1.0% | -1.0% | -1.2% | -1.8% | -1.5% |
| All Avg | -0.18% | -0.49% | -0.46% | -0.65% | -1.02% | -0.95% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 101% |  |  | 100% |  |  |

Table 4 Experimental results of B

|  | Intra, HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% |
| Class B | -0.1% | 0.0% | -0.1% |
| Class C | -0.4% | -0.2% | -0.2% |
| Class D | -0.2% | -0.1% | -0.1% |
| Class E | -0.1% | -0.1% | 0.0% |
| All Avg | -0.15% | -0.10% | -0.09% |
| Enc Time (%) | 100% |  |  |
| Dec Time (%) | 101% |  |  |

Table 5 Experimental results of A+B

|  | Intra, HE |  |  | Intra, Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | -0.3% |
| Class B | -0.2% | -0.2% | -0.2% | -0.9% | -0.9% | -0.9% |
| Class C | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% | -0.5% |
| Class D | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class E | -0.6% | -0.6% | -0.6% | -1.2% | -1.2% | -1.2% |
| All Avg | -0.30% | -0.62% | -0.57% | -0.65% | -1.02% | -1.02% |
| Enc Time (%) | 100% |  |  | 100% |  |  |
| Dec Time (%) | 102% |  |  | 100% |  |  |

# Conclusion

Extended MDCS is proposed in this contribution, which includes two extensions:

- A modified MDCS is applied for larger block sizes including 16×16 and 32×32;

- MDCS is applied for scanning the absolute values in CABAC entropy coding.

Combination of both proposed methods achieves BD-Rate gains of 0.3% and 0.7% for AI-HE and AI-LC, respectively. The proposed methods introduce minor changes to the HM 3.0 software. Moreover, almost no complexity increase is observed at both encoder and decoder.

# References

[1] Y. Zheng, M. Coban, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D393, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[2] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding,” JCTVC-B024, 2nd Meeting: Geneva, Switzerland, Jul 2010.

[3] R. A. Horn and C. R. Johnson, Topics in Matrix Analysis. New York: Cambridge Univ. Press, 1991, pp. 254–255.

[4] F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

Scanning order for 16×16 coefficient matrix with vertical prediction, i.e., ScanOrderH16

Scanning order for 32×32 coefficient matrix with vertical prediction, i.e., ScanOrderH32

# Appendix B

The proposed text changes to the working draft are provided as below,

## 6.1 Scanning order array initialisation process

Input to this process is a block size blkSize.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a zig-zag scan as specified in subclause 6.5, scanIdx equal to 1 specifies a horizontal scan and scanIdx equal to 2 specifies a vertical scan. The array index pos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = ZigZag

if (blkSize<16){
			i  =  0
			y  =  0
			while( y  < blkSize ) {
				x  =  0
				while( x  <  blkSize ) {
					ScanOrder[ 1 ][ i ][ 0 ]  = x
					ScanOrder[ 1 ][ i ][ 1 ]  = y
					x++
					i++
				}
				y++
			}

}
else if(blkSize==16){
    ScanOrder[1] = ScanOrderH16
} 
else if(blkSize==32){
    ScanOrder[1] = ScanOrderH32
}


i  =  0
while( i  < blkSize* blkSize ) {
    ScanOrder[ 2 ][ i ][ 0 ] = ScanOrder[ 1] [ i ]>>log2(blkSize) 
    ScanOrder[ 2 ][ i ][ 1 ] = ScanOrder[ 1] [ i ]%log2(blkSize)
}

Table 7-15 – Specification of ScanType[ log2TrafoSize – 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 | 3 | 4 | 5 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 2 | 2 | 2 | 2 |
| 2-3 | 0 | 0 | 0 | 0 |
| 4-5 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 |
| 7-8 | 2 | 2 | 0 | 0 |
| 9-10 | 0 | 0 | 0 | 0 |
| 11-12 | 1 | 1 | 0 | 0 |
| 13-14 | 0 | 0 | 0 | 0 |
| 15-16 | 2 | 2 | 0 | 0 |
| 17-19 | 0 | 0 | 0 | 0 |
| 20 | 1 | 1 | 0 | 0 |
| 21-22 | 1 | 1 | 1 | 1 |
| 21 | 1 | 1 | 0 | 0 |
| 24-27 | 0 | 0 | 0 | 0 |
| 28 | 2 | 2 | 0 | 0 |
| 29-30 | 2 | 2 | 2 | 2 |
| 31 | 2 | 2 | 0 | 0 |
| 32-33 | 0 | 0 | 0 | 0 |

### 8.5.2	Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nS specifying the size of the current transform unit,

–	a list of (nS)x(nS) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block,

–	a variable entropy_coding_mode_flag specifying the entropy_coding_mode,

–	a variable scanIdx specifying the inverse scanning method,

Output of this process is a variable c containing a two-dimensional array of (nS)x(nS) values.

- The variable c[ i ][ j ] which is located in the ( i, j ) position in the array c is derived as follows.

- If entropy_coding_mode_flag is equal to 1, the current transform unit is divided into ( nS >> 2 ) * ( nS >> 2 ) subsets of 4 * 4 samples. The subsets are processed in zig-zag scan order. The position ( k0, l0 ) in the array c, corresponding to the top left position of a subset s, is derived as follows.

k0 = ZigZag[ log2( nS >> 2 ) ][ s ][ 0 ] << 2		
l0 = ZigZag[ log2( nS >> 2 ) ][ s ][ 1 ] << 2, 		
with s = 0.. ( ( nS >> 2 ) * ( nS >> 2 ) ) – 1 )                                        (8-152)

  - For every subset s, the 16 transform coefficient level at scanning position n are mapped to the position ( i, j ) in the array c using the a reverse scan order specified by scanIdx.

i = k0 + ScanOrder [ 0 ][ scanIdx ][ 15 - n ][ 0 ]		
	j = l0 + ScanOrder [ 0][ scanIdx ][ 15 - n ][ 1 ]		
	c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n + ( s << 4 ) ],		
	with n = 15..0                                                              (8-153)

- If entropy_coding_mode_flag is equal to 0, the (nS)x(nS) values at a scanning position n are mapped to the position ( i, j ) in the array c using the a reverse scan order specified by scanIdx

i = ScanOrder [ log2(nS)-2 ][ scanIdx ][ n ][ 0 ] 		
j = ScanOrder [ log2(nS)-2 ][ scanIdx][ n ][ 1 ]		
c[ i ][ j ] = transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ][ n ], 	 with n = (nS)x(nS)-1 ..0                                                        (8-154)