| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22, July, 2011 | Document: JCTVC-F125 |
| --- | --- |

| Title: | Progressive MV Resolution |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Jicheng An *Xiang Li *Xun Guo #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | jicheng.an@mediatek.com x.li@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes a progressive MV resolution (PMVR) method, which allows adaptation among different MV resolutions. In PMVR, MV resolution is not explicitly signaled with specific syntax but progressively adjusted based on the magnitude of MV difference. It is reported that 1.0% luma BD-rate reduction is obtained on average for all four inter configurations (RA-HE, RA-LC, LB-HE, and LB-LC). Moreover, it is also reported that average encoding time is saved by 5% while average decoding time is slightly increased by 2%. Additionally, for LP-HE and LP-LC configurations, averagely 2.8% luma BD-rate reduction and similar encoding and decoding time are achieved.

# Introduction

In H.264/AVC [1] and current HEVC [2], MV resolution is always fixed to 1/4 pixel. In JCTVC-A121[3], an adaptive MV resolution (AMVRES) method is proposed. In AMVRES, MV resolution is adaptively selected between 1/4 and 1/8 pixel for each block. To signal the MV resolution to decoder, additional parameters have to be coded at block level. Clearly, the overhead of the additional parameters degrades the performance of AMVRES.

In this contribution, a progressive MV resolution (PMVR) method is proposed. In PMVR, MV resolution is progressively adjusted based on the magnitude of MV difference (MVD) so that overhead can be almost avoided. Moreover, with slice level adaptation, MV resolution can be properly selected based on input video content. Consequently, both rate reduction and encoding time saving can be obtained.

# General Idea

In the coding environment of H.264/AVC [1] and current HEVC [2], it is observed that MV close to MV predictor (MVP) is more likely to be rate-distortion (R-D) efficient. Therefore, it is proposed to employ different MV resolutions in a progressive manner. That is, using higher MV resolution when MV is close to MVP and lower MV resolution when MV is far away from MVP.

Figure 1 presents examples of fixed MV resolution and progress MV resolution. In the left example, MV accuracy is always set to 1/4 pixel. While in the right one, MV resolution is progressively decreased as the distance to MVP is increased. For simplicity, up to three kinds of resolution are possible with different sizes of boxes, which can be implemented by thresholds. Specifically, the highest MV resolution, i.e., 1/8-pixel accuracy is used within the dashed red square which is indicated by threshold TH0. For the region between the dashed red square and the solid red square which is determined by threshold TH1, 1/4-pixel accuracy is retained. Finally, 1/2 pixel accuracy is used outside the solid red square.

Figure 1 Comparison between fixed MV resolution and progress MV resolution

Left: Fixed MV resolution (1/4 pixel accuracy). Right: Progressive MV resolution (1/8, 1/4, and 1/2 pixel accuracy)

Generally, the right example in Figure 1 indicates that MV resolution is determined by MVD and certain predefined thresholds. Hence, no resolution selection overhead is needed. Based on this idea, PMVR is proposed. In principle, PMVR can support any number of resolutions. In this contribution, only three resolutions (1/8, 1/4, and 1/2 pixel accuracy) are implemented as a tradeoff between coding performance and complexity. In section 3, the proposed algorithm is described in details.

# Algorithm Description

To support three different resolutions in PMVR, two thresholds TH0 and TH1 are needed. Based on these two thresholds, MVD encoding is described in section 3.1. Then MV derivation for skip and merge modes is introduced in section 3.2. Subsequently, the interpolation for 1/8-pixel resolution and mode decisions are depicted in section 3.3 and 3.4, respectively. Finally, threshold adaptation at slice level is depicted in section 3.5.

Note that in PMVR, MV is always kept with the highest MV resolution. In current implementation, 1/8, 1/4, and 1/2 pixel accuracy are supported. Therefore, MV is always kept with 1/8-pixel accuracy.

## MVD Encoding

Firstly, define MVD8 and MVP8 as actual MVD and MVP, respectively. Since MV is always kept with 1/8-pixel accuracy in the current implementation, MVD8 and MVP8 are also with 1/8-pixel accuracy. The target of MVD encoding is to derive the value of MVDC (the MVD value to be coded in bitstream) based on MVD8, MVP8, TH0 and TH1.

To ensure that integer and half pixel positions are always accessible and facilitate the motion estimation in PMVR, CTR0 and CTR1 are defined as biased centers of 1/8-pixel-resolution region and 1/4-pixel-resolution region, namely,

Figure 2 Proposed PMVR with (TH1, TH0) = (4, 2)

Left: MVP is at half or integer pixel position. Right: MVP is at 1/8-pixel position

CTR0_x = (MVP8_x>>1<<1) – MVP8_x

CTR0_y = (MVP8_y>>1<<1) – MVP8_y

CTR1_x = ((MVP8_x+1)>>2<<2) – MVP8_x

CTR1_y = ((MVP8_y+1)>>2<<2) – MVP8_y

Figure 2 shows two examples of PMVR with (TH1, TH0) = (4, 2). In Figure 2 (left), MVP is at half or integer pixel position. According to , CTR0 and CTR1 are both zero. In this case, MVP, CTR0 and CTR1 coincide. On the other hand, MVP in Figure 2 (right) is at 1/8-pixel position. According to , CTR0_x=-1, CTR0_y=0, CTR1_x=-1, CTR1_y=-2. In this case, CTR0, CTR1 and MVP are at different positions.

As mentioned before, TH0 and TH1 are two thresholds which specify the region of 1/8, 1/4, and 1/2 pixel MV resolutions. In the current implementation, TH0 and TH1 are restricted to integer multiples of 2 and 4, respectively. In an extreme case where the TH1 and TH0 are both set to infinite, 1/8 pixel resolution is always used. Similarly, if both TH0 and TH1 are set to 0, 1/2 pixel resolution is always employed.

Since not all the 1/8 or 1/4 pixel positions are needed to represent an MVD in PMVR, the usual representation of MVD is converted by Algorithm 1 to a more efficient (or dense) representation, MVDc. Then MVDc is coded with CABAC or CAVLC in the same way as MVD coding in HEVC.

Algorithm 1: MVD encoding

input : MVD8, MVP8, CTR0, CTR1, TH0 and TH1 ouput : MVDc begin If ¦MVD8_x-CTR0_x¦<=TH0 and ¦MVD8_y-CTR0_y¦<=TH0, MVDc_x = MVD8_x; MVDc_y = MVD8_y; Else if the ¦MVD8_x-CTR1_x ¦ <=TH1 and ¦MVD8_y-CTR1_y ¦<=TH1, If ¦MVD8_x-CTR0_x¦>TH0, If MVD8_x-CTR0_x>=0 MVDc_x = (MVD8_x+CTR0_x+TH0)/2; Else MVDc_x = (MVD8_x+CTR0_x-TH0)/2; MVDc_y = (MVD8_y-CTR0_y)/2; Else If MVD8_y-CTR0_y>=0 MVDc_y = (MVD8_y+CTR0_y+TH0)/2; Else MVDc_y = (MVD8_y+CTR0_y-TH0)/2; MVDc_x = (MVD8_x-CTR0_x)/2; Else If ¦MVD8_x-CTR1_x¦>TH1, If MVD8_x-CTR1_x>=0, MVDc_x = (MVD8_x+2(CTR0_x+TH0)+CTR1_x+TH1)/4; Else MVDc_x = (MVD8_x+2(CTR0_x-TH0)+CTR1_x-TH1)/4; MVDc_y = (MVD8_y-CTR1_y)/4; Else If MVD8_y-CTR1_y>=0, MVDc_y = (MVD8_y+2(CTR0_y+TH0)+CTR1_y+TH1)/4; Else MVDc_y = (MVD8_y+2(CTR0_y-TH0)+CTR1_y-TH1)/4; MVDc_x = (MVD8_x-CTR1_x)/4.

end

## MV in Skip and Merge Modes

In skip and merge modes, MVD value is always zero and doesn’t need to be coded. Accordingly, PMVR is not applied and MVP8 is taken as MV directly.

When both TH1 and TH0 are set to zero, 1/2-pixel resolution is always used for inter mode other than skip and merge. In this case, 1/8-pixel and 1/4-pixel MV positions will only occur due to the MV scaling in temporal skip and merge modes. When such high accuracy MV is taken as a MVP for blocks in inter mode other than skip and merge, the MV has to be converted to 1/2-pixel accuracy as follows.

MVP8_x=(MVP8_x+1)>>2<<2;

MVP8_y=(MVP8_y+1)>>2<<2;

Similarly, when TH0 is set to zero and TH1 is not set to zero, 1/4-pixel resolution is the highest resolution used for inter mode other than skip and merge. In this case, 1/8-pixel MV positions will only occur due to the MV scaling in temporal skip and merge modes. When such high accuracy MV is taken as a MVP for blocks in inter mode other than skip and merge, the MV has to be converted to 1/4-pixel accuracy as follows.

MVP8_x=MVP8_x>>1<<1;

MVP8_y=MVP8_y>>1<<1;

## Interpolation for 1/8 Pixel Resolution

To support 1/8-pixel resolution, 1/8 pixel interpolation filter for luma and 1/16-pixel interpolation filter for chroma are proposed based on DCT-IF as follows.

Firstly, the DCT-IF [4][5] with 8 taps and 6-bit coefficient representation is proposed as 1/8-pixel luma interpolation filter. The filter coefficients and the number of operations are summarized in Table 1.

Table 1 Proposed 1/8-Pixel Luma Interpolation Filter

| Position | Filter Coefficients |  |  |  |  |  |  |  | ">>" | "+" |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/8 | {-1, | 3, | -6, | 62, | 9, | -4, | 2, | -1} | 4 | 11 |
| 3/8 | {-2, | 5, | -12, | 50, | 30, | -10, | 4, | -1} | 5 | 13 |

Secondly, a 4-tap 6-bit filter is proposed for 1/16-pixel chroma interpolation. The filter coefficients and the number of operations are presented in Table 2.

Table 2 Proposed 1/16 Pixel Chroma Interpolation Filter

| Position | Filter Coefficients |  |  |  | ">>" | "+" |
| --- | --- | --- | --- | --- | --- | --- |
| 1/16 | {-2, | 63, | 4 | -1} | 3 | 4 |
| 3/16 | {-5, | 59, | 13 | -3} | 3 | 9 |
| 5/16 | {-6, | 52, | 23, | -5} | 5 | 9 |
| 7/16 | {-7, | 43, | 34, | -6} | 4 | 9 |

Note that the tap number and coefficient representation of the proposed 1/8-pixel resoultion interpolation filters are the same as those for 1/2 pixel and 1/4 pixel resolution interpolation filters in current HEVC. Accordingly, the computational cost of proposed filters is similar to that of current HEVC.

## Motion Estimation and Mode Decision

The motion estimation (ME) and mode decision processes are similar to those in HM-3.0 [7] except that only the progressive resolution points need to check.

## Threshold Adaptation at Slice Level

To further improve coding efficiency, the thresholds TH1 and TH0 are adaptively selected for each reference picture at slice level. For each inter slice, one flag is coded to indicate whether all reference pictures of this slice share the same threshold pair. If so, the threshold index is encoded once. Otherwise, threshold index of each reference picture is coded separately. As a tradeoff between complexity and coding performance, it is proposed to adaptively select threshold pair (TH1, TH0) among (4, 2), (4, 0) and (0, 0). Section A.2 in Annex shows the syntax revisions in slice header.

In our encoder implementation, for random-access configurations, all reference pictures of one slice share the same threshold pair which is determined by Algorithm 2, where  RDCost0, RDCost2, RDCost4, RDCost8 denote the sum ME RD cost of integer, 1/2, 1/4, and 1/8 pixel positions of the previously coded same-type slice.

For low-delay configurations, reference pictures whose QPs are no larger than the QP of current slice share the same threshold pair which is also determined by the algorithm 2. For reference pictures whose QPs are larger than that of current slice, the threshold pair is always set to (0, 0).

As shown in Algorithm 2, threshold pair is selected based on RD cost of previously coded slice. In general, the larger the RD cost reduction by sub-pixel ME, the larger the MV resolution threshold is selected.

Algorithm 2: MV Resolution Threshold Adaptation at Slice Level

input : RDCost0, RDCost2, RDCost4, RDCost8 output : TH1, TH0 begin if the first inter slice set (TH1, TH0) to (4,0) else set (TH1, TH0) to that of the previously coded slice RDCReduc0=(RDCost0-RDCost2)/RDCost0; RDCReduc1=(RDCost2-RDCost4)/RDCost2; RDCReduc2=(RDCost4-RDCost8)/RDCost4; if (TH1, TH0) of the previously coded slice is (4, 0) RDTH = max(RDCReduc0*0.2,0.5%); if RDCReduc1<RDTH && RDCReduc0<8% set (TH1, TH0) to (0, 0); set RDTH= RDCReduc0*min(RDTH/ RDCReduc1, 1.5); //used for following inter slices else if (RDCReduc1>6%) ¦¦ (RDCReduc1>3% && RDCReduc1> RDCReduc0*0.4) set (TH1, TH0) to (4, 2). else if (TH1, TH0) of the previously coded slice is (0, 0) if RDCReduc0>min(RDTH, 8%) set (TH1, TH0) to (4, 0) else if (TH1, TH0) of the previously coded slice is (4, 2) if (RDCReduc2<1%) ¦¦ ((RDCReduc2<2% && RDCReduc2< RDCReduc1*0.4) set (TH1, TH0) to (4, 0)

end

# Experimental Results

To evaluate the performance of PMVR, simulations are conducted under conditions defined in JCTVC-E700 [6]. First in section 4.1, the proposed threshold adaptive PMVR as described in section 3.5 is verified. Then in section , PMVR with three pairs of fixed thresholds are tested separately. Finally in section 4.3, PMVR without 1/8-pixel resolution and slice level threshold adaptation is investigated.

## Performance of Threshold Adaptive PMVR

The simulation results of proposed PMVR are summarized in Error: Reference source not found. On average, 1.0% luma BD-rate reduction is obtained for the four inter configurations (RAHE, RALC, LBHE, and LBLC). Average encoding time is reduced by 5%, which is since that interpolation and motion estimation at unnecessary positions are avoided. However, the decoding time is slightly increased by 2%, which is due to the support of 1/8-pixel resolution. Additionally, for LP-HE and LP-LC configurations, averagely 2.8% luma BD-rate reduction and similar encoding and decoding time are achieved.

Table 3 Performance of proposed threshold adaptive PMVR

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -1.3 | -1.0 | -0.9 | -0.7 | -0.8 |
| Class B | -0.3 | -0.7 | -0.6 | -0.6 | -0.8 | -0.7 |
| Class C | -0.8 | -1.0 | -1.2 | -1.1 | -1.5 | -1.6 |
| Class D | -1.2 | -1.5 | -1.6 | -1.9 | -1.8 | -1.9 |
| Class E |  |  |  |  |  |  |
| Overall | -0.7 | -1.1 | -1.1 | -1.1 | -1.2 | -1.2 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.7 | -0.6 | -0.8 | -1.5 | -1.5 |
| Class C | -1.1 | -1.2 | -1.2 | -1.5 | -1.2 | -1.2 |
| Class D | -1.6 | -2.3 | -2.8 | -2.3 | 0.1 | 1.1 |
| Class E | -0.2 | 0.1 | -1.6 | -0.9 | -1.1 | -0.6 |
| Overall | -0.8 | -1.1 | -1.5 | -1.3 | -0.9 | -0.6 |
| Enc Time[%] | 94% |  |  | 92% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.4 | -1.7 | -1.7 | -1.4 | -0.2 | 0.0 |
| Class C | -3.3 | -2.5 | -2.6 | -3.0 | -1.5 | -1.4 |
| Class D | -5.6 | -5.3 | -5.5 | -5.3 | -1.1 | -0.1 |
| Class E | -2.0 | -1.1 | -0.9 | -0.8 | 1.1 | 1.6 |
| Overall | -3.0 | -2.7 | -2.8 | -2.6 | -0.5 | -0.1 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

## Performance of PMVR with fixed Thresholds

To study the impact of threshold slice level adaptation, fixed thresholds are tested in this section. Table 4, Table 5, and Table 6 present the simulation results of (TH1, TH0) = (4, 2), (4, 0), and (0, 0), respectively. On average, 0.5% and 0.7% luma BD-rate reduction are obtained for the four inter configurations with similar coding time for (TH1, TH0) = (4, 2) and (4, 0) respectively. In general, (TH1, TH0) = (4, 0) performs best on average. While (TH1, TH0) = (4, 2) is more efficient for high bit-rate videos or videos with slow camera motion such as BQSqaure and PartyScence. The (0, 0) case has the lowest encoding complexity and is efficient for many low bit rate cases. When threshold adaptation is enabled, a better performance is achieved, as shown in section 4.1.

Table 4 Performance of PMVR with fixed threshold (TH1, TH0) = (4, 2)

|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4% | -1.0% | -1.2% | -0.2% | -1.5% | -1.2% |
| Class B | -0.6% | -0.9% | -0.9% | -0.5% | -0.8% | -0.6% |
| Class C | -1.0% | -1.2% | -1.2% | -1.1% | -1.4% | -1.5% |
| Class D | -1.1% | -1.5% | -1.5% | -1.5% | -1.5% | -1.5% |
| Class E |  |  |  |  |  |  |
| All | -0.8% | -1.1% | -1.1% | -0.8% | -1.2% | -1.2% |
| Enc Time[%] | 109% |  |  | 111% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.3% | 0.3% | 2.1% | 2.3% |
| Class C | -0.6% | -0.8% | -0.7% | -0.7% | 0.7% | 0.8% |
| Class D | -1.2% | -1.3% | -1.6% | -1.7% | 4.9% | 5.9% |
| Class E | 1.0% | 1.2% | 0.7% | 2.7% | 6.0% | 4.0% |
| All | -0.3% | -0.4% | -0.5% | 0.0% | 3.2% | 3.1% |
| Enc Time[%] | 113% |  |  | 116% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |

Table 5 Performance of PMVR with fixed threshold (TH1, TH0) = (4, 0)

|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6% | -1.2% | -1.4% | -0.7% | -1.1% | -1.0% |
| Class B | -0.5% | -0.7% | -0.6% | -0.6% | -0.7% | -0.6% |
| Class C | -0.8% | -0.9% | -1.1% | -0.9% | -1.2% | -1.3% |
| Class D | -1.0% | -1.3% | -1.4% | -1.3% | -1.3% | -1.4% |
| Class E |  |  |  |  |  |  |
| All | -0.7% | -1.0% | -1.1% | -0.9% | -1.1% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.2% | -0.5% | -0.4% | -0.6% | -0.6% |
| Class C | -0.7% | -0.9% | -0.9% | -0.8% | -0.7% | -0.6% |
| Class D | -0.7% | -0.9% | -1.0% | -1.0% | -0.7% | -0.8% |
| Class E | -0.2% | -0.2% | -0.6% | -0.2% | 0.4% | -0.1% |
| All | -0.5% | -0.5% | -0.7% | -0.6% | -0.4% | -0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |

Table 6 Performance of PMVR with fixed threshold (TH1, TH0) = (0, 0)

|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7% | -0.9% | -0.6% | -0.1% | -0.5% | -0.5% |
| Class B | 1.4% | 0.1% | 0.4% | 1.1% | -0.1% | 0.1% |
| Class C | 1.8% | 0.5% | 0.3% | 1.7% | 0.1% | 0.0% |
| Class D | 1.7% | 0.3% | 0.2% | 1.7% | 0.4% | 0.1% |
| Class E |  |  |  |  |  |  |
| All | 1.4% | 0.0% | 0.1% | 1.1% | 0.0% | 0.0% |
| Enc Time[%] | 86% |  |  | 80% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.3% | 0.4% | 0.6% | 1.0% | 0.5% | 0.6% |
| Class C | 1.4% | 0.5% | 0.5% | 1.7% | 1.4% | 1.3% |
| Class D | 2.9% | 1.0% | 1.5% | 3.2% | 3.6% | 4.3% |
| Class E | 1.3% | 0.6% | -0.5% | -0.3% | 0.1% | 0.7% |
| All | 1.7% | 0.6% | 0.6% | 1.5% | 1.4% | 1.7% |
| Enc Time[%] | 82% |  |  | 78% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

## Performance of PMVR without 1/8 Pixel-Resolution and Threshold Adaptation

In this section, PMVR without 1/8-pixel resolution and threshold adaptation is tested. That is PMVR without TH0 and with fixed TH1. Here the TH1 has only 1/4 pixel accuracy instead of 1/8 pixel accuracy as defined in Section 3.1. The simulation results of TH1=2 are summarized in Table 7. On average, 0.35% luma BD-rate reduction, and 2%~3% encoding time saving are achieved for the four inter configurations. Compared with the results in section 4.1 and , it can be concluded that 1/8-pixel resolution and threshold adaptation can provide significant coding gain.

Table 7 Performance of PMVR with 1/8-pixel resolution disabled and fixed threshold TH1=2

|  | RandomAccess |  |  | RandomAccess_LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6% | -1.2% | -1.1% | -0.8% | -0.9% | -0.9% |
| Class B | -0.4% | -0.6% | -0.5% | -0.4% | -0.6% | -0.5% |
| Class C | -0.4% | -0.6% | -0.8% | -0.4% | -0.8% | -0.8% |
| Class D | -0.2% | -0.7% | -0.7% | -0.2% | -0.6% | -0.7% |
| Class E |  |  |  |  |  |  |
| All | -0.4% | -0.8% | -0.8% | -0.4% | -0.7% | -0.7% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | LowDelay |  |  | LowDelay_LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.5% | -0.4% | -0.4% | -0.4% |
| Class C | -0.4% | -0.6% | -0.4% | -0.4% | -0.2% | -0.5% |
| Class D | -0.2% | -0.4% | -0.3% | -0.2% | -0.6% | -0.5% |
| Class E | -0.3% | -0.7% | -0.6% | -0.3% | 0.5% | -0.2% |
| All | -0.3% | -0.5% | -0.5% | -0.3% | -0.3% | -0.4% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

In this proposal, a progressive MV resolution (PMVR) method is proposed. In PMVR, MV resolution is not explicitly signaled with specific syntax but progressively adjusted based on the magnitude of MV difference (MVD). Basically, higher MV resolution is employed when absolute value of MVD is small (namely, MV is close to MV predictor). Simulations show that 1.0% luma BD-rate reduction is obtained on average for the four inter cases (RA-HE, RA-LC, LB-HE, and LB-LC). Moreover, average encoding time is saved by 5% while average decoding time is slightly increased by 2%. It is suggested to include PMVR into HEVC.

# References

- Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification ITU-T Rec. H.264/ISO/IEC 14496-10 AVC), Mar. 2003.

- T. Wiegand, W.J. Han, B. Bross, J.-R. Ohm, and G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, Geneva, Switzerland, Mar. 2011.

- M. Karczewicz, P. Chen, R. Joshi, X. Wang, W.-J. Chien, and R. Panchal, “Video coding technology proposal by Qualcomm Inc”, JCT-VC meeting contribution JCTVC-A121, Dresden, DE, April, 2010.

- K. McCann, W. -J. Han and I.-K. Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- E. Alshina, J. Chen, A. Alshin, N. Shlyakhov, and W.-J. Han, “CE3: Experimental results of DCTIF by Samsung”, JCT-VC meeting contribution JCTVC-D344, Daegu, KR, Jan., 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, Switzerland, Mar., 2011.

- K. McCann, B. Bross, S. Sekiguchi, W.-J. Han, “HM3: High Efficiency Video Coding (HEVC) Test Model 3 Encoder Description”, JCTVC-E602, Geneva, Switzerland, Mar., 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex Proposed WD Revisions and Decoder Description

## Semantics in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| cu_qp_delta_enabled_flag | u(1) |
| seq_pmvr_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

seq_pmvr_enabled_flag equal to 1 specifies that PMVR with 1/8-pixel, 1/4-pixel and 1/2-pixel resolution is applied to inter slices.

## Semantics in Slice Header

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| …… |  |
| if (slice_type==P ¦¦ slice_type==B) { |  |
| if(seq_pmvr_enabled_flag) { |  |
| one_mv_thres_for_all_ref_flag | u(1) |
| if (one_mv_thres_for_all_ref_flag) |  |
| slice_pmvr_thres_idx | ue(v) |
| else { |  |
| for (i=0; i<=num_ref_idx_l0_active_minus1; i++) |  |
| slice_pmvr_thres_idx_l0[i] | ue(v) |
| if(slice_type==B) |  |
| for (i=0; i<=num_ref_idx_l1_active_minus1; i++) |  |
| slice_pmvr_thres_idx_l1[i] | ue(v) |
| } |  |
| } |  |
| num_ref_idx_active_override_flag | u(1) |
| …… |  |
| } |  |
| …… |  |
| } |  |

one_mv_thres_for_all_ref_flag equal to 1 specifies all reference pictures of this slice share the same PMVR threshold pair. one_mv_thres_for_all_ref_flag equal to 0 specifies that each reference picture of this slice has its own PMVR threshold pair.

slice_pmvr_thres_idx specifies the PMVR threshold index which is shared by all reference pictures of this slice. The mapping between PMVR threshold index and PMVR threshold pair is presented in Table 8.

Table 8 Indices of PMVR Threshold Pairs

| PMVR Threshold Index | PMVR Threshold Pair (TH1, TH0) |
| --- | --- |
| 0 | (4, 0) |
| 1 | (4, 2) |
| 2 | (0, 0) |

slice_pmvr_thres_idx_l0[i] specifies the PMVR threshold index of reference picture i in reference list 0.

slice_pmvr_thres_idx_l1[i] specifies the PMVR threshold index of reference picture i in reference list 1.

## MVD Decoding

Firstly, The MVDc is decoded by CABAC or CAVLC in the same way as MVD decoding in HEVC. Then MVD8 is derived with Algorithm 3.

Algorithm 3: MVD Decoding

input : MVDc, MVP8, CTR0, CTR1, TH0 and TH1 ouput : MVD8 begin CTRdec_x = (CTR1_x+CTR0_x)/2; CTRdec_y = (CTR1_y+CTR0_y)/2; TH1dec = (TH1+TH0)/2; If ¦MVDc_x-CTR0_x¦<=TH0 and ¦MVDc_y-CTR0_y¦<=TH0, MVD8_x = MVDc_x and MVD8_y= MVDc_y; Else if the ¦MVDc_x-CTRdec_x ¦ <= TH1dec and ¦MVDc_y-CTRdec_y ¦<= TH1dec, If ¦MVDc_x-CTR0_x¦>TH0, If MVDc_x-CTR0_x>=0 MVD8_x = MVDc_x*2-CTR0_x-TH0; Else MVD8_x = MVDc_x*2-CTR0_x+TH0; MVD8_y = MVDc_y*2+CTR0_y; Else If MVDc_y-CTR0_y>=0 MVD8_y = MVDc_y*2-CTR0_y-TH0; Else MVD8_y = MVDc_y*2-CTR0_y+TH0; MVD8_x = MVDc_x*2+CTR0_x; Else if ¦MVDc_x-CTRdec_x¦>TH1dec, if MVDc_x-CTRdec_x>=0, MVD8_x = MVDc_x*4-2(CTR0_x+TH0)-CTR1_x-TH1; Else MVD8_x = MVDc_x*4-2(CTR0_x-TH0)-CTR1_x+TH1; MVD8_y = MVDc_y*4+CTR1_y; Else if MVDc_y-CTRdec_y>=0, MVD8_y = MVDc_y*4-2(CTR0_y+TH0)-CTR1_y-TH1; Else MVD8_y = MVDc_y*4-2(CTR0_y-TH0)-CTR1_y+TH1; MVD8_x = MVDc_x*4+CTR1_x.

End

## MVP Rounding

When both TH1 and TH0 are set to zero, 1/2-pixel resolution is always used for inter mode other than skip and merge. In this case, 1/8-pixel and 1/4-pixel MV positions will only occur due to the MV scaling in temporal skip and merge modes. When such high accuracy MV is taken as a MVP for blocks in inter mode other than skip and merge, the MV has to be converted to 1/2-pixel accuracy as follows.

MVP8_x=(MVP8_x+1)>>2<<2;

MVP8_y=(MVP8_y+1)>>2<<2;

Similarly, when TH0 is set to zero and TH1 is not set to zero, 1/4-pixel resolution is the highest resolution used for inter mode other than skip and merge. In this case, 1/8-pixel MV positions will only occur due to the MV scaling in temporal skip and merge modes. When such high accuracy MV is taken as a MVP for blocks in inter mode other than skip and merge, the MV has to be converted to 1/2-pixel accuracy as follows.

MVP8_x=MVP8_x>>1<<1;

MVP8_y=MVP8_y>>1<<1;

## Interpolation for 1/8-Pixel Resolution

This part of decoding is the same as that in section 3.3.