| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F127 |
| --- | --- |

| Title: | CE9: Cross-check of TI’s study on partial merge JCTVC-F082 (PART18) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Result Report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of test PART18 proposed by Texas Instruments within core experiment 9 evaluating motion vector derivation and skip tools. Test PART18 disables partial merge and restricts the encoder to not use inter prediction for 4x4 partitions. The proposed code has been checked, compiled and the results reported by Texas Instruments can be confirmed.

# Tested Proposal

The tested proposal has been integrated into HEVC HM 3.0 [2] und the partial merge mode was disabled by defining the macro as follows.

#define PART_MRG                          0

The code was checked and the software reflects what is proposed.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [3]. The HM3.0 default settings, both for configuration file and compile-time, are used in this test as anchor. Due to its influence on motion vector coding, a bug in the spatial motion vector scaling process was fixed in the anchor as well as in the tested software (http://hevc.kw.bbc.co.uk/trac/ticket/146).

## Hardware

Two types of computers have been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system. They are listed in Table 2-1.

Table 2-1	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Decoding on 1 core without yuv | 1x4 | 4 x Intel(R) Core(TM)2 Extreme CPU Q9300 @ 2.53GHz | 4 GB |
| 2 | Encoding for test SP12 and HM3.0 anchor to be compared with | 2x6 | 2 x 6 x Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB |

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-F127-PART18.xls.

Table 2-2	Test PART18: Partial merge is disabled and the encoder is restricted not to use inter prediction for 4x4 partitions.

# References

- Yu-Wen Huang, Benjamin Bross, Minhua Zhou, Wei-Jung Chien, Il-Koo Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge operations”, JCTVC-E709, Geneva, March, 2011.

- JCT-VC, “HEVC Test Model Software 3.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.