| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F129 WG11 Number: m20546 |
| --- | --- |

| Title: | CE11: Parallelization of HHI_TRANSFORM_CODING (Fixed Diagonal Scan from C227) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. Madhukar Budagavi Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the use of fixed diagonal scan (C227) for the significance map to enabled parallel processing in conjunction with the use of the highly adaptive context selection for significant_coeff_flag.  While the simplified context selection method in JCTVC-D260 (adopted for HM-2.0), eliminates the neighboring dependency on the most recently decoded significant_coeff_flag within a diagonal, dependencies on the previous position when moving from one scan line to another.  The fixed diagonal scan proposed in this contribution eliminates this dependency.  The fixed diagonal scan was implemented in HM-3.0 and had coding efficiency impact of -0.1%, -0.1% and 0.0% for All Intra, Random Access and Low Delay for the High Efficiency configuration.

# Introduction

For improved coding efficiency, HM-2.0 uses a highly adaptive context selection approach for significant_coeff_flag in the significance map, where the context selection can depend on the value of the coefficients of the neighbouring positions (typically in the left, top-left and top directions) in the transform.  A context selection method was proposed in JCTVC-D260 (adopted for HM-2.0) that eliminates the neighboring dependency to the diagonal neighbours (as shown in Fig. 1), which is typically the most recently decoded significant_coeff_flag.  However, when the processing significant_coeff_flags that are near the edge of the transform, the most recently decoded bin may not be the diagonal neighbour, but the immediate top or left neighbour (as shown in Fig. 2).  To minimize speculative computations for parallel bin decoding, it would be desirable to avoid dependency on the previously decoded bin.

B

E

F

H

I

X

Fig.1 – Simplified context selection for significant map in HM-2.0.  Context for position X depends on B, E, F, H and I.

…

…

Fig.2 – When processing the significant_coeff_flag in positions highlighted in blue (every other position in first row and every other position in first column), there is dependency on previously decoded bin highlighted in grey. These dependencies occur at the edges of the transform.  This dependency will be a challenge for parallel decoding of two bins (or more) at a time.

# Fixed Diagonal Scan

For significant_coeff_flag, rather than using the traditional zig-zag scan, the contribution proposes a fixed diagonal scan approach were all diagonal scans are in the same direction.    The benefit of this is that there are no longer dependencies on the previously decoded bin (Fig. 3)

…

…

Fig.3 – When processing the significant_coeff_flag with fixed diagonal scan, the blue positions at the edge of the transform are no longer dependent on previously decoded bin highlighted in grey.

# Experiment Results

The HM-3.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.2 | -0.2 |
| Overall | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.2 | -0.5 | -0.1 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.2 | -0.1 |
| Class E |  |  |  |
| Overall | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1 | 0.1 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.5 | 0.8 |
| Class E | 0.1 | 0.7 | -0.8 |
| Overall | 0.0 | 0.3 | 0.0 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1 Average loss from changing the significant_coeff_flag scan order from zig-zag to fixed diagonal scan to help enable parallel processing. Positive numbers in the table means coding loss.

# Text for Working Draft

Scanning order array initialisation process

Input to this process is a block size blkSize.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a fixed diagonal scan, scanIdx equal to 1 specifies a horizontal scan and scanIdx equal to 2 specifies a vertical scan. The array index pos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

i  =  blkSize* blkSize

j  = 0  
z =  0
while( z  < j) {
	x  =  z

y =  0
	while( x  <  blkSize ) {
		y++

x--
	}

while( x  >= 0 && y <  blkSize ) {
		ScanOrder[ 0 ][ i ][ 0 ]  = x
		ScanOrder[ 0 ][ i ][ 1 ]  = y

z++

y++

x--
	}
}

i  =  0
y  =  0
while( y  < blkSize ) {
	x  =  0
	while( x  <  blkSize ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
}

i  =  0
x  =  0
while( x  < blkSize ) {
	y  =  0
	while( y  <  blkSize ) {
		ScanOrder[ 2 ][ i ][ 0 ]  = x
		ScanOrder[ 2 ][ i ][ 1 ]  = y
		y++
		i++
	}
	x++
}

# Summary

This contribution proposes the use of a fixed diagonal scan for the significance map in order to eliminate dependencies on the previously decoded significant_coeff_flag.  Eliminating dependency on the most recently decoded bin is useful for parallel processing since it reduces the amount of speculative computations.  Using a fixed diagonal scan instead of zig-zag scan has no coding penalty (a slight gain is observed)

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).