| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F130 WG11 Number: m20547 |
| --- | --- |

| Title: | Parallel Context Processing of Coefficient Level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Bypass bins do not require contexts, and thus multiple bypass bins can be coded in a single cycle for increased throughput.  Grouping the bypass bins together maximized the throughput impact.  In HM-3.0, only the first two bins of the coefficient level are context coded.  This contribution proposes grouping the bypass bins together in the coefficient level syntax elements in order to increase the throughput of the CABAC.  This modification was tested for the high efficiency configuration in HM-3.0 and no coding loss was observed (0.0% for AI-HE, RA-HE and LD-HE).

# Introduction

In E253, which was adopted into HM-3.0, only the first two bins of the coefficient level need to be context coded.  The rest of the bins are bypass coded.  The current ordering of the bins in the coefficient level in HM-3.0 is shown in Fig. 1.

b0:coeff_abs_level_greater1_flag (context coded)

b1: coeff_abs_level_greater2_flag (context coded)

GR-EG: coeff_abs_level_minus3 (bypass coded)

s: coeff_sign_flag (bypass coded)

cu_header is all other syntax elements in CU.

Fig.1. Current bin order for coefficient level in HM-3.0. Bypass coded bins are highlighted in blue.

Since bypass bins do not require context fetching and adaptation, many bypass bins can be coded in a single cycle for higher throughput (e.g. [1], [2]).  Grouping the bypass bins together maximizes the throughput impact.  This contribution proposes grouping the bypass bins as shown in Fig. 2. Specifically, it proposes sending coeff_abs_level_minus and coeff_sign_flag together.

Fig.2. Proposed bin order for coefficient level.  All bypass bins (highlighted in blue) are grouped together to achieve high throughput.

# Experiment Results

The HM-3.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |

Table 1 No change in coding efficiency is observed for parallel context processing of the coefficient level.

# Text for Working Draft

### Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| n++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| } |  |
| numSubsets = max( 1, ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 6 : 4 )) |  |
| for( i = 0; i < numSubsets; i++ ) { |  |
| offset = i << 4 |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ 0 ][ i ][ 0 ] << 2 |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ 0 ][ i ][ 1 ] << 2 |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

# Summary

This contribution proposes grouping the bypass bins the coefficient level together to improve the throughput of the CABAC.  Specifically, it proposes sending coeff_abs_level_minus and coeff_sign_flag together. There is no impact on coding efficiency.

# Reference

- Y.-C. Yang and J.-I. Guo, “High-Throughput H.264/AVC High-Profile CABAC Decoder for HDTV Applications,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 19, no. 9, pp. 1395 –1399, September 2009.

- B. Shi, W. Zheng, H.-S. Lee, D.-X. Li, and M. Zhang, “Pipelined Architecture Design of H.264/AVC CABAC Real-Time Decoding,” in IEEE International Conference on Circuits and Systems for Communications, May 2008, pp. 492 –496.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).