| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F131 WG11 Number: m20548 |
| --- | --- |

| Title: | Modifications to Slice Header termination for low delay encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Akira Osamoto Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-567-4005 sze@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

SAO and ALF are performed in the last stage of video coding; however, SAO and ALF parameters are inserted in the slice header which is before slice data (generated from earlier stages of video coding).  Inconsistency between order of processing and order of data transmission under the current that slice header format implies that at the encoder EPB insertion and CABAC bin encoding must be performed after SAO and ALF which can be an issue for low delay encoders.  This contributions proposes that 1) slice header is byte aligned to enable EPB insertion as bits are being produced; 2) ensure that last byte in slice header is non-zero; this allows slice header and slice data to be easily stitched together without changing slice data; 3) CABAC is terminated after slice header to enable CABAC encoding as syntax elements are produced;. This modification was implemented in HM-3.2 and its coding efficiency was evaluated with ALF on for both HE and LC: 0.0% AI-HE, 0.1% RA-HE, 0.1%, LD-HE, 0.0% AI-LC(ALF on), 0.0% RA-LC (ALF on),  0.0%  LD-LC(ALF on); and ALF off for both HE and LC: 0.0% AI-HE (ALF off), 0.1% RA-HE(ALF off),  0.1%  LD-HE(ALF off),  0.0% AI-LC, 0.0% RA-LC,  0.0%  LD-LC.

# Introduction

In HM-3.0, SAO and ALF are performed in the last stage of video coding; however, SAO and ALF parameters are inserted in the slice header which is before slice data (generated from earlier stages of video coding).  Inconsistency between order of processing and order of data transmission under the current slice heard format can result in issues for EPB insertion and CABAC encodingparticularly if low delay encoding is required.  Since SAO and ALF will insert bits at the front of the slice data, EPB insertion must wait until SAO and ALF processing is complete, since the slice header will affect how EPB is inserted in the slice data.  In the case of CABAC, since SAO and ALF will insert bins at the front of the slice, bin encoding must wait until the SAO and ALF processing is complete.

To address these two issues, this contributions proposes that

1) slice header is byte aligned to enable EPB insertion as bits are being produced

2) ensure that last byte in slice header is non-zero; this allows slice header and slice data to be easily stitched together without changing slice data.  This is similar to rbsp_trailing_bits( ) but after the slice header instead of slice data. Note: Simulations for this contribution use stuffing bits with value ‘one’’; however, stuffing bits of value ‘zero’ can also be used if stop bit of one is inserted before the byte stuffing (i.e. just as with rbsp_trailling_bits.  Coding efficiency should be the same regardless of whether byte stuffing is with ‘ones’ or ‘zeros’.

3) CABAC is terminated after slice header to enable CABAC encoding as syntax elements are produced

# Experiment Results

The HM-3.2 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  |  |  |  |  | HE (ALF off) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  | All Intra HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class E | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Overall | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 1 Average coding efficiency for High Efficiency configuration with ALF on (left) and ALF off (right). Positive numbers in the table means coding loss.

|  |  |  |  |  | LC (ALF on) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra LC |  |  | All Intra LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 101% |  |  |
|  | Random Access LC |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 112% |  |  | #NUM! |  |  |
| Dec Time[%] | 105% |  |  | 100% |  |  |
|  | Low delay B LC |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

Table 2 Average coding efficiency for Low Complexity configuration with ALF off (left) and ALF on (right). Positive numbers in the table means coding loss.  Note: the HM-3.2 anchor encoder crashes for PeopleOnStreet for Random Access LC with ALF on QP22 (bug reported – changeset#1097).  Results are omitted for Class A.

# Text for Working Draft

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ (adaptive_loop_filter_enabled_flag & (adaptive_loop_filter_flag ¦¦!shared_pps_info_enabled_flag) )) |  |
| { |  |
| if( entropy_coding_mode_flag ) |  |
| end_of_slice_header_flag /*Terminate CABAC*/ | f(1) |
| /* Byte Alignment */ |  |
| stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_one_bit /* equal to 1 */ | f(1) |
| } |  |
| } |  |

end_of_slice_header_flag equal to 1 to terminate CABAC.

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 3 |
|  | B |  | 0 | Error: Reference source not found | 6 |
| end_of_slice_flag | all | FL, cMax = 1 | 0 | 0 | 0 |
| end_of_slice_header_flag | all | FL, cMax = 1 | 0 | 0 | 0 |

# Summary

This contribution presents a modification to the slice header termination such that EPB insertion and CABAC encoding does not have to wait for SAO and ALF processing to complete.  This is beneficial for encoders that want to achieve low delay.  The change in slice header termination has at most a 0.1% coding loss in the HE-RA and HE-LD cases.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).