| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F132 WG11 Number: m20549 |
| --- | --- |

| Title: | Reduction in contexts used for significant_coeff_flag and coefficient level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: | sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes removal of several contexts used for CABAC entropy coding of syntax elements significant_coeff_flag coding and coefficient level.  Specifically, the four contexts used for the first four significant_coeff_flag in a 32x32 transforms are removed; accordingly, all contexts for significant_coeff_flag in 16x16 and 32x32 are shared.  The contexts for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are each reduced from 60 to 54 and 60 to 48 respectively. A total of 22 contexts are removed per slice type with negligible impact on coding efficiency in HM-3.2 (0.1 % AI-HE, 0.0% RA-HE, -0.1% LD-HE).

# Introduction

Currently, over 75% of the contexts used in CABAC belong to syntax elements significant_coeff_flag, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag.  To reduce the memory cost, this contribution investigated removal of certain contexts for these syntax elements.

- For significant_coeff_flag, 16x16 and 32x32 luma blocks share all contexts except for position x<=1, y<=1.  This contribution proposes sharing the contexts for x<=1, y<=1 positions as well for a context reduction of 4.

- For coeff_abs_level_greater1_flag, the value of c1 is accumulated based on the trailing ones to a maximum of 4 to select a context.  Tests were run for maximum 0, 1, 2, and 3. This contribution proposes reducing the maximum to 3, for a context reduction of 6.

- For coeff_abs_level_greater2_flag, the value of c1 is accumulated based on the number of coeff_abs_level_greater2_flag to a maximum of 4 to select a context.  Tests were run for maximum 0, 1, 2, and 3. This contribution proposes reducing the maximum to 2, for a context reduction of 12.

The above changes reduce the total number of contexts by 22 for each slice type.

# Experiment Results

The HM-3.2 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

Sharing for x<=1, y<=1 positions of 16x16 and 32x32 blocking in significant_coeff_flag

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | -0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0 | -0.3 | 0.3 |
| Class B | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.2 | -0.1 |
| Class E |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.1 | -0.1 |
| Class D | 0.0 | -0.2 | -0.2 |
| Class E | -0.2 | -0.8 | -0.4 |
| Overall | 0.0 | -0.2 | -0.1 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1 Average loss from sharing contexts for x<=1, y<=1 positions of 16x16 and 32x32 blocking in significant_coeff_flag. Positive numbers in the table means coding loss.

Measurement of coding penalty for different maximums in coeff_abs_level_greater1_flag.

| Maximum | All Intra HE |  |  | Random Access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.3 | -0.3 |
| 2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 | -0.2 | -0.1 |
| 1 | 0.3 | 0.1 | 0.1 | 0.0 | -0.1 | 0.0 | -0.2 | -0.4 | -0.4 |

Table 2 Average loss from reducing maximum in coeff_abs_level_greater1_flag. Positive numbers in the table means coding loss.

Measurement of coding penalty for different maximums in coeff_abs_level_greater2_flag.

| Maximum | All Intra HE |  |  | Random Access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.4 |
| 2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | -0.2 | -0.1 |
| 1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 |
| 0 | 0.3 | 0.3 | 0.3 | 0.2 | 0.1 | 0.3 | 0.1 | 0.1 | -0.1 |

Table 3 Average loss from reducing maximum in coeff_abs_level_greater2 _flag. Positive numbers in the table means coding loss.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1 | 0.1 | 0.0 |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.0 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.1 | 0.1 | 0.1 |
| Overall | 0.1 | 0.1 | 0.0 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1 | -0.7 | 0.0 |
| Class B | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.2 | 0.2 |
| Class E |  |  |  |
| Overall | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | -0.3 | -0.2 |
| Class D | 0.0 | -0.2 | -1.0 |
| Class E | -0.2 | -0.6 | -1.1 |
| Overall | -0.1 | -0.2 | -0.6 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 4 Average loss from removing 22 contexts. Positive numbers in the table means coding loss.

# Text for Working Draft

###### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  min( 3, greater1Ctx )	(9-1)

###### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

…

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

ctxIdxInc  =  ( ctxSet  *  5 )  +  min( 2, greater2Ctx )	(9-2)

# Summary

This contribution proposes the removal of 22 contexts in significant_coeff_flag, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag, which have negligible effect on coding efficiency but helps to reduce memory requirements.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).