| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F133 WG11 Number: m20550 |
| --- | --- |

| Title: | Simplified MVD context selection (Extension of E324) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Anantha P. Chandrakasan Massachusetts Institute of Technology | Email: Email: | sze@ti.com |
|  |  |  | anantha@mit.edu |
| Source: | Texas Instruments, Massachusetts Institute of Technology |  |  |

_____________________________

# Abstract

In contribution E324, the line buffer memory size was reduced by modifying the context selection of MVD.  The number of contexts used for MVD remained the same.  This contribution extends this idea and proposes further reduction to both the contexts and line buffer memory by using a fixed context for binIdx0 of MVD such that it is independent of its neighbors.  This modification was tested for the high efficiency configuration in HM-3.0 and no coding loss was observed (0.0% for AI-HE, RA-HE and LD-HE).

# Introduction

In contribution E324, the line buffer memory size was reduced by modifying the context selection of MVD; specifically, it change the logic for the neighbor dependencies (top & left) by computing the threshold before the summation which reduced the line buffer storage from 6 bits per component per direction to 1 bits per component per direction. The number of contexts used for MVD remained the same.

This contribution extends this idea and proposes further reduction to both the contexts and line buffer memory by using a fixed context for binIdx0 of MVD such that it is independent of its neighbors.   Accordingly, no line buffer is necessary for MVD, and the total number of contexts used is reduced from 7 per component per direction to 5 per component per direction.

# Experiment Results

The HM-3.0 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1 | -0.2 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |
| Overall | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0 | 0.2 | 0.1 |
| Class C | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.1 |
| Class E | 0.0 | -0.2 | 0.2 |
| Overall | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1 Average coding efficiency impact of mvd context selection simplification. Positive numbers in the table means coding loss.

# Text for Working Draft

| Table 9-1 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| mvd_l0[ ][ ][ 0 ] | Error: Reference source not found | 0 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ] | Error: Reference source not found | 5 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found | 10 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found | 15 | 0 | 1 | 2 | 3 | 4 |

# Summary

This contribution proposes an extension of E324 to further reduce memory associated with MVD context selection.  Removing the neighboring dependency for context selection of binIdx0 in MVD has no coding penalty (a slight gain is observed)

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

To the best of our personal awareness TI does not have patent rights to the content of the document.