| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F134 |
| --- | --- |

| Title: | CE11: Cross checking of JCTVC-C227 and proposal on semantic, syntax, and implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

The cross-checked results of CE11-C227-TI matched for HE test cases. However, the software being cross-checked resulted in coding losses for LC. This contribution provided semantics, syntax, and alternate implementations with the same coding performance as CE11-C227-TI for HE and without coding loss for LC.

# Introduction

JCTVC-C227 [1, 3] proposed a fixed diagonal scan (instead of zig-zag scan) in order to enable parallel processing in conjunction with the use of the highly adaptive context selection for significance map.  Within a given transform, all diagonals will scan in the same direction, either always up-right or always down-left.

As a part of the CE11 Sub Experiment CE.A [2], the source code CE11-C227-TI was distributed for cross-checking. It implemented up-right scan for all block sizes. It was cross checked officially by Qualcomm and Institute for Infocomm Research for the High Efficiency test cases. Sony is not an official cross-checker of CE11-C227-TI in CE11.A.

Although Low Complexity test cases are not parts of the CE11.A, this contribution provides additional information on the BD performance of the source code CE11-C227-TI for Low Complexity test cases.

This contribution also proposes syntax, semantics, and implementation of up-right scan based on the work at Sony and TI’s contribution JCTVC-C227 [1] prior to the code release of CE11-C227-TI for both High Efficiency and Low Complexity test cases.

Figure 1: Zig-zag scan is replaced by up-right scan in CE11-C227-TI for all block sizes.

# Cross-checking results

As showed in Figure 1, the source code CE11-C227-TI replaced the zig-zag scan in HM3.0 by up-right scan for the coding of significance map with CABAC for all block sizes. As a side effect of the implementation, the source code also replaced the zig-zag scan by up-right scan for coding transform coefficients with CAVLC.

As shown in Table 1, for HE test cases, the CE11-C227-TI resulted in BD-Rate of -0.08% for intra, -0.08%  for random access, and -0.02% for low delay. The HE results reported in this contribution matched the results provided by TI for CE11.A.

Although CE11.A does not require the testing of LC, this contribution also provides results for LC. For LC test cases, the CE11-C227-TI resulted in BD-Rate of -0.09% for intra, 0.1% for random access, and 0.45% for low delay.

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.11 | -0.23 | -0.20 | -0.30 | -0.03 | 0.01 |
| Class B | -0.08 | -0.12 | -0.10 | -0.15 | 0.11 | 0.21 |
| Class C | -0.06 | -0.03 | 0.01 | 0.00 | 0.24 | 0.18 |
| Class D | -0.04 | -0.03 | -0.04 | 0.00 | 0.24 | 0.24 |
| Class E | -0.12 | -0.22 | -0.17 | 0.07 | 0.26 | 0.27 |
| Overall | -0.08 | -0.12 | -0.10 | -0.09 | 0.15 | 0.18 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.22 | -0.53 | -0.12 | -0.18 | 0.14 | 0.24 |
| Class B | -0.10 | -0.14 | -0.09 | 0.14 | 0.34 | 0.41 |
| Class C | -0.01 | 0.14 | 0.11 | 0.26 | 0.36 | 0.24 |
| Class D | 0.00 | 0.15 | -0.07 | 0.29 | 0.15 | 0.37 |
| Class E |  |  |  |  |  |  |
| Overall | -0.08 | -0.10 | -0.04 | 0.13 | 0.25 | 0.32 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.09 | 0.08 | -0.15 | 0.34 | 0.19 | 0.39 |
| Class C | -0.03 | -0.09 | -0.13 | 0.44 | 0.55 | 0.27 |
| Class D | -0.02 | 0.52 | 0.81 | 0.42 | 0.35 | 0.20 |
| Class E | 0.11 | 0.75 | -0.85 | 0.67 | 0.78 | -0.96 |
| Overall | -0.02 | 0.27 | -0.03 | 0.45 | 0.43 | 0.06 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 1: BD-rate of CE11-C227-TI

# Additional results relating to JCTVC-C227

One of the purposes of JCTVC-C227 is to improve parallel processing for the coding of significance map with CABAC. A sufficient condition to improve parallel processing with CABAC is to only replace zig-zag scan for coding of 16x16 and 32x32 blocks by up-diagonal scan and keep the zig-zag scan with 4x4 and 8x8 blocks.  As shown in Table 2, when up-right scan applied only to 16x16 and 32x32 blocks for improving parallel processing, the BD-Rate dropped to -0.02% for intra, -0.06%  for random access, and -0.02% for low delay. Consequently, for better coding performance and parallel processing, up-right scan should be applied to all block sizes.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.06 | -0.10 | -0.05 |
| Class B | -0.03 | -0.05 | -0.03 |
| Class C | -0.01 | 0.01 | 0.06 |
| Class D | 0.00 | -0.02 | -0.02 |
| Class E | -0.02 | -0.09 | -0.06 |
| All | -0.02 | -0.05 | -0.02 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.14 | 0.07 | -0.50 |
| Class B | -0.05 | 0.00 | 0.02 |
| Class C | -0.02 | 0.21 | -0.14 |
| Class D | -0.04 | 0.16 | -0.04 |
| Class E |  |  |  |
| All | -0.06 | 0.11 | -0.15 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.03 | 0.09 | 0.03 |
| Class C | -0.02 | 0.01 | -0.03 |
| Class D | -0.06 | 0.40 | 0.18 |
| Class E | 0.06 | 0.12 | 0.00 |
| All | -0.02 | 0.15 | 0.05 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 2: BD-Rate for HE when up-right scan was applied only to 16x16 and 32x32 blocks.

# Proposed semantics

This contribution proposed changes to the WD and HM software to map scanIdx based on HE or LC. In particular, this contribution proposed to replace the text from section 6.6 of JCTVC-E603_d5 by the following text:

## 6.7 	Scanning order array initialisation process

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a zig-zag scan as specified in subclause Error: Reference source not found, scanIdx equal to 1 specifies a horizontal scan, scanIdx equal to 2 specifies a vertical scan, and scanIdx equal to 3 specifies an up-right scan in subclause 6.6. The array index pos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = ZigZag

ScanOrder[3] = UpRight

i  =  0
y  =  0
while( y  < blkSize ) {
	x  =  0
	while( x  <  blkSize ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
}

i  =  0
x  =  0
while( x  < blkSize ) {
	y  =  0
	while( y  <  blkSize ) {
		ScanOrder[ 2 ][ i ][ 0 ]  = x
		ScanOrder[ 2 ][ i ][ 1 ]  = y
		y++
		i++
	}
	x++
}

This contribution also proposes the following addition:

## 6.6 	Up-right scanning array initialisation process

Input to this process is a block size blkSize.

Output of this process is the array UpRight[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array UpRight is derived as follows.

UpRight[ 0 ][ 0 ]  =  0
UpRight[ 0 ][ 1 ]  =  0
i  =  1
x  =  0
y  =  1
stopLoop  =  ( blkSize  = = 1 ) ? true : false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if (x  >=  0  &&  x  <  blkSize  &&  y  >=  0  &&  y  <  blkSize ) {
			UpRight[ i ][ 0 ]  =  x
			UpRight[ i ][ 1 ]  =  y
			i++
		}

y--

x++
	}

y = x
	x = 0
	if ( i  >=  blkSize  *  blkSize ) 
		stopLoop  =  true
}

# Proposed Syntax

This contribution proposed the syntax in Table 3 to support up-right scan for HE and zig-zag scan for LC. In particular, the function mapZZtoUR( scanIdx )  in Table 3 is defined as:

mapZZtoUR( scanIdx ) = .

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| if( !entropy_coding_mode_flag && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if ( cIdx = = 0 ) { |  |
| cbp_luma[ trafoDepth + 1 ] | vlc(n,v) |
| cbf_luma[ x0 ][ y0 ][ trafoDepth + 1 ] = cbp_luma[ trafoDepth + 1 ] >> 3 |  |
| cbf_luma[ x1 ][ y0 ][ trafoDepth + 1 ] = cbp_luma[ trafoDepth+1 ] >> 2 |  |
| cbf_luma[ x0 ][ y1 ][ trafoDepth + 1 ] = cbp_luma[ trafoDepth + 1 ] >> 1 |  |
| cbf_luma[ x1 ][ y1 ][ trafoDepth + 1 ] = cbp_luma[ trafoDepth + 1 ] |  |
| } else if( cIdx = = 1 ) { |  |
| cbp_cb[ trafoDepth + 1 ] | vlc(n,v) |
| cbf_cb[ x0 ][ y0 ][ trafoDepth + 1 ] = cbp_cb[ trafoDepth + 1 ] >> 3 |  |
| cbf_cb[ x1 ][ y0 ][ trafoDepth + 1 ] = cbp_cb[ trafoDepth+1 ] >> 2 |  |
| cbf_cb[ x0 ][ y1 ][ trafoDepth + 1 ] = cbp_cb[ trafoDepth + 1 ] >> 1 |  |
| cbf_cb[ x1 ][ y1 ][ trafoDepth + 1 ] = cbp_cb[ trafoDepth + 1 ] |  |
| } else if( cIdx = = 2 ) { |  |
| cbp_cr[ trafoDepth + 1 ] | vlc(n,v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth + 1 ] = cbp_cr[ trafoDepth + 1 ] >> 3 |  |
| cbf_cr[ x1 ][ y0 ][ trafoDepth + 1 ] = cbp_cr[ trafoDepth+1 ] >> 2 |  |
| cbf_cr[ x0 ][ y1 ][ trafoDepth + 1 ] = cbp_cr[ trafoDepth + 1 ] >> 1 |  |
| cbf_cr[ x1 ][ y1 ][ trafoDepth + 1 ] = cbp_cr[ trafoDepth + 1 ] |  |
| } |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, mapZZtoUR(scanIdx), cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

Table 3: Proposed syntax to remap zig-zag scan to up-right scan for HE.

# Software and cross-checking

Software was developed with the syntax and semantics described in this contribution prior to the code release of CE11-C227-TI. This software was distributed to TI for cross-checking after their code release. The following is the BD-Rate of this software. In particular, the BD-Rate performance for HE is the same as the performance of CE11-C227-TI, and the BD-Rate performance for LC is the same as the anchor HM3.0.

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.11 | -0.23 | -0.20 | 0.00 | 0.00 | 0.00 |
| Class B | -0.08 | -0.12 | -0.10 | 0.00 | 0.00 | 0.00 |
| Class C | -0.06 | -0.03 | 0.01 | 0.00 | 0.00 | 0.00 |
| Class D | -0.04 | -0.03 | -0.04 | 0.00 | 0.00 | 0.00 |
| Class E | -0.12 | -0.22 | -0.17 | 0.00 | 0.00 | 0.00 |
| Overall | -0.08 | -0.12 | -0.10 | 0.00 | 0.00 | 0.00 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.22 | -0.53 | -0.12 | 0.00 | 0.00 | 0.00 |
| Class B | -0.10 | -0.14 | -0.09 | 0.00 | 0.00 | 0.00 |
| Class C | -0.01 | 0.14 | 0.11 | 0.00 | 0.00 | 0.00 |
| Class D | 0.00 | 0.15 | -0.07 | 0.00 | 0.00 | 0.00 |
| Class E |  |  |  |  |  |  |
| Overall | -0.08 | -0.10 | -0.04 | 0.00 | 0.00 | 0.00 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.09 | 0.08 | -0.15 | 0.00 | 0.00 | 0.00 |
| Class C | -0.03 | -0.09 | -0.13 | 0.00 | 0.00 | 0.00 |
| Class D | -0.02 | 0.52 | 0.81 | 0.00 | 0.00 | 0.00 |
| Class E | 0.11 | 0.75 | -0.85 | 0.00 | 0.00 | 0.00 |
| Overall | -0.02 | 0.27 | -0.03 | 0.00 | 0.00 | 0.00 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 4: BD-Rate of the software from which that this contribution is based on.

# Conclusions

This contribution provided the semantic, syntax, and software implementation to support up-right scan for HE and zig-zag scan for LC. Since the up-right scan provides coding gain and improves parallel processing with CABAC, this contribution recommends that the up-right scan from CE11-C227-TI and the semantics and syntax from this contribution to be adopted as a part of HM4. This contribution also recommends JCT-VC to adopt the software implementation associated with this contribution as a part of HM4 software.

# References

[1]	V. Sze and M. Budagavi, “Parallelization of HHI_TRANSFORM_CODING”, JCTVC-C227, 3rd Meeting: Guangzhou, CN, 07-14 October, 2010.

[2]	V. Sze, T. Nguyen, J. Chen, J. Sole, and K. Panusopone, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-E711, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3]  V. Sze and M. Budagavi, “Parallelization of HHI_TRANSFORM_CODING (Fixed Diagonal Scan from C227)”, JCTVC-F129, 6th Meeting: Torino, IT, 14-22 July, 2011’

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).