| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F135 WG11 Number: m20553 |
| --- | --- |

| Title: | Analysis of Multi-core Processing approaches |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. Madhukar Budagavi Texas Instruments Inc. Minhua Zhou Texas Instruments Inc. | Email: Email: Email: | sze@ti.com madhukar@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Low power and high frame rate/resolution requirements for future video coding applications make the need for parallelism in the video codec implementation ever more important.   Several methods have been proposed to enable high-level parallelism on multi-core architectures.  This contribution begins by describing the differences between regular slices, entropy slices, interleaved entropy slices/wavefront parallel processing and Tiles.  It then provides an analysis of these tools based on throughput, coding efficiency, implementation complexity and latency.

# Introduction

Parallelism can be used to achieve the high processing speed that is necessary to deliver high resolution and frame rates.  At the same time, power consumption is becoming increasingly important as more video codecs are found in battery-operated devices.  An effective method of reducing power consumption is to increase throughput (via parallelism) and then trade-off of the processing speed for power savings with voltage scaling.  Thus the amount of parallelism extends beyond the minimum necessary to achieve a given frame rate or resolution.  In other words, if it takes a minimum of N parallel cores to achieve 4kx2k @ 60fps, for power reduction, it may be desirable to have 2N cores running at half speed and a lower voltage.  Accordingly, parallel processing can be used to address both the power and performance requirements of future video codecs.  One of the key throughput bottlenecks in the video codec is entropy coding.   In particular, parallelism is difficult to achieve with the existing AVC CABAC due to its inherent serial nature.

Various forms of parallelism have been proposed for HEVC at the bin, syntax element, and slice level; for each form, various methods have been proposed.  For instance, for slice level parallelism Regular Slices (i.e. slices used in AVC), Entropy Slices [1][2], Ordered Entropy Slices [3][4], Interleaved Entropy Slices [5][6]/Wavefront Parallel Processing [7] and Tiles [8] have been proposed for HEVC.  It is important to understand the impact of the various parallel processing methods in terms of

- Throughput

- Coding efficiency

- Implementation complexity  (e.g. memory bandwidth)

- Latency

This contribution begins by describing the differences between regular slices, entropy slices, interleaved entropy slices/wavefront parallel processing and tiles.  It then provides an analysis of these tools based on the above metrics.  Please see enclosed slides for details.

# References

- A. Segall and J. Zhao, “Entropy slices for parallel entropy decoding,” ITU-T SGI 6/Q.6 Doc. COM16-C405, Geneva, CH, April 2008.

- A. Segall, T. Yamamoto, J. Zhao, Y. Kitaura, Y. Yasugi, and T. Ikai, “Video coding technology proposal by Sharp,” JCTVC-A105, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- Xun Guo, Yu-Wen Huang, and Shawmin Lei “Ordered Entropy Slices for Parallel CABAC,” ITU-T Q.6/SG16 VCEG, VCEG-AK25, Yokohama, Japan, 15-18 April, 2009.

- Yu-Wen Huang, Xun Guo, Chih-Ming Fu, Yu-Pao Tsai and Shawmin Lei “Ordered Entropy Slices for Parallel CABAC,” ITU-T Q.6/SG16 VCEG, VCEG-AL25, Geneva, July 2009.

- Daniel F. Finchelstein, Vivienne Sze, and Anantha P. Chandrakasan, “Multi-Core Processing and Efficient On-Chip Caching for H.264 and Future Video Decoders”, IEEE Transactions on Circuits and Systems for Video Technology, November 2009.

- V. Sze. M. Budagavi, and A. P. Chandrakasan, "Massively Parallel CABAC," ITU-T Q.6/SG16 VCEG, VCEG-AL21, Geneva, July 2009.

- F. Henry, S. Pateux, “Wavefront Parallel Processing,” JCTVC-E196, Geneva, March 2011.

- A. Fuldseth, M. Horowitz, M. Zhou, “Tiles,” JCTVC-E408, Geneva, March 2011.

- M. Budagavi, V. Sze, M. U. Demircin, S. Dikbas, M. Zhou, and A. P. Chandrakasan, “Video coding technology proposal by Texas Instruments,” JCTVC-A101, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- TK Tan, G. Sullivan, and T. Wedi, "Recommended Simulation Common Conditions for Coding Efficiency Experiments Revision 1," ITU–T Standardization Sector, Document VCEG-AE010, January 2007.

- G. Bjøntegaard, "Calculation of Average PSNR Differences between RD curves", 
VCEG-M33, Austin, TX, USA, April 2001.

- Stéphane Pateux and Joel Jung, “An Excel Add-In for Computing Bjøntegaard Metric and Its Evolution,” ITU-T SG16 Q.6 Document, VCEG-AE07, Marrakech, January 2007.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).