| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F138 WG11 Number: m20556 |
| --- | --- |

| Title: | CE7: Tool 1 - Mode dependent intra residual coding for 8x8 blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Peisong Chen, Marta Karczewicz (Qualcomm) Akiyuki Tanizawa, Jun Yamaguchi (TOSHIBA) Chuohao Yeo, Yih Han Tan, Zhengguo Li (Institute for Infocomm Research) | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com akiyuki.tanizawa@toshiba.co.jp chyeo@i2r.a-star.edu.sg |
| Source: | Qualcomm, TOSHIBA, Institute for Infocomm Research |  |  |

_____________________________

# Abstract

For intra predicted blocks, the prediction residual is first transformed and then, the quantized transform coefficients are scanned and entropy coded. By selecting the transform based on intra prediction modes, higher coding efficiency can be realized with some complexity increase. Such a scheme, for 4x4 blocks, with the additional transform being the ODST-3 is already adopted in HM3. In this contribution, an 8-point derived KLT is proposed as the additional transform for mode-dependent transform coding of 8x8 intra blocks. The average BD-Rate metrics are -0.4% and -0.2% for Intra HE/LC respectively, and -0.2% and -0.1% for Random Access HE/LC respectively.

# Mode dependent intra residual coding for 8x8 blocks

The same mode-to-transform mapping in HM3 is used. For an 8x8 block, when a DST is signaled to be used as the transform, we would use a derived KLT from JCTVC-B024 [1]. In particular, when an image correlation parameter of ρ is used, i.e., Εxkxl=ρ|k−l, then, the (i,j) entries of the covariance matrix along the dominant prediction direction can be expressed as:

Σij=1+ρ|i−j|−ρ|i|−ρ|j|

When ρ → 1, the eigenvectors of the covariance matrix are that of the Odd Type-3 DST (ODST-3). However, for large block sizes, such an assumption might not be appropriate. Therefore, for 8x8 blocks, we propose using a smaller value of ρ, i.e., ρ = 0.65, such that the resulting KLT, after scaling by 128√2 to be compatible with the E243 core transforms, would be:

{   26,   46,   59,   68,   74,   76,   75,   71, },

{   49,   81,   84,   56,    8,  -41,  -75,  -79, },

{   69,   77,   17,  -61,  -91,  -48,   32,   81, },

{   85,   43,  -62,  -72,   28,   87,   16,  -78, },

{   86,  -21,  -83,   36,   73,  -55,  -64,   66, },

{   78,  -70,  -13,   83,  -62,  -25,   86,  -53, },

{   58,  -87,   71,  -19,  -43,   82,  -80,   36, },

{   31,  -58,   78,  -87,   84,  -71,   48,  -19, },

In contrast, the 8-point ODST-3 is:

{   16,   32,   46,   59,   70,   79,   84,   87, },

{   46,   79,   87,   70,   32,  -16,  -59,  -84, },

{   70,   84,   32,  -46,  -87,  -59,   16,   79, },

{   84,   46,  -59,  -79,   16,   87,   32,  -70, },

{   87,  -16,  -84,   32,   79,  -46,  -70,   59, },

{   79,  -70,  -16,   84,  -59,  -32,   87,  -46, },

{   59,  -87,   70,  -16,  -46,   84,  -79,   32, },

{   32,  -59,   79,  -87,   84,  -70,   46,  -16, },

Comparing the two transform matrices, we see that the first basis function of the proposed 8-point KLT has a more gradual slope.

As mentioned above, the same intra prediction mode-to-transform mapping in HM3 is used. In other words, whenever the alternative transform is specified to be used for a particular intra prediction mode along a direction (horizontal or vertical), the proposed 8-point KLT would be applied instead of the DCT.

Since there are no known fast methods for this 8-point KLT, a matrix multiply implementation is used.

# Experimental results

As specified by the CE7 conditions [2], coding efficiency is evaluated in Intra and Random Access (RA) configurations for both the high efficiency and low complexity cases based on the common conditions defined in JCTVC-E700 [3]. The coding efficiency is measured by using 4 point BD-Rate.

## Compression performance

The experimental results when the proposed 8-point transform is used are shown in Table 1.

Table 1. BD-Rate metrics for proposed 8-point transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8 | 0.2 | -0.2 | -0.4 | -0.3 | -0.5 |
| Class B | -0.4 | 0.0 | 0.1 | -0.1 | -0.4 | -0.4 |
| Class C | -0.3 | -0.1 | 0.0 | -0.3 | -0.3 | -0.3 |
| Class D | -0.2 | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 |
| Class E | -0.4 | 0.4 | 0.1 | -0.2 | -0.1 | -0.3 |
| Overall | -0.4 | 0.1 | 0.0 | -0.2 | -0.3 | -0.4 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | 0.3 | -0.4 | -0.2 | -0.1 | -0.2 |
| Class B | -0.3 | 0.2 | 0.3 | -0.1 | 0.0 | -0.1 |
| Class C | -0.2 | 0.1 | -0.1 | -0.1 | -0.2 | -0.2 |
| Class D | -0.1 | 0.0 | -0.2 | -0.1 | -0.1 | -0.3 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | 0.2 | -0.1 | -0.1 | -0.1 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

We also show additional results when this derived 8-point additional transform is used with SDIP. In the following experiment, the anchor is SDIP with 4-point DST used for all UIP and SDIP modes, while the test is the anchor with the 8-point additional transform used for all UIP and SDIP modes. Table 2 below shows the results for the All-Intra coding configuration for both HE and LC.

Table 2. SDIP+DST4 vs SDIP+DST4+KLT8

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8 | 0.3 | -0.1 | -0.4 | -0.4 | -0.6 |
| Class B | -0.5 | 0.0 | 0.1 | -0.3 | -0.4 | -0.4 |
| Class C | -0.3 | 0.0 | 0.0 | -0.4 | -0.3 | -0.3 |
| Class D | -0.3 | -0.1 | -0.1 | -0.4 | -0.3 | -0.3 |
| Class E | -0.5 | 0.6 | 0.4 | -0.5 | 0.3 | 0.0 |
| Overall | -0.5 | 0.1 | 0.1 | -0.4 | -0.2 | -0.3 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

## Complexity

A summary is presented in the table below. This also includes the operations needed to complete the de-quantization process.

| Transform size |  | HM 2 core 2-D transform | HM3 partial butterfly implementation of 2-D inverse DCT | Proposed 8-point transform |
| --- | --- | --- | --- | --- |
| 8x8 | Mults | 0 | 352 | 1024 |
|  | Adds | 640 | 576 | 1024 |
|  | Shifts | 352 | 128 | 128 |

## Bit precision

No change from the bit precision used in HM3 is required.

# Conclusions

In this CE report, we have presented results for the proposed mode-dependent transforms for 8x8 intra coded blocks using DCT and a derived KLT that show coding gains with small complexity increase. We recommend that these tools be considered for adoption into HM.

# Reference

[1] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” in JCTVC-B024, Geneva, Switzerland, July 2010.

[2] R. Cohen et al., “Description of Core Experiment 7: Additional Transforms,” JCTVC-E707, Geneva, Switzerland, Mar. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Switzerland, Jan 2011.

# Patent rights declaration(s)

Qualcomm, TOSHIBA and Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex – Draft text

Add highlighted parts to Section 8.5.4.2, “Transformation process for 8 samples”, as follows:

Inputs of this process are an array of 8 samples of the scaled transform coefficients x with elements xi, with i = 0..7 and the transform type variable trType.

Output of this process is an array of 8 samples of the residual samples y with elements yi, with i = 0..7.

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- The output values yi with i = 0..7 are then specified as follows:

y0 = ( 26*x0 + 49*x1 + 69*x2 + 85*x3 + 86*x4 + 78*x5 + 58*x6 + 31*x7 + 64 ) >> 7		
y1 = ( 46*x0 + 81*x1 + 77*x2 + 43*x3 – 21*x4 – 70*x5 – 87*x6 – 58*x7 + 64 ) >> 7		
y2 = ( 59*x0 + 84*x1 + 17*x2 – 62*x3 – 83*x4 – 13*x5 + 71*x6 + 78*x7 + 64 ) >> 7		
y3 = ( 68*x0 + 56*x1 – 61*x2 – 72*x3 + 36*x4 + 83*x5 – 19*x6 – 87*x7 + 64 ) >> 7		
y4 = ( 74*x0 +  8*x1 – 91*x2 + 28*x3 + 73*x4 – 62*x5 – 43*x6 + 84*x7 + 64 ) >> 7		
y5 = ( 76*x0 – 41*x1 – 48*x2 + 87*x3 – 55*x4 – 25*x5 + 82*x6 – 71*x7 + 64 ) >> 7		
y6 = ( 75*x0 – 75*x1 + 32*x2 + 16*x3 – 64*x4 + 86*x5 – 80*x6 + 48*x7 + 64 ) >> 7		
y7 = ( 71*x0 – 79*x1 + 81*x2 – 78*x3 + 66*x4 – 53*x5 + 36*x6 – 19*x7 + 64 ) >> 7

–	Otherwise, the following ordered steps apply: [8-point DCT]