| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F141 WG11 Number: m20559 |
| --- | --- |

| Title: | CE7: Cross-check of Samsung’s proposed Tools 2 (F282) and 6 (F283) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Hui Li Tan Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by the Institute for Infocomm Research on Samsung’s proposed Tool 2 (8-point DST for intra mode-dependent residual coding) and Tool 6 (the “(28,56)” 4-point DST) in CE7 on additional transforms. The cross-check rate-distortion performance results match those provided by Samsung. The implemented algorithm is also in line with Samsung’s proposal.

# Introduction

The 4-point ODST-3 has been adopted in HM3 for mode-dependent transform coding of intra prediction residuals [1] [2]. CE7 was set up to test the use of 8-point DSTs for intra mode-dependent residual coding as well as slight adjustments of the 4-point DST matrix [3].

## Samsung Tool 2

Samsung’s Tool 2 is on the use of the 8-point DST. The transform matrix was computed using the following equation:

Ki,j=22N+1sin(2i−1)jπ2N+1

To match the scaling used in the core transforms that was adopted in the HM3 software [4], the transform matrix shown above should be scaled by 128 √2. The resulting 8x8 transform matrix is therefore:

{    16,    32,    46,    59,    70,    79,    84,    87,},

{    46,    79,    87,    70,    32,   -16,   -59,   -84,},

{    70,    84,    32,   -46,   -87,   -59,    16,    79,},

{    84,    46,   -59,   -79,    16,    87,    32,   -70,},

{    87,   -16,   -84,    32,    79,   -46,   -70,    59,},

{    79,   -70,   -16,    84,   -59,   -32,    87,   -46,},

{    59,   -87,    70,   -16,   -46,    84,   -79,    32,},

{    32,   -59,    79,   -87,    84,   -70,    46,   -16,},

The implementation of this transform is done using a matrix multiplication.

## Samsung Tool 6

Samsung’s Tool 6 is on adjusting some of the coefficients of the 4-point DST to enable a faster implementation of the transform. The 4-point DST transform is denoted as “(29,55)”, and has the following transform matrix:

{29,   55,    74,   84},

{74,   74,    0 ,  -74},

{84,  -29,   -74,   55},

{55,  -84,    74,  -29},

The proposed transform in Tool 6 is denoted as “(28,56)”, and has the following transform matrix:

{28,   56,    74,   84},

{74,   74,    0 ,  -74},

{84,  -28,   -74,   56},

{56,  -84,    74,  -28},

Essentially, all the entries with absolute values of 29 are changed to 28, and all the entries with absolute values of 55 are changed to 55. By doing so, the number of operations can be decreased from 11 additions and 8 multiplications as implemented in HM3 to 11 additions, 5 multiplications and 3 shifts as implemented in Tool 6, ignoring the operations needed to complete the descaling after dequantization.

# Simulation Results

The test conditions as specified in CE7 [3] are followed. The implementation is on HM 3.0. Tests were conducted on Intra and Random Access configurations for both high efficiency and low-complexity settings [5].

## RD results

Table 1 summarizes the BD-Rate [6] results for Samsung’s Tool 2.

Table 1. BD-Rate results for Tool 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.0 | -0.3 | -0.2 | -0.4 | -0.6 |
| Class B | -0.1 | -0.2 | -0.1 | 0.2 | -0.6 | -0.6 |
| Class C | 0.0 | -0.2 | -0.2 | -0.1 | -0.4 | -0.4 |
| Class D | 0.0 | -0.2 | -0.2 | 0.0 | -0.3 | -0.3 |
| Class E | -0.2 | 0.0 | -0.3 | 0.0 | -0.4 | -0.6 |
| Overall | -0.2 | -0.1 | -0.2 | 0.0 | -0.4 | -0.5 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | 0.1 | -0.4 | -0.1 | -0.2 | -0.4 |
| Class B | -0.2 | 0.3 | 0.3 | 0.1 | -0.1 | -0.2 |
| Class C | -0.1 | 0.1 | -0.1 | -0.1 | -0.2 | -0.3 |
| Class D | -0.1 | -0.1 | -0.2 | -0.1 | -0.4 | -0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | 0.1 | -0.1 | 0.0 | -0.2 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 2 summarizes the BD-Rate results for Samsung’s Tool 6.

Table 2. BD-Rate results for Tool 6

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | -0.4 | 0.1 | -0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.2 | 0.0 | -0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Following the test conditions in CE10, we also ran the same set of tests but at a set of low QP values (1, 5, 9, 13) and high QP values (36, 42, 47, 51) to compare the performance of (28,56) vs (29,55) 4-point DST. This was done for the all-intra configurations only, and cross-checked by Toshiba in JCTVC-F329.

Table 3. BD-Rate results for Tool 6 at low QP values

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.3 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class D | 0.5 | 0.1 | 0.1 | 0.2 | 0.0 | 0.0 |
| Class E | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 |
| Overall | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 4. BD-Rate results for Tool 6 at high QP values

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class C | -0.1 | 0.2 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -4.3 | 0.2 | 0.0 | 0.3 | -0.2 |
| Class E | 0.0 | -0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | -0.8 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Futher discussion of Tool 6

We note that the (29,55) 4-point DST can actually be implemented with a smaller number of operations  compared to the current implementation in HM3. In fact, in JCTVC-B024, we have already shown an implementation that can implement the mathematically equivalent transform using just 10 additions and 8 multiplications [7]. However, we can do even better, using just 10 additions and 6 multiplications. Alternatively, it can also be implemented with 13 additions, 4 multiplications and 2 bit-shifts. This is further elaborated in a related contribution [8]. Similarly, Tool 6 can also be implemented with just 10 additions, 5 multiplications and 1 bit-shifts. Alternatively, it can also be implemented with 11 additions, 4 multiplications and 2 bit-shifts.

Therefore, we would argue that the (28,56) 4-point DST does not have any significant complexity advantage over the (29,55) 4-point DST.

# Conclusion

We have cross-checked Samsung’s Tool 2 and Tool 6 in CE7, studied the algorithm and verified their coding simulation results. Furthermore, we also discussed the relative complexity between the (29,55) and (28,56) 4-point DST.

# References

[1] A. Saxena, and F. C. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” in JCTVC-E125, Geneva, Switzerland, Mar. 2011.

[2] R. Joshi, P. Chen, M. Karczewicz, A. Tanizawa, J. Yamaguchi, C. Yeo, Y. H. Tan, H. Yang, and H. Yu, “CE7: Mode dependent intra residual coding,” in JCTVC-E098, Geneva, Switzerland, Mar. 2011.

[3] R. Cohen et al., “Description of Core Experiment 7: Additional Transforms,” JCTVC-E707, Geneva, Switzerland, Mar. 2011.

[4] A. Fuldseth, G. Bjøntegaard, M. Sadafale, and M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, Geneva, Switzerland, Mar 2011.

[5] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, Geneva, Switzerland, Jan 2011.

[6] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

[7] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” in JCTVC-B024, Geneva, Switzerland, July 2010.

[8] C. Yeo, Y. H. Tan and Z. G. Li, “On fast implementation of 4-point ODST-3 in HM3,” in JCTVC-F153, Torino, Italy, July 2011.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.