| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F142 |
| --- | --- |

| Title: | Improvement on derivation process for luma motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Satoshi Shimada, Kimihiko Kazui, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 sshimada@jp.fujitsu.com kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes two modifications of the derivation process for luma motion vector prediction in WD3.0[1]. Firstly, the rounding process in the equations for scaling motion vector is slightly modified. Secondly, the derivation process for temporal luma motion vector prediction is modified so that a motion vector predictor is chosen among two motion vectors in a co-located picture.

The coding gain of the proposed scheme for BQSquare sequence is up to 2.0%. The average coding gain for Class D sequences is 0.9%. The overall coding gains for random access configurations and low delay configurations are 0.5% and 0.1%, respectively. Additional computational complexity both in encoding and decoding is negligible.

# Proposed modification

## Rounding process for scaling motion vector

### Overview

The scaling process for mvLXZ (with Z is being A or B) in Subclause 8.4.2.1.5 is defined as follows.

mvLXZ = ClipMv( ( DistScaleFactor * mvLXZ + 128 ) >> 8 )

The rounding direction in the above equation is toward positive infinity.

The proposed rounding direction is toward zero. Moreover, the rounding offset is slightly reduced.

### Proposed change

The following equation is applied for mvLXA and mvLXB in subclause 8.4.2.1.5, and mvLXCol in subclause 8.4.2.1.6.

mvLXZ = 
  ClipMv( Sign( DistScaleFactor * mvLZ ) * (( Abs( DistScaleFactor * mvLXZ ) + 127 ) >> 8) )

(Z being A or B or Col)

## Derivation process for temporal luma motion vector prediction

### Overview

Subclause 8.4.2.1.6 of WD3.0 defines that colPu is located at lower-right side of the current PU's position in the collocated picture. If colPu is not available, colPU', which is located at the center of the current PU's position in the collocated picture, is used instead of colPu (See Figure 1).

Figure 1　Definition of ColPu in HM3.0

The proposed scheme chooses colPu among PuCandA and PuCandB in Figure 2 according to the evaluation criteria described later.

Figure 2　Definition of ColPu in proposed scheme

The positions of PuCandA and PuCandB are defined according to the position of CenterPu (illustrated in Figure 2) relative to the grid of 16x16 CU (See Figure 3).

Please note that motion vector information of a reference picture is compressed to 1/16 (i.e. the minimum size of CU which hold motion vector information is 16x16) in WD3.0. And the representative motion vector of the 16x16 CU area is the motion vector of the PU which covers the most upper-left part of the 16x16 CU area.

Figure 3　Position of PuCandA and PuCandB according to the position of CenterPu relative to the grid of 16x16 CU

The evaluation criteria for choosing colPu among PuCandA and PuCandB are the following.

- Firstly, the candidate motion vector MvColX of PuCandX (X being replaced by either A or B) is derived. MvColX is equal to (MvCol * scalingFactor), where MvCol is the motion vector derived by the method described in subclause 8.4.2.1.6, and scalingFactor is the scaling value calculated from the distance in POC between the current picture and its collocated picture.

- Then the distance LX between CenterPU and the cross point of the motion vector MvColX emitted from PuCandX to the current picture is calculated. Specifically, LX is equal to Abs(xPColX + MvColX[ 0 ] - xPCenter) + Abs(yPColX + MvColX[ 1 ] - yPCenter), where (xPColX, yPColX) is the positions of the top-left luma sample of PuCandX relative to the top-left sample of the current picture, and (xPCenter, yPCenter) is the positions of the top-left luma sample of CenterPu relative to the top-left sample of the current picture.

- If LA is equal or smaller than LB, colPu is PuCandA. Otherwise, colPu is PuCandB.

### Proposed change

Subclause 8.4.2.1.6 is modified as follows.

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

The position ( xPColA, yPColA ) and the position ( xPColB, yPColB ) are derived in the order of following steps:

- Central luma position of the current prediction unit is defined by

xPCtr = xP + ( nPSW >> 1) 								(3-1)

yPCtr = yP + ( nPSH >> 1)								(3-2)

- xPColA, yPColA, xPColA, and yPColB are derived by referring Table 3-1.

Table 3-1 – xPColA, yPColA, xPColB, and yPColB associated to xPCtr and yPCtr

| xPCtr % 16 | yPCtr % 16 | xPColA | yPColA | xPColB | yPColB |
| --- | --- | --- | --- | --- | --- |
| [0, 7] | [0, 7] | (xPCtr >> 4) * 16 | (yPCtr >> 4) * 16 | (xPCtr >> 4) * 16 + 16 | (yPCtr >> 4) * 16 + 16 |
| [8, 15] | [0, 7] | (yPCtr >> 4) * 16 + 16 | (yPCtr >> 4) * 16 | (xPCtr >> 4) * 16 + 16 | (yPCtr >> 4) * 16 + 16 |
| [0, 7] | [8, 15] | (xPCtr >> 4) * 16 | (yPCtr >> 4) * 16 + 16 | (xPCtr >> 4) * 16 + 16 | (yPCtr >> 4) * 16 + 16 |
| [8, 15] | [8, 15] | (yPCtr >> 4) * 16 + 16 | (yPCtr >> 4) * 16 + 16 | (xPCtr >> 4) * 16 | (yPCtr >> 4) * 16 |

For Z being replaced by either A or B in the variables mvLXColZ, availableFlagLXColZ, xPColZ, and yPColZ, the following is specified.

The variables mvColZ, refIdxColZ, listColZ and availableFlagLXColZ are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvColZ are set equal to 0 and availableFlagLXColZ is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvColZ and refIdxColZ are derived as follows:

- If PredFlagL0[ xPColZ ][ yPColZ ] is equal to 0, the motion vector mvColZ and the reference index refIdxColZ are set equal to MvL1[ xPColZ ][ yPColZ ] and RefIdxL1[ xPColZ ][ yPColZ], respectively.

- Otherwise (PredFlagL0[ xPColZ ][ yPColZ ] is equal to 1), the following applies.

  - If PredFlagL1[ xPColZ ][ yPColZ ] is equal to 0, the motion vector mvColZ and the reference index refIdxColZ are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPColZ ][ yPColZ ], respectively.

  - Otherwise (PredFlagL1[ xPColZ ][ yPColZ ] is equal to 1), the following applies.

    - The following asignments are made with X being 0 or 1.

- RefIdxColLX = RefIdxLX[ xPColZ ][ yPColZ ]

- If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

  - MvXCross = 1

- Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

  - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvColZ, the reference index refIdxColZ and ListColZ are set equal to MvL1[ xPColZ ][ yPColZ ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvColZ, the reference index refIdxColZ and ListColZ are set equal to MvL0[ xPColZ ][ yPColZ ], RefIdxColL0 and L0, respectively.

And the variable availableFlagLXColZ is set equal to 1 and the following applies.

For Z being replaced by either A or B in the variables mvLXColZ, and availableFlagLXColZ, the following is specified.

The variable mvLXColZ is derived as follows.

- If availableFlagLXColZ is equal to 0, no further process is taken.

- Otherwise (availableFlagLXColZ is equal to 1), the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColZ, ListColZ ) is equal to PicOrderCnt( colPic ) - PicOrderCnt( currPic ),

mvLXColZ = mvColZ								(3-3)

  - Otherwise, mvLXColZ is derived as scaled version of the motion vector mvColZ as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td							(3-4)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )					(3-5)

mvLXColZ = ClipMv									
( Sign( DistScaleFactor * mvColZ ) *(( Abs( DistScaleFactor * mvColZ ) + 127 ) >> 8) )	(3-6)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxColZ, ListColZ ) )	(3-7)

tb = Clip3( -128, 127, PicOrderCnt( colPic ) – PicOrderCnt( currPic )				(3-8)

The variable distZ is derived as follows.

distZ = Abs(xPColZ + xMvLXColZ - xPCtr) + Abs(yPColZ + yMvLXColZ - yPCtr)  		(3-9)

,where xMvLXColZ and yMvLXColZ are the horizontal component of mvLXColZ and the vertical component of mvLXColZ, respectively.

mvLXColNoScale and availableFlagLXCol are derive as follows.

- If availableFlagLXColA and availableFlagLXColB are both equal to 0, availableFlagLXCol is set equal to 0.

- Otherwise, if availableFlagLXColA is equal to 1, and availableFlagLXColB is equal to 0 or distA <= distB, mvLXColNoScale, availableFlagLXCol, refIdxCol, and ListCol are set to mvLXColA, availableFlagLXColA, refIdxColA, and ListColA, respectively.

- Otherwise, mvLXColNoScale, availableFlagLXCol, refIdxCol, and ListCol are set to mvLXColB, availableFlagLXColB, refIdxColB, and ListColB, respectively.

mvLXCol is derived as follows.

- If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvLXColNoScale								(3-10)

- Otherwise, mvLXCol is derived as scaled version of the motion vector mvLXColNoScale as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td							(3-11)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )					(3-12)

mvLXCol = ClipMv										
( Sign( DistScaleFactor * mvLXColNoScale ) *(( Abs( DistScaleFactor * mvLXColNoScale ) + 127 ) >> 8) )	(3-13)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )		(3-14)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )		(3-15)

# Simulation result

Table 2 summarizes the coding gain in BD-Bitrate of the proposed scheme over HM3.0. The test condition accords with the condition specified in JCTVC-E700 [2].

Table 2: Summary results of BD-Rate comparing Proposal to HM3.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.2 | 0.2 | -0.1 | 0.0 | 0.1 |
| Class B | -0.4 | -0.2 | -0.1 | -0.4 | -0.1 | -0.1 |
| Class C | -0.6 | -0.3 | -0.3 | -0.5 | -0.3 | -0.4 |
| Class D | -0.9 | -0.7 | -0.8 | -0.9 | -0.6 | -0.5 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.3 | -0.2 | -0.5 | -0.3 | -0.2 |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | 0.2 | 0.0 | -0.1 | -0.4 |
| Class C | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | -0.1 | 0.6 | 0.0 | 0.0 | -0.1 |
| Class E | -0.1 | 0.3 | -0.5 | -0.3 | -0.6 | -0.8 |
| Overall | -0.1 | 0.0 | 0.1 | -0.1 | -0.2 | -0.4 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

The coding gain of the proposed scheme for BQSquare sequence is up to 2.0%. The average coding gain for Class D sequences is 0.9%. The overall coding gains for random access configurations and low delay configurations are 0.5% and 0.1%, respectively. Additional computational complexity both in encoding and decoding is negligible.

The measured computational complexity for encoding is 99% - 102% for various configurations. The measured computational complexity for decoding is 95% - 97% for various configurations.

In addition, coding gain in BD-Bitrate of the first modification, "rounding process for scaling motion vector" over HM3.0 is measured. The result is summarized in Table 3.

Table 3: Summary results of BD-Rate comparing first modification to HM3.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.3 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class B | -0.2 | -0.2 | -0.1 | -0.3 | 0.0 | 0.0 |
| Class C | -0.4 | -0.2 | -0.1 | -0.4 | -0.1 | -0.3 |
| Class D | -0.8 | -0.3 | -0.4 | -0.8 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -0.2 | -0.2 | -0.4 | -0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | -0.1 | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | -0.1 | 0.2 | 0.2 | 0.0 | -0.1 | 0.1 |
| Class E | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 |
| Overall | -0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

The coding gains of "rounding process for scaling motion vector" scheme are 0.4% on average for random access configurations and 0.0-0.1% on average for low delay configurations.

The proposed scheme (both the first modification and the second modification) was cross-verified with  Mitsubishi (JCTVC-F183)[3] and got the same BD-Bitrate gain and the similar computational complexity.

# Conclusion

The coding gains of the proposed scheme over HM3.0 are 0.5% on average for random access configurations and 0.1% on average for low delay configurations. Increase of computational complexity is negligible.

We request to adopt the proposed scheme into new version of HEVC test model.

References

[1] "WD3: Working Draft of High-Efficiency Video Coding", JCTVC-E602, March 2011, Geneva CH

[2] "Common test conditions and software reference configurations", JCTVC-E700, March 2011, Geneva CH

[3] K.Sugimoto, A.Minezawa, and S.Sekiguchi, "Cross-verification report on Fujitsu's proposal (JCTVC-F142) on AMVP", JCTVC-F183, July 2011, Torino IT

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).