| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F143_r1 WG11 Number: m20561 |
| --- | --- |

| Title: | CE12: Deblocking filter parameter adjustment in slice level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Saori Asaka1 Takashi Watanabe1 Takeshi Chujoh1 Marta Karczewicz2 In Suk Chong2 | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | Toshiba Corporation1 and Qualcomm2 |  |  |

_____________________________

# Abstract

Appropriate parameters for deblocking filter to improve coding efficiency for CE12 are presented.  Modification of offset to intra tc, a new offset to tc is introduced in slice level syntax and a default value to the offset to tc are proposed.  Coding efficiency improvements are 0.7% for All Intra and 0.3 % for Random Access and 0.1% for Low delay B/P cases for High Efficiency configuration and 0.5% for All Intra and 0.2 % for Random Access and 0.0% for Low delay B/P cases for Low Complexity configuration.  Results are verified by Ericsson.

# Introduction

At the Geneva meeting in Mar. 2011, a contribution (JCTVC-E193[1]) reported that there is a possibility to optimize deblocking filter strength.  The contribution modified  and tc values and obtained coding efficiency improvements of 0.5% to 0.7%.  However, there was a comment that modifying  value might affect subjective picture quality for some sequences.

In this contribution, modification of offset to intra tc, a new offset to tc is introduced in slice level syntax and a default value to the offset to tc are proposed.  Evaluation was conducted based on JCTVC-E700[2] and JCTVC-E712[3].  Coding efficiency improvements are 0.7% for All Intra and 0.3 % for Random Access and 0.1% for Low delay B/P cases for High Efficiency configuration and 0.5% for All Intra and 0.2 % for Random Access and 0.0% for Low delay B/P cases for Low Complexity configuration.  Results are verified by Ericsson.

# Modification points from WD 3

Since the current WD3 and HM3 software does not support interface to offset tc, a syntax and an interface to support offsetting are introduced in slice level.  In addition, when decoding intra blocks, a Qp value is increased by 4 when referring tc table in WD3.  In this contribution, it is recommended to modify from 4 to 3.  Furthermore, the default offset for tc is recommended to -1.

There is no additional modification to the current WD3 and HM3 software other than offsetting tc (e.g. number of coefs., filter shapes, encoding technique, etc.)  The followings are the modification points of WD3 (highlighted in yellow).

7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| First_tb_in_slice | ue(v) |
| ………… |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| slice_tc_offset |  |
| } |  |
| } |  |
| } |  |

    - Slice header semantics

(Between “slice_qp_delta” and “collocated_from_l0_flag”)

slice_tc_offset specifies an offset to QPY when obtaining a tc value.

      - Filtering process for coding unit

(Replace the corresponding sentences as follows:)

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter ( qPL + 3 + tC_Offset) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter (qPL + tC_Offset) as input.

8.6.1.4.3	Filtering process for chroma block edge

(Replace the corresponding sentences as follows:)

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter ( qPL + 3 + tC_Offset) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter (qPL + tC_Offset) as input.

# Experimental results

Experiments are conducted based on the test condition JCTVC-E700 [2] and JCTVC-E608 [3].  Tc offset -1 is recommended.  Tables 1 and 2 show experimental results for High Efficiency and Low Complexity cases, respectively.  Coding efficiency improvement is 0.7% for All Intra case and 0.5 % for Random Access and Low delay cases with no impact of encoding and decoding time.  Regarding the subjective picture quality, an informal viewing was conducted by using CG-301W by the proponent, in particular Vidyo3 whose subjective picture quality was pointed out slightly worse than anchor.  Ini addition, several sequences that were used for informal subjective viewing in CE12 were used to compare the anchor and the proposal.  Viewing results are similar to those of CE12, which means that there is not a significant difference between the anchor and the proposal.

Table 1  Results for High Efficiency configuration

| High Efficiency |  |  | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | -1.0 | -0.8 |  |  |
|  | S02 | PeopleOnStreet | -1.1 | -0.3 |  |  |
|  | Sxx | Nebuta | -0.7 | -0.2 |  |  |
|  | Syy | SteamLocomotive | -0.8 | -0.6 |  |  |
| B | S03 | Kimono | -0.6 | -0.2 | 0.0 | 0.0 |
|  | S04 | ParkScene | -1.2 | -0.8 | -0.2 | -0.1 |
|  | S05 | Cactus | -0.8 | -0.4 | -0.1 | -0.1 |
|  | S06 | BasketballDrive | -0.7 | -0.2 | 0.0 | 0.0 |
|  | S07 | BQTerrace | -0.6 | -0.3 | -0.1 | 0.0 |
| C | S08 | BasketballDrill | -0.7 | -0.3 | -0.2 | -0.1 |
|  | S09 | BQMall | -0.6 | -0.2 | 0.0 | 0.0 |
|  | S10 | PartyScene | -0.3 | -0.2 | -0.1 | 0.0 |
|  | S11 | RaceHorses | -0.5 | -0.3 | -0.1 | -0.2 |
| D | S12 | BasketballPass | -0.6 | -0.1 | -0.2 | -0.1 |
|  | S13 | BQSquare | -0.2 | -0.2 | -0.2 | 0.0 |
|  | S14 | BlowingBubbles | -0.4 | -0.2 | 0.0 | -0.2 |
|  | S15 | RaceHorses | -0.7 | -0.3 | 0.0 | 0.1 |
| E | S16 | Vidyo1 | -0.7 |  | 0.0 | -0.1 |
|  | S17 | Vidyo3 | -0.7 |  | 0.0 | -0.2 |
|  | S18 | Vidyo4 | -0.8 |  | 0.1 | -0.3 |
| Class A |  |  | -0.9 | -0.5 |  |  |
| Class B |  |  | -0.8 | -0.4 | -0.1 | -0.1 |
| Class C |  |  | -0.5 | -0.2 | -0.1 | -0.1 |
| Class D |  |  | -0.5 | -0.2 | -0.1 | -0.1 |
| Class E |  |  | -0.7 |  | 0.0 | -0.2 |
| Total |  |  | -0.7 | -0.3 | -0.1 | -0.1 |
| Encoding time |  |  | 100% | 100% | 100% | 100% |
| Decoding time |  |  | 100% | 99% | 99% | 99% |

Table 2  Results for Low Complexity configuration

| Low Complexity | AI | RA | LDB | LDP |
| --- | --- | --- | --- | --- |
| Class A | -0.6 | -0.2 |  |  |
| Class B | -0.6 | -0.3 | 0.0 | 0.1 |
| Class C | -0.5 | -0.2 | 0.0 | 0.0 |
| Class D | -0.5 | -0.2 | -0.1 | 0.0 |
| Class E | -0.5 |  | -0.1 | 0.0 |
| Total | -0.5 | -0.2 | 0.0 | 0.0 |
| Encoding time | 100% | 100% | 99% | 100% |
| Decoding time | 99% | 100% | 100% | 100% |

# Conclusion

In this contribution, it is shown that adjusting tc offset improves coding efficiency 0.7% for All Intra and 0.3 % for Random Access and 0.1% for Low delay B/P cases for High Efficiency configuration and 0.5% for All Intra and 0.2 % for Random Access and 0.0% for Low delay B/P cases for Low Complexity configuration with negligible impact on subjective picture quality.  It is recommended to introduce modification of offset to intra tc, a new offset to tc is introduced in slice level syntax and a default value to the offset to tc.

# References

[1]	T. Yamakage, S. Asaka, T. Watanabe, T. Chujoh, C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I.S. Chong, “CE8.1: Deblocking parameter adjustment for 1-input ALF”, JCTVC-E193, March 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011.

[3]	A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of CE12: deblocking filtering”, JCTVC-E712, March 2011.

# Patent rights declaration(s)

Toshiba Corporation and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).