| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F145 |
| --- | --- |

| Title: | CE9: Cross-verification result of Subtest SP07 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Satoshi Shimada, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This document reports cross-verification result of subtest SP07 (originally proposed in JCTVC-E064 [1], the report from the proponent is JCTVC-F169 [3]) in core experiment 9 evaluating motion vector coding and Skip/Merge operations [2].

The result is identical to the result reported by the proponent.

# Verified scheme

The purpose of subtest SP07 is to evaluate the scheme which is described in section 3 of JCTVC-E064 proposed by Mitsubishi. Specifically, the evaluated scheme excludes the temporal motion vector predictor from the list of motion vector predictor candidates if the size of PU is 4x4. This restriction is applied both to AMVP mode and Merge/Skip mode.

We confirmed that the algorithm implemented in the source code for SP07 is identical to that in the core experiment document.

# Result

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | -0.2 | 0.4 | 0.0 | -0.1 | -0.3 |
| Class E | -0.1 | -0.2 | -0.2 | 0.0 | -0.5 | 0.6 |
| Overall | 0.0 | -0.1 | 0.1 | 0.0 | -0.2 | 0.0 |
| Enc Time[%] | 102% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

The result is identical to the result reported from the proponent (Mitsubishi).

References

[1] Y.Itani, S.Sekiguchi, T.Murakami and K.Asai, "Improvement to AMVP/Merge process", JCTVC-E064, March 2011, Geneva CH

[2] "Description of Core Experiment 9: MV Coding and Skip/Merge Operations", JCTVC-E709, March 2011, Geneva CH

[3] Y.Itani, and S.Sekiguchi, "CE9: Temporal vector restriction for small PUs (SP07/SP09/SP12)", JCTVC-F169, July 2011, Torino IT