| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F146 |
| --- | --- |

| Title: | Cross-verification result of JCTVC-F172 proposed by Mitsubishi |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Satoshi Shimada, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This document reports the cross-verification result of JCTVC-F172 proposed by Mitsubishi [1]. JCTVC-F172 proposes to modify the intra sample prediction process especially for Intra_Vertical prediction mode and Intra_Horizontal prediction mode.

The result is identical to the result reported from the proponent (Mitsubishi).

# Verified scheme

The proposed modification is applied to the intra sample prediction process for Intra_Vertical prediction mode and Intra_Horizontal prediction mode. This modification is only applied to luma component and a PU whose size is smaller than the pre-defined value LOWER_LIMIT_OF_DISABLE_BL_SIZE.

The modification for Intra_Vertical prediction mode is given as the following.

WD3.0

predSamples[ x, y ] = p[ x, -1 ], with x, y = 0..nS-1

JCTVC-F172

predSamples[ x, y ] = p[ x, -1 ], with x = 1..nS-1, y = 0..nS-1

predSamples[ x, y ] = clip( p[ x, -1 ] + ( p[ -1, y ] - p[ -1, -1 ]) / 2 ), with x = 0, y = 0..nS-1

The modification for Intra_Horizontal prediction mode is given as the following.

WD3.0

predSamples[ x, y ] = p[ -1, y ], with x, y = 0..nS-1

JCTVC-F172

predSamples[ x, y ] = p[ -1, y ], with x = 0..nS-1, y = 1..nS-1

predSamples[ x, y ] = clip( p[ -1, y ] + ( p[ x, -1 ] - p [ -1, -1 ] ) / 2 ), with x = 0..nS-1, y = 0

In this cross-verification, two tests are performed.

Test 1: LOWER_LIMIT_OF_DISABLE_BL_SIZE = 16

Test 2: LOWER_LIMIT_OF_DISABLE_BL_SIZE = 32

# Simulation result

Table 1 and Table 2 summarize the coding gain in BD-Bitrate of the scheme proposed in JCTVC-F172 over HM3.0. The test condition accords with the condition specified in JCTVC-E700 [2].

Table 1: Summary results of BD-Rate comparing Proposal with LOWER_LIMIT_OF_DISABLE_BL_SIZE = 16 to HM3.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.2 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class B | -0.2 | -0.1 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class D | -0.3 | -0.2 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class E | -0.4 | -0.3 | -0.3 | -0.4 | -0.2 | -0.3 |
| Overall | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Summary results of BD-Rate comparing Proposal with LOWER_LIMIT_OF_DISABLE_BL_SIZE = 32 to HM3.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class B | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Class D | -0.3 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class E | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.4 |
| Overall | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

The result is identical to the result reported from the proponent (Mitsubishi).

References

[1] A.Minezawa, K.Sugimoto, and S.Sekiguchi, "An improved intra vertical and horizontal prediction", JCTVC-F172, July 2011, Torino IT

[2] "Common test conditions and software reference configurations", JCTVC-E700, March 2011, Geneva CH