| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F148 |
| --- | --- |

| Title: | Inter modes for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Xiaoyin Che Wenpeng Ding Yunhui Shi Baocai Yin | Tel: Email | wpding@bjut.edu.cn syhzm@bjut.edu.cn ybc@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

The proposed technique in this contribution is aimed to simplify contexts using in coefficients entropy coding, including three parts, last_flag, one_flag and abs_flag. All tests about this contribution is based on HM3.0. The reduction of the used contexts is reported as -34.6% (from 104 to 68) for last_flag, and -50.0% (from 60 to 30) for both one_flag and abs_flag. The BD-Rate for the high efficiency intra, random access and low delay configurations is reported as 0.1%, 0.1% and 0.0% with all three parts, and 0.1%, 0.0% and 0.0% for only one_flag and abs_flag, respectively.

# Introduction

In HM3.0 high efficiency tools, lots of contexts has been used in coefficients entropy coding. The detailed statistics are showed in Table 1, and “last”, “one” and “abs” represent last_flag, one_flag and abs_flag, respectively. After research on these contexts, some of them are found with similar probabilities when using. So they could probably be combined.

Table 1 : Number of used context size for HM3.0 in coefficients coding (high efficiency)

|  | last | one | abs | all |
| --- | --- | --- | --- | --- |
| intra | 52 | 30 | 30 | 162 |
| random access | 104 | 60 | 60 | 324 |
| low delay | 104 | 60 | 60 | 324 |

# Proposed Solution

In coefficients entropy coding in HM3.0, a whole set of contexts are used for intra frames only, and for inter frames, another set of contexts of same size are used. To simplify the contexts of coefficients entropy coding, new context assignment are proposed below, and all the simplification on intra frames contexts set and inter frames contexts set are absolutely same.

last_flag

Originally, the position of the last_flag in the block is coded by explicitly signaling its (X, Y)-coordinates, and there are 26 different contexts (26 for intra frames and another 26 for inter frames) are used on each X or Y. Proposed method decrease the number from 26 to 17, for both X and Y. Details showed in Table 2.

Table 2 : Context simplification on last_flag

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 0 | 2 | 3 | 5 | 5 | 6 | 7 | 8 |
| Original | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| Proposed | 9 | 10 | 10 | 13 | 13 | 13 | 15 | 16 | 17 |
| Original | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |  |
| Proposed | 19 | 19 | 21 | 21 | 21 | 23 | 25 | 25 |  |

one_flag

The 30 contexts using for one_flag (30 for intra and another 30 for inter) can be divided into 6 groups, according to the stats showed by previous blocks, with 5 contexts respectively. After simplification, only half contexts have been retained. Details showed in Table 3.

Table 3 : Context simplification on one_flag

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Original | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| Proposed | 10 | 11 | 12 | 13 | 14 | 0 | 3 | 3 | 3 | 4 |
| Original | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 0 | 4 | 4 | 4 | 5 | 0 | 1 | 2 | 3 | 4 |

abs_flag

The proposed solution on abs_flag is quite similar with one_flag . Details showed in Table 4.

Table 4 : Context simplification on abs_flag

| Original | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proposed | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| Original | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| Proposed | 10 | 11 | 12 | 13 | 14 | 0 | 7 | 1 | 2 | 3 |
| Original | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| Proposed | 7 | 8 | 2 | 2 | 4 | 0 | 1 | 2 | 3 | 4 |

After all, the whole simplification proposal is showed in Table 5 below.

Table 5 : Numbers of context simpification for coefficients coding

|  | last flag | one flag | abs flag | All |
| --- | --- | --- | --- | --- |
| Original | 104 | 60 | 60 | 324 |
| Proposed | 68 | 30 | 30 | 194 |
| Reduced | 36 | 30 | 30 | 130 |
| Reduce Ratio | 34.6% | 50.0% | 50.0% | 40.1% |

# Results

The proposed method’s performance relative to the HM3.0 software. The computing platform is Intel(R) Xeon(R) CPU X5570 @2.93GHz, Microsoft Windows Server 2003 R2 Enterprise x64 Edition Service Pack 2. Table 6 and Table 7 show the test result with all three parts context simplification and with only One_Flag and Abs_Flag. .

Table 6 : Test result with Last, One and Abs Flags

|  | Intra |  |  |  |  | Random access |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  | 0.3 | 0.2 | 0.2 | 0.2 | -0.1 | 0.3 |  |  |  |
| Class B |  | 0.1 | 0.1 | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 | -0.3 | -0.5 |
| Class C |  | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | -0.6 | -0.6 |
| Class D |  | 0.0 | 0.0 | 0.0 | 0.1 | -0.2 | -0.2 | -0.1 | -0.5 | -0.8 |
| Class E |  | 0.1 | 0.1 | 0.1 |  |  |  | 0.1 | -0.5 | 0.4 |
| All | 0.1 |  | 0.1 | 0.1 | 0.1 | -0.1 | 0.0 | 0.0 | -0.5 | -0.4 |
| Enc Time[%] | 101% |  |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  |  | 98% |  |  | 98% |  |  |

Table 7 : Test result with only One and Abs Flags

|  | Intra |  |  |  | Random access |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 | -0.4 |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 | -0.4 | -0.5 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 | 0.0 | -0.4 | -0.3 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | -0.4 | -0.5 |
| Class E | 0.0 | 0.0 | 0.1 |  |  |  | 0.1 | 0.3 | -1.2 |
| All | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 | -0.3 | -0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |

# Patent rights declaration(s)

Beijing University of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)