| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F150 |
| --- | --- |

| Title: | Inter modes for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Weijia Zhu Wenpeng Ding Yunhui Shi Baocai Yin | Tel: Email | sparkjj@emails.bjut.edu.cn wpding@bjut.edu.cn syhzm@bjut.edu.cn ybc@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

This contribution introduces two inter coding tools (InterBCIM and InterRSQ) for screen content coding. The two new inter coding tools are implemented into the HM2.0 software. The experimental results show that the bitrate saving tested on screen content is 20% or 3.36 dB improvement on average.

# Introduction

Screen content video is a combination of texts, graphics, and natural image. Unlike natural images/videos, screen content videos usually have different properties. The edge of text or graphics contents having complicated shape is much sharper. So predictive coding for them is more difficult than natural images/videos. This leads to the variance of input signals of DCT transform is so large that the residual signals cannot compactly represent in DCT domain. After DCT transform and quantization, there are still many non-zero coefficients. So the conventional video coding framework is not suitable for coding screen content videos.

Consequently, researchers introduces two intra coding tools in JVTVC-B084 [1], JVTVC-C276 [2],and JCTVC-E145 [3]in order to improve the performance of coding screen content videos. Two intra coding tools are RSQ and BCIM, respectively. In the RSQ mode, the prediction residue skips the transform process and is directly quantized. Without transform process, the intra prediction is done in pixels rather than blocks for improving the precision of intra prediction. And the BCIM mode makes use of the fact that text or graphic content usually has limited colors but complicated shapes. It decomposes current block into several base colors together with an index map in spatial domain. The reconstruction work of decoder-side is based on this information.

Although the RSQ and BCIM mode achieves significant coding gains in intra coding, the coding gains is much less in inter coding in that the correlation of consecutive frames is not considered. So we propose two inter coding tools to improve the performance of inter coding of screen content videos, InterRSQ and InterBCIM. We will describe them in the following sections, respectively.

# InterRSQ

Now that the text or graphic content is not suitable for transform coding, our method skip the DCT transform and perform scalar quantization on residues. After motion estimation and motion compensation, the residues are usually smaller than the case in intra coding. So the proposed method tends to achieve better performance than RSQ. The Fig.1 illustrates the process of InterRSQ. For each transform unit, two ways are available to process inter prediction residue. One way is the conventional transform coding, such as DCT transform. Another is the residual scalar quantization.  It skips the transform process and directly performs quantization. One bit is coded for each TU to represent which way is performed. The method of quantization is similar to APEC [4] used in KTA inter block coding.

Fig.1 The illustration of InterRSQ

# InterBCIM

Text or graphic content usually has a property that the histogram of them is much sparser. So the BCIM mode utilizes this property to decompose the text or graphic block to into several base colors together with an index map in spatial domain. The base colors can be obtained through the clustering method. The index map represents the pixels within current block are close to which base color. In here, being close is described by Euclidean distance. The Fig.2 describes this method. But the BCIM can be further improved through reducing the bits that represents the index map and several base colors.

We use the same method as described above to get the base colors and index map of the prediction block. But we specially emphasizes that the index map of prediction block represents that the pixels are close to which base color of current block. Then we compare the index map of current block to the index map of prediction block. If the two corresponding elements in respective index map are identical, only one bit is transmitted to the decoder-side. If one element is different from another, extra bits representing the current index are transmitted to the decoder-side. In here, corresponding means the two elements have identical location. The Fig.3 describes this method. Though the index map of current block are identically same with the index map of prediction block, this method still saves bit significantly. We simply calculate the cost of bits without the consideration of context. If we adopt the equal length coding to coding the index map, it costs 16 2=32bit. However, we can reduce this number to 20(14 1+ 3 2) through our method. The degree of reduction can be aggravated in larger blocks. After many times observations, we find that the probability that two corresponding elements in respective index map are identical is high. In order to achieve better performance, we design 16 contexts for the bit that represents two corresponding elements is identical or not. We observe whether the neighboring indices of current pixel are equal to the corresponding indices in the prediction block. The meaning of neighboring is described in Fig.4. The number of contexts is 16 in that there are four neighboring indices in all and each of them has two possibilities. For the case that needs extra bits to represent the current index, another 15 contexts are designed. The method of designing is described as follow. There are 15 basic patterns of the neighboring indexes, as {AAAA, AAAB, AABA, ABAA, BAAA, AABB, ABAB, ABBA, AABC, BAAC, BCAA, ABAC, ABCA, BACA, ABCD}, where A, B, C and D stand for indices and have been sorted by occurrences. We also remap the current index to 0, 1, 2 or 3 when the current index equals to A, B, C or D before the entropy encoding.

We use the similar idea for reduce the bits that represent the several base colors. In the above paragraph, we get the several base colors of prediction block and current block. The general method of transmission of several base colors is equal length coding. But it can be further improved through variable length coding. In here, we refer the idea of DPCM. We transmit the residue of base colors using Columbus code rather than the values of base colors using equal length coding. The prediction values are the value of several base colors of prediction block. For example, represents the several base colors of current block, represents the several base colors of prediction block, and  represents the residues of several base colors of current block. We note that several base colors of prediction blocks and current block are sort by ascending order before they are used to produce the residues in here.

But we consider the case that the prediction block does not match the current coding block well

sometimes. So we reserve the equal length coding method for coding the base colors, and code one bit to represent which way is adaptively adopted.

Fig.2 Text or graphic block composed of several base colors and an index map

Fig.3 The illustration of coding of index map in InterBCIM

Fig.4 The neighboring indices of the current index.

# Experimental results

We integrate the InterRSQ, InterBCIM modes into HM2.0 software. InterBCIM is applied for luminance component and chrominance component. InterRSQ is applied for luminance component only. In order to illustrate the robustness of our new modes, we test three sequences: slide_1280x720_500_20, web_1280x720_500_10, and doc_1280x720_500_10. The configure file is encoder_lowdelay.cfg. But we turn off the loop-filter so as to achieve better coding efficiency.

The Experimental results are calculated based on [5] and are exhibited in Table I. From Table I, we see that the proposed two modes improve HM2.0 significantly. Fig 4, Fig 5 and Fig 6 show the RD-curve of the experimental results.

TableⅠ.  The simulation results (reference software is HM2.0 standard, test software is HM2.0 software adding our two modes)

Fig 4 the RD-curve of web_1280x720_500_10

Fig 5 the RD-curve of slide_1280x720_500_20

Fig 6 the RD-curve of doc_1280x720_500_10

# Conclusions

In this document, we propose two inter coding tools for coding screen content to improve the coding performance of HM2.0. The simulation results show that the proposed coding tools achieve 20% bit saving or 3.36db improvement on average.

# References

[1] C. Lan, J. Xu, F. Wu and G. Sullivan, “Screen content coding,”  JCT-VC Doc JCT-B084, Geneva, CH, July, 2010.

[2] C. Lan, J. Xu and F. Wu, “Screen content coding results using TMuC,” JCT-VC Doc JCT-C276, Guangzhou, CN, Oct. 2010.

[3] C. Lan, X. Peng, J. Xu and F. Wu, “Intra and inter coding tools for screen contents,” JCT-VC Doc JCT-E145, Geneva, CH, 16-23 March, 2011

[4] M. Narroschke, H. G. Musmann, “Adaptive prediction error coding in spatial and frequency domain with a fixed scan in the spatial domain,” ITU-T SG16/Q6/VCEG-AB06, Bangkok, TH, Jan. 2006

[5]Gisle Bjontegarrd, "Calculation of Average PSNR Differences between RD curves," ITU-T SC16/Q6, 13th VCEG   Meeting, Austin, Texas, USA, Apr. 2001, Doc. VCEG-M33.

# Patent rights declaration(s)

Beijing university of technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).