| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Turin, IT, 14-22 July 2011 | Document: JCTVC-F152 |
| --- | --- |

| Title: | Cross-check of BBC transform skip mode (JCTVC-F077) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information (Cross-check) |  |  |
| Author(s) or Contact(s): | Thomas Davies | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document is a cross-check of JCTVC-F077 on Transform Skip mode. The results obtained agree with those reported in that document.

# Algorithm description

JCTVC-F077 [1] proposes that vertical and horizontal transforms can be independently turned on or off. When both transforms are turned off, the resulting values are scaled by the transform vector norms before quantization.

The transform mode is binarized by the recipe:

1 – 2D DCT

01 – Horizontal only

001 – Vertical only

000 – No transform

The same transform is used for Luma and Chroma (when there are coefficients). When luma is not coded,  no transform mode is sent and Chroma uses 2D DCT.

The transform skip tool is applied to Inter blocks only at the moment.

# Analysis of the software

The software appears to implement the algorithm as described. In order to run the software it is necessary to limit the RQT transform depth to 1 with --QuadTreeTUMaxDepthInter=1. What the software does when RQT depth is >1 is not clear. If adopted, it is recommended that RDO search and CU coding is refactored to remove dependence on RQT elements.

# Simulation results

Summary results are as follows:

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5 | 0.6 | 0.3 | 1.1 | 0.1 | 0.4 |
| Class B | 0.2 | -0.2 | 0.2 | 0.5 | -0.9 | 0.0 |
| Class C | -0.1 | 0.1 | 0.4 | 0.2 | -0.3 | 0.4 |
| Class D | -0.4 | 0.4 | 0.5 | 0.0 | 0.2 | 0.5 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.2 | 0.3 | 0.4 | -0.3 | 0.3 |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.9 | 0.6 | -2.4 | 0.0 |
| Class C | -0.5 | 0.0 | 0.5 | -0.1 | -0.6 | 1.0 |
| Class D | -1.2 | 1.4 | 1.6 | -0.5 | 0.9 | 1.4 |
| Class E | -0.5 | 0.5 | 0.0 | 0.0 | -0.5 | -0.9 |
| Overall | -0.5 | 0.5 | 0.8 | 0.0 | -0.7 | 0.4 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.7 | -0.5 | 0.5 | -2.3 | 0.2 |
| Class C | -0.6 | -0.3 | 0.2 | -0.3 | -0.6 | 0.7 |
| Class D | -1.4 | 0.9 | 1.1 | -0.9 | 0.6 | 0.8 |
| Class E | -0.8 | 0.2 | 0.2 | -0.1 | 0.4 | -0.5 |
| Overall | -0.7 | 0.0 | 0.2 | -0.2 | -0.6 | 0.3 |
| Enc Time[%] | 105% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

# The results agree with JCTVC-F077. Encoder simulation times show a slight increase (approximately 5% on average), and this also agrees with JCTVC-F077. Decoder simulation times appear very slightly higher but this is unlikely to be statistically significant.

# Comments

Matching TU size and PU size where possible is a potentially valuable simplification, especially for encoder mode selection. Skipping a transform, rather than using an alternative, is potentially helpful as it allows modes to be evaluated in series and in-place, for example:

- Evaluate no transform cost.

- Perform H transform in-place. Evaluate H transform cost.

- Perform V transform in-place. Evaluate HV transform cost

- Perform inverse H transform in place. Evaluate (approximate) V transform cost

The BBC software does not use this approach and the encoder is slower than the anchor, probably because the additional transforms that are evaluated with RQT use smaller transform sizes.

It would make sense to incorporate the required scaling into the quantizer design for modes other than 2D DCT. This avoids a double scaling operation.

Entropy coding has not been designed for the non-2D transform cases. It is likely to be quite suboptimal in the no-transform case. In particular, coefficients near (0,0) get special treatment. How to code values in the different transform cases, but especially for the no transform case, should be studied further. It is worth remarking that the results are competitive with RQT despite this limitation.

The binarization of modes tends to introduce a bias towards H coding, especially in the CAVLC case. Coding flags at TU level is probably non-optimal for CAVLC. This may explain the lower performance with CAVLC.

# Patent rights declaration(s)

Cisco Systems may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]	JCTVC-F077, “Transform skip mode”, M.Mrak, A. Gabriellini, N. Sprljan, D. Flynn (BBC), 6th JCT-VC Meeting, Turin, IT, 14-22 July, 2011

[2] JCTVC-E700, “Common test conditions and software reference configuration”, F. Bossen (DoCoMo), 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011