| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F153 WG11 Number: m20571 |
| --- | --- |

| Title: | On fast implementation of 4-point ODST-3 in HM3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

In this contribution, possible fast implementations of the 4-point ODST-3 that are mathematically equivalent to that in HM3 but have lower operations count than that found in the current HM3 software are presented. The (29,55) approximation of ODST-3 required 11 additions and 8 multiplications as implemented in HM3, while two other implementations require either 10 additions and 6 multiplications or 13 additions, 4 multiplications and 2 bit-shifts. A previously mentioned (28,56) approximation of ODST-3 required 11 additions, 5 multiplications and 3 bit-shifts, while two other implementations require either 10 additions, 5 multiplications and 1 bit-shift or 11 additions, 4 multiplications and 2 bit-shifts.

# Introduction

The 4-point ODST-3 has been adopted in HM3 for mode-dependent transform coding of intra prediction residuals [1] [2]. To match the scaling used in the core transforms that was adopted in the HM3 software [3], the “(29,55)-variant” of the ODST-3, obtained by scaling the ODST-3 transform matrix by 128 [5], was used. Later, a faster implementation of the ODST-3 was also discussed on the reflector, made possible by tweaking some of the coefficients to obtain the “(28,56)-variant” of the ODST-3.

In this contribution, we review the fast implementations that have been discussed, and introduce two new fast implementations of the (29,55) and (28,56) 4-point ODST-3 that have lower operations count. In particular, the (29,55) ODST-3 now has an operations count of 10 additions and 6 multiplications.

Table 1 shows a summary of the operation counts for all alternatives presented in this document.

Table 1. Summary of operation counts

| ODST-3 Variant | Method | Additions | Multipli-cations | Shifts | Stages |
| --- | --- | --- | --- | --- | --- |
| (29,55) | HM3 | 11 | 8 | 0 | 2 |
|  | B024 | 10 | 8 | 0 | 2 |
|  | Alternative 1 | 10 | 6 | 0 | 3 |
|  | Alternative 2 | 13 | 4 | 2 | 3 |
| (28,56) | F283 | 11 | 5 | 3 | 2 |
|  | Alternative 1 | 10 | 5 | 1 | 3 |
|  | Alternative 2 | 11 | 4 | 2 | 3 |

- Fast Implementations

## Background

The (29,55) 4-point ODST-3 has the following backward transform matrix:

K1=297484555574−29−84740−747484−7455−29

Its fast implementation for x = K1y, as implemented in HM3, is:

| Stage 1 | Stage 2 |
| --- | --- |
| c0 = y0 + y2 c1 = y2 + y3 c2 = y0 – y3 c3 = 74*y1 | x0 = 29*c0 + 55*c1 + c3 x1 = 55*c2 – 29*c1 + c3 x2 = 74*(y0 – y2 + y3) x3 = 55*c0 + 29*c2 – c3 |

Ignoring bit-shifts due to de-quantization, this requires a total of 11 additions and 8 multiplications.

In JCTVC-B024, we had in fact already proposed an alternative implementation that requires only 10 additions and 8 multiplications [4]:

| Stage 1 | Stage 2 |
| --- | --- |
| c0 = y0 + y2 c1 = y2 + y3 c3 = 74*y1 | x0 = 29*c0 + 55*c1 + c3 x1 = 55*c0 – 84*c1 + c3 x2 = 74*(y0 – y2 + y3) x3 = 84*c0 – 29*c1 – c3 |

Motivated by the possibility of reducing complexity, Samsung has also proposed a (28,56) variant of the ODST-3, with the following backward transform matrix:

K2=287484565674−28−84740−747484−7456−28

This has a fast implementation for x = K2y as follows:

| Stage 1 | Stage 2 |
| --- | --- |
| c0 = y0 + y2 c1 = y2 + y3 c2 = y0 – y3 c3 = 74*y1 | x0 = 28*(c0 + c1 << 1) + c3 x1 = 28*(c2 << 1 – c1) + c3 x2 = 74*(y0 – y2 + y3) x3 = 28*(c0 << 1 + c2) – c3 |

By trading in some multiplications for shifts, this requires a total of 11 additions, 5 multiplications and 3 bitshifts, again ignoring bit-shifts due to de-quantization.

## New Fast Implementation

We have found new fast implementations for both the (29,55) and (28,56) transforms.

For the (29,55) backward transform, K1, the following fast implementation is mathematically equivalent:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = y0 – y3 a1 = y2 + y3 a2 = y0 – y2 + y3 a3 = 74*y1 | b0 = 29*a0 + 84*a1 b1 = 55*a0 – 29*a1 | x0 = b0 + a3 x1 = b1 + a3 x2 = 74*a2 x3 = b0 + b1 – a3 |

However, this only requires 10 additions and 6 multiplications.

Another possibility is the following:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = y0 – y3 a1 = y2 + y3 a2 = y0 – y2 + y3 a3 = 74*y1 a4 = y0 + y2 | b0 = 28*(a1<<1 + a4) + a0 b1 = 28*(a0<<1 – a1) – a4 | x0 = b0 + a3 x1 = b1 + a3 x2 = 74*a2 x3 = b0 + b1 – a3 |

This would require 13 additions, 4 multiplications and 2 bit-shifts.

For the (28,56) backward transform, K2, the following fast implementation is mathematically equivalent:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = y0 – y3 a1 = y2 + y3 a2 = y0 – y2 + y3 a3 = 74*y1 | b0 = 28*a0 + 84*a1 b1 = 28*(a0 << 1 – a1) | x0 = b0 + a3 x1 = b1 + a3 x2 = 74*a2 x3 = b0 + b1 – a3 |

This requires 10 additions, 5 multiplications and 1 bit-shift.

Another possibility is the following:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = y0 – y3 a1 = y2 + y3 a2 = y0 – y2 + y3 a3 = 74*y1 | b0 = 28*(a0 + a1<<1 + a1) b1 = 28*(a0 << 1 – a1) | x0 = b0 + a3 x1 = b1 + a3 x2 = 74*a2 x3 = b0 + b1 – a3 |

This would require 11 additions, 4 multiplications and 2 bit-shifts.

The appendix also describes the fast implementation for the forward transform for both (29,55) and (28,56) ODST-3.

# Conclusions

In this contribution, we have presented alternative fast implementations for both the (29,55) and (28,56) approximations of the 4-point ODST-3 with lower operations count that previously reported.

Given the possibility of finding ever more efficient implementations of transforms, we recommend that decisions should be made first based on coding performance, especially in cases where the operations count is very similar. Furthermore, until the core transforms have been decided in HM, any decision to tweak approximations of ODST-3 for complexity reduction may be premature.

# References

[1] A. Saxena, and F. C. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” in JCTVC-E125, Geneva, Switzerland, Mar. 2011.

[2] R. Joshi, P. Chen, M. Karczewicz, A. Tanizawa, J. Yamaguchi, C. Yeo, Y. H. Tan, H. Yang, and H. Yu, “CE7: Mode dependent intra residual coding,” in JCTVC-E098, Geneva, Switzerland, Mar. 2011.

[3] A. Fuldseth, G. Bjøntegaard, M. Sadafale, and M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, Geneva, Switzerland, Mar 2011.

[4] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” in JCTVC-B024, Geneva, Switzerland, July 2010.

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix – Fast forward transform implementation

For the (29,55) forward transform to compute y = K1Tx, the following steps uses 10 additions and 6 multiplications.

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = x0 + x3 a1 = x1 + x3 a2 = 74*x2 a3 = x0 + x1 – x3 | b0 = 29*a0 + 55*a1 b1 = 84*a0 – 29*a1 | y0 = b0 + a2 y1 = 74*a3 y2 = b1 – a2 y3 = b1 – b0 + a2 |

Equivalently, one can also use the following, which requires 13 additions, 4 multiplications and 2 bit-shifts:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = x0 + x3 a1 = x1 + x3 a2 = 74*x2 a3 = x0 + x1 – x3 a4 = x0 – x1 | b0 = 28*(a0 + a1 << 1) + a4 b1 = 28*(a0 << 1 + a4) – a1 | y0 = b0 + a2 y1 = 74*a3 y2 = b1 – a2 y3 = b1 – b0 + a2 |

For the (28,56) forward transform to compute y = K2Tx, the following steps uses 10 additions, 5 multiplications and 1 bit-shift.

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = x0 + x3 a1 = x1 + x3 a2 = 74*x2 a3 = x0 + x1 – x3 | b0 = 28*(a0 + a1 << 1) b1 = 84*a0 – 28*a1 | y0 = b0 + a2 y1 = 74*a3 y2 = b1 – a2 y3 = b1 – b0 + a2 |

Equivalently, one can also use the following, which requires 11 additions, 4 multiplications and 2 bit-sfhits:

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| a0 = x0 + x3 a1 = x1 + x3 a2 = 74*x2 a3 = x0 + x1 – x3 | b0 = 28*(a0 + a1 << 1) b1 = 28*(a0 << 1 + a0 – a1) | y0 = b0 + a2 y1 = 74*a3 y2 = b1 – a2 y3 = b1 – b0 + a2 |