| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F155 WG11 Number: m20574 |
| --- | --- |

| Title: | CE6 Subtest B: Cross-check report on SDIP Test 2b |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

CE6 Subtest B aims to investigate the coding performance and complexity when Short-Distance Intra Prediction (SDIP) is used. This report documents the results and observations from the experiments done by Institute for Infocomm Research on Test 2 of Subtest B, looking at the interaction between SDIP and Mode-Dependent DCT/DST. The cases tested was on the use of 8x8 DST, in addition to the 4x4 DST. The coding results obtained match what was provided by the proponents.

# Introduction

JCTVC-E278 [1] proposed Short-Distance Intra Prediction (SDIP) that uses rectangular instead of square blocks as the PU. The purpose of CE Subtest B is to study its trade-off between coding performance and complexity, as well as the interaction between SDIP and other coding tools.

In this document, we provide the results of Test 2b of Subtest B, which looks at the additional gains possible when 8x8 DST is added, with and without SDIP.

Two tests, Tests 5 and 6, were done. In Test 5, the impact of adding 8x8 DST was studied, using an anchor where SDIP is turned off (similar to HM3). In Test 6, the impact of adding 8x8 DST was studied, using an anchor where SDIP is turned on, and 4x4 DST is used for both UDI and SDIP blocks. The table below summarizes the conditions:

| Test | Anchor Conditions | Tested Conditions |
| --- | --- | --- |
| Test 5 | HM3 | HM3 + 8x8 DST |
| Test 6 | HM3 + SDIP + 4x4 DST | HM3 + SDIP + 4x4 DST + 8x8 DST |

In the current implementation of SDIP, the SDIP feature can be used for 32x32, 16x16 and 8x8 CU. The table below summarizes the choice of rectangular PUs that are available:

| CU Size | SDIP sizes |
| --- | --- |
| 64x64 | NA |
| 32x32 | 32x8, 8x32 |
| 16x16 | 16x4, 4x16 (16x1, 1x16 available as TU) |
| 8x8 | 8x2, 2x8 |

Some fast decision is also being used to avoid the checking of SDIP modes depending on the number of bits used in for 2Nx2N Intra coding and the current QP value.

When the 4x4 DST is used in SDIP, it is applicable to the 16x4 and 4x16 SDIP sizes. When the 8x8 DST is used in SDIP, it is applicable to the 32x8, 8x32, 8x2, and 2x8 SDIP sizes.  The same intra prediction direction to transform mapping table is used.

# Simulation Results

The proposed modifications were made in HM 3.0. Tests were conducted using the two Intra configurations, i.e. high-efficiency and low complexity, following the common testing conditions [2].

Table 1 below shows the results of Test 5 (under Test 2b of Subtest B).

Table 1. BD-Rate results for  Test 5 (under Test 2b of Subtest B)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.0 | -0.3 | -0.2 | -0.4 | -0.6 |
| Class B | -0.1 | -0.2 | -0.1 | 0.2 | -0.6 | -0.6 |
| Class C | 0.0 | -0.2 | -0.2 | -0.1 | -0.4 | -0.4 |
| Class D | 0.0 | -0.2 | -0.2 | 0.0 | -0.3 | -0.3 |
| Class E | -0.2 | 0.0 | -0.3 | 0.0 | -0.4 | -0.6 |
| Overall | -0.2 | -0.1 | -0.2 | 0.0 | -0.4 | -0.5 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Table 2 below shows the results of Test 6 (under Test 2b of Subtest B).

Table 2. BD-Rate results for Test 6 (under Test 2b of Subtest B)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.1 | -0.2 | -0.4 | -0.4 | -0.6 |
| Class B | -0.3 | -0.2 | -0.1 | -0.2 | -0.5 | -0.5 |
| Class C | -0.2 | -0.2 | -0.2 | -0.3 | -0.4 | -0.4 |
| Class D | -0.2 | -0.3 | -0.2 | -0.3 | -0.4 | -0.3 |
| Class E | -0.5 | 0.3 | 0.1 | -0.5 | 0.1 | -0.1 |
| Overall | -0.3 | -0.1 | -0.1 | -0.3 | -0.3 | -0.4 |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

# Conclusions

These CE6 cross-check experiments report results which are consistent with those provided by the proponents.

# References

[1]	X. Cao et al., " CE6.b1 Report on Short Distance Intra Prediction Method," Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E278, 5th Meeting: Geneva, CH, Mar., 2011.

[2] F. Bossen, “Common conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, KR, Jan., 2011.