| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Turin, IT, 14-22 July 2011 | Document: JCTVC-F160 |
| --- | --- |

| Title: | CAVLC table-size reduction for Inter/chroma run-level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Davies | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

A modification to the method for computing code numbers for run-level pairs in inter and chroma blocks is proposed. This reduces the size of the table used from 434 bytes to 104 bytes. For I_LC, RA_LC, LDB_LC and LDP_LC the BD-rate performance is reported to be 0.0%, 0.0%, 0.0% and 0.1%. There is no significant change to encoder or decoder simulation times.

# Algorithm description

## Current LUT operation

In CAVLC mode, blocks are categorized according to the following criteria:

- Intra or inter

- Luma or chroma

- Block size

8 combinations of these parameters are defined for the purposes of coding run-level pairs. Two combinations relate to intra luma blocks (large and small); the remainder correspond to chroma and inter blocks. JCTVC-D366 [1] introduced a formula for deriving code number values for intra blocks which was adopted into the HM. A common look-up table remains for coding the other block types, conditioned on maxrun - the maximum run of zeroes possible until there is a non-zero value in the block, a value maxrun+1 indicating all zeroes.

This LUT provides for each value of maxrun a mapping from the set of possible integers [0, maxrun+1] to [0, maxrun+1] indicating the code number to be used for the value of run for run between 0 and maxrun+1,  and lev=(level>1) equal 0. For maxrun>=28, cn(run,0)=run.

Pairs (run, 1) (i.e. level>1) are coded with code-number

cn(run,1)=maxrun+1+cn(run,0)

for all values of maxrun. In other words, all runs terminating with a large value have shorter codes than runs ending in 1.

## Proposed modification

It is proposed to limit the LUT for inter and chroma blocks to values 0<=maxrun<14 and use a formula for computing code numbers for all larger values. This reduces the size of the table from 434 bytes to 104 bytes.

The formula for maxrun>=14 is modified as follows:

cn(run,0)=(run==maxrun+1) ? 0 : run+1
	cn(run,1)=maxrun+2+run

i.e. it is deemed most likely that all the remaining values are zero when level==1.

# Performance

## Coding efficiency

The modification has negligible effect on performance. Simulations were performed using the common test configurations [2]. The summary figures are as follows:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #REF! | #REF! | #REF! | 0.0 | 0.1 | 0.0 |
| Class B | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Class C | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Class D | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Class E | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Overall | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #REF! |  |  | 98% |  |  |
| Dec Time[%] | #REF! |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #REF! | #REF! | #REF! | 0.1 | 0.1 | 0.2 |
| Class B | #REF! | #REF! | #REF! | 0.0 | -0.1 | 0.0 |
| Class C | #REF! | #REF! | #REF! | 0.0 | 0.0 | -0.1 |
| Class D | #REF! | #REF! | #REF! | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | #REF! | #REF! | #REF! | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #REF! |  |  | 99% |  |  |
| Dec Time[%] | #REF! |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #REF! | #REF! | #REF! | 0.1 | -0.1 | 0.0 |
| Class C | #REF! | #REF! | #REF! | 0.0 | 0.1 | 0.0 |
| Class D | #REF! | #REF! | #REF! | 0.1 | -0.1 | 0.6 |
| Class E | #REF! | #REF! | #REF! | 0.0 | -0.4 | -0.4 |
| Overall | #REF! | #REF! | #REF! | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | #REF! |  |  | 99% |  |  |
| Dec Time[%] | #REF! |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #REF! | #REF! | #REF! | 0.1 | 0.0 | 0.0 |
| Class C | #REF! | #REF! | #REF! | 0.1 | 0.0 | 0.0 |
| Class D | #REF! | #REF! | #REF! | 0.1 | 0.1 | 0.3 |
| Class E | #REF! | #REF! | #REF! | 0.1 | 0.4 | -0.2 |
| Overall | #REF! | #REF! | #REF! | 0.1 | 0.1 | 0.0 |
| Enc Time[%] | #REF! |  |  | 100% |  |  |
| Dec Time[%] | #REF! |  |  | 99% |  |  |

## Complexity

In simulation there is no significant difference in encoding or decoding time. In software implementation, it is likely that tables will be used for all values and so there is no effect on efficiency. In hardware, the table size is significantly reduced with negligible additional logic, resulting in an overall reduction in complexity.

# Modifications to bitstream syntax

Section 9.2.2.1 Parsing process for run_level_one to be modified as follows:

...

- Otherwise (blockType is not equal to 2 and blockType is not equal to 5), the value of run_level_one is derived as follows.

  - The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag  = codeNum <= maxRun + 1 ? 0 : 1

  - If maxRun is less than 14, the variable runOfZeros is derived from maxRun and codeNum as shown in Table 9-41.

  - Otherwise (maxRun is not less than 14), the variable runOfZeros is derived as

runOfZeros = codeNum – (codeNum <= maxRun + 1) % maxRun+2

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + n : runOfZeros

- The variable trOne is derived as

trOne = (trOne = = 0 | | levelGreaterThanOneFlag) ? 0 : Max(4, trOne + 1)

.....

Table 9-41– Derivation of runOfZeros from maxRrun and codeNum

|  | codeNum%(maxRun+1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| maxRun | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 1 | 0 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 | 2 | 1 | 0 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 | 3 | 0 | 2 | 1 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 | 4 | 1 | 0 | 2 | 3 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 4 | 5 | 0 | 3 | 2 | 4 | 1 | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 5 | 6 | 0 | 1 | 4 | 5 | 3 | 2 | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 | 7 | 1 | 0 | 2 | 4 | 3 | 6 | 5 | NA | NA | NA | NA | NA | NA | NA |
| 7 | 8 | 0 | 3 | 4 | 2 | 1 | 5 | 7 | 6 | NA | NA | NA | NA | NA | NA |
| 8 | 9 | 0 | 5 | 1 | 6 | 4 | 8 | 3 | 2 | 7 | NA | NA | NA | NA | NA |
| 9 | 10 | 0 | 1 | 6 | 7 | 2 | 9 | 5 | 4 | 3 | 8 | NA | NA | NA | NA |
| 10 | 11 | 1 | 0 | 2 | 3 | 6 | 7 | 4 | 5 | 8 | 10 | 9 | NA | NA | NA |
| 11 | 12 | 0 | 3 | 2 | 1 | 4 | 6 | 5 | 7 | 11 | 8 | 9 | 10 | NA | NA |
| 12 | 13 | 0 | 1 | 5 | 6 | 4 | 2 | 3 | 7 | 8 | 12 | 10 | 11 | 9 | NA |
| 13 | 14 | 0 | 1 | 7 | 2 | 8 | 6 | 5 | 3 | 4 | 13 | 12 | 9 | 11 | 10 |

# Recommendations

# Patent rights declaration(s)

Cisco Systems may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]	JCTVC-D366, “CE5: Improved intra prediction mode coding with LCEC”, Marta Karczewicz, Xianglin Wang, Wei-Jung Chen (Qualcomm), Arild Fuldseth (Cisco), 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

[2] JCTVC-E700, “Common test conditions and software reference configuration”, F. Bossen (DoCoMo), 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011